| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AAI_ORAT-ADV message format for the IEEE 802.16m Amendment (AWD-15.2.6.5.2.1.1) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Keiichi Nakatsugawa, Masato Okuda Fujitsu Jin Lee, Ronny Yongho Kim, Kiseon Ryu LG Electronics Yang Liu, Feng Xie ZTE Corporation | nakatsugawa@jp.fujitsu.com {jins978, ronnykim}@lge.com {liu.yang8,xie.feng2}@zte.com.cn *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content”. Target call: Comments and Related Contributions on AWD |  |
| Abstract | Proposed AAI_ORAT-ADV message format for inter-RAT Handover. |  |
| Purpose | To be discussed and adopted in 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AAI_ORAT-ADV message format for the 802.16m Amendment (AWD-15.2.6.5.2.1.1)

Keiichi Nakatsugawa, Masato Okuda

Fujitsu

Jin Lee, Ronny Yongho Kim, Kiseon Ryu

LG Electronics

Yang Liu, Feng Xie

ZTE Corporation

# Introduction

As described in 15.2.6.5.2.1.1 of the AWD[1], AAI_ORAT-ADV message is used for other RAT information delivery.

<15.2.6.5.2.1.1 Passive Other RAT Discovery>

The AAI_ORAT-MSG message includes the following:

RAT Type: This field specifies air interface technology type.

- Pre-registration supported: This field indicates whether pre-registration is supported or not for Inter RAT handover.

- PHY Profile ID: The PHY Profile ID contains the information related to scan the corresponding RAP. The contents of PHY profile ID are TBD.Network boundary indication: This field indicates the whether the ABS which is sending this message is located in the boundary area of the 802.16m network or not.

- RAN Information: The RAN information specifies information for different radio access networks with various RATs defined by different standard bodies

- RAP Information: The RAP information specifies information for different radio access points (such as carrier frequency, BSID, preamble).

We propose to define AAI_ORAT-ADV message format based on the above description. Note that both PHY Profile ID and RAP information are related to radio access points, we define only RAP information in the proposed format.

And, the 2nd paragraph of 15.2.6.5.2.1.1 describes “SFH defines other RAT information flag.” For this flag, Additional broadcast information indicator (ABI) in S-SFH SP2 IE (15.3.6.5.1.2, Table 664) can be used. We propose to define the flag in the S-SFH SP2 IE.

# Proposed Amendment Text

[Insert the following new subclause to 15.2.5 AAI MAC Management Messages]

(Note) Subclause and table number x should be assigned accordingly.

============================== Start of the Text=================================

15.2.5.x AAI_ORAT-ADV

ABS may broadcast AAI_ORAT-ADV to advertise other RAT information and the network boundary indication

of the WirelessMAN-OFDMA Advanced network system (See 15.2.6.5.2.1.1).

Table x AAI_ORAT-ADV message format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_ORAT-ADV_Message_format() { | - | - |
| Management Message Type = Undecided | 8 | - |
| Configuration Change Count | 8 | Incremented each time the information for the associated neighbor BS has changed. |
| N_NBR_RANs | 8 | Number of neighbor radio access networks |
| For ( j = 0; j< N_NBR_RANs; j++) { | - | - |
| Length | 8 | Length of message information including all fields within the RAN FOR loop. |
| RAT Type | 3 | 0b000: 802.11 0b001: GERAN(GSM/GPRS/EGPRS) 0b010: UTRAN 0b011: E-UTRAN 0b100: CDMA 2000 0b101-0b111: Reserved |
| Pre-registration supported | 1 | 0: Not supported. 1: Supported. |
| N_NBR_RAPs | 3 | Number of neighbor radio access points |
| For ( k = 0; k< N_NBR_RAPs; k++) { |  |  |
| RAP Information | variable | TLV-specific. |
| } |  |  |
| RAN Information | variable | TLV-specific. |
| Padding | variable | If needed for alignment to byte boundary. |
| } |  |  |
| Network boundary indication | 1 | 0: This ABS is NOT located in the boundary area of the WirelessMAN-OFDMA Advanced network. 1: This ABS is located in the boundary area of the WirelessMAN-OFDMA Advanced network. |
| Inter-RAT scanning threshold | 6 | 0b000000 ~ 0b111111: -10 ~ 53dB, 1dB increments |
| Reserved | 1 | Shall be set to zero |
| } //End of AAI_ORAT-ADV |  |  |

Table x. AAI_ORAT-ADV message format

The following parameters shall be included in the AAI_ORAT-ADV message:

Configuration Change Count

Incremented each time the information for the associated neighbor BS has changed.

RAT Type

This field specifies air interface technology type.

Pre-registration supported

This field indicates whether pre-registration is supported or not for Inter RAT handover.

RAP Information

The RAP information specifies information for different radio access points (such as carrier frequency, BSID, preamble).

Table x RAP Information TLV encodings

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | TBD | TBD | AAI_ORAT-ADV |

Network boundary indication

This field indicates the whether the ABS which is sending this message is located in the boundary area of the WirelessMAN-OFDMA Advanced network or not.

Inter-RAT scanning threshold

This field indicates when to trigger inter-RAT scanning. If signal quality of serving ABS is less than this threshold value, AMS starts inter-RAT scanning.

The AAI_ORAT-ADV message also includes the following parameters encoded as TLVs:

RAN Information

The RAN information specifies information for different radio access networks with various RATs defined by different standard bodies. Depends on the RAT type, all or subset of these TLVs are included.

Table x RAN Information TLV encodings

| Type | Length | Value | Scope |
| --- | --- | --- | --- |
| TBD | 1 | Network Type [UTRAN] 0: WCDMA, 1:TD-SCDMA | AAI_ORAT-ADV |
| TBD | 1 | Network Subtype [802.11] 0:11b, 1:11g, 2:11a, 3:11n [UTRAN] 0:HSDPA, 1:HSUPA | AAI_ORAT-ADV |
| TBD | TBD | Operator Id | AAI_ORAT-ADV |
| TBD | TBD | FrequencyBand | AAI_ORAT-ADV |
| TBD | TBD | ChannelBandwidth | AAI_ORAT-ADV |
| TBD | 1 | TDD/FDD 0: TDD 1: FDD | AAI_ORAT-ADV |

============================== End of Proposed Text =================================

[Delete the following texts of 15.2.6.5.2.1.1]  # These texts are already included in the above texts.

============================== Start of the Text=================================

The AAI_ORAT-MSG message includes the following:

- RAT Type: This field specifies air interface technology type.

- Pre-registration supported: This field indicates whether pre-registration is supported or not for Inter RAT handover.

- PHY Profile ID: The PHY Profile ID contains the information related to scan the corresponding RAP. The contents of PHY profile ID are TBD.

- Network boundary indication: This field indicates the whether the ABS which is sending this message is located in the boundary area of the 802.16m network or not.

- RAN Information: The RAN information specifies information for different radio access networks with various RATs defined by different standard bodies

- RAP Information: The RAP information specifies information for different radio access points (such as carrier frequency, BSID, preamble).

============================== End of Proposed Text =================================

[Modify Table 664 in 15.3.6.5.1.2 as follows]

============================== Start of the Text=================================

Table 664—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Duplexing mode | 1 |  |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 6 |  |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | [12] | (Need more discussion) |
| UL bandwidth | [3] | (Need more discussion) |
| } |  |  |
| MSB bytes of BSID | 24 | Specifies the Operator ID |
| FFR partitioning info for DL region | [11] | DL_SAC(4), DL_FPSC(3), DL_FPC(4) (Up to 11 bits, Need the decision from DL physical structure section) |
| FFR partitioning info for UL region | [11] | UL_SAC(4), UL_FPSC(3), UL_FPC(4) (up to 11 bits, Need the decision from UL physical structure section) |
| Initial ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 (Need the decision from UL Ctrl or MAC operation section) |
| NSP ID | 24 | Network service provider ID |
| Additional broadcast information indicator (ABI) | 1TBD | Bit#0: other RAT information flag |
| If (other RAT information flag == 1) { |  |  |
| SFN (start frame number) | 2 | Frame number within the current super frame which contains AAI_ORAT-ADV message. 0b00: Frame 0 0b01: Frame 1 0b10: Frame 2 0b11: Frame 3 |
| } |  |  |
| MS Transmit Power Limitation Level | 8 | Unsigned 8-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| Minimum level of power offset adjustment | TBD |  |
| Maximum level of power offset adjustment | TBD |  |
| reserved |  |  |
| } |  |  |

============================== End of Proposed Text =================================

# References

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document” (AWD)