| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improved Persistent Allocation A-MAP IE Structure |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Sun Bo, Lu Zhaohua ZTE Corporation | {sun.bo1, lu.zhaohua}@zte.com.cn |
| Re: | Category: AWD comments / Area: Chapter 15.3.6.4.2.7.2 (UL Composite PA A-MAP IE) “Comments on AWD 15.3.6.4.2.7.2 UL Composite PA A-MAP IE” |  |
| Abstract | The contribution proposes an improved Persistent Allocation A-MAP IE structure to be included in the 802.16m AWD. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improve Persistent Allocation A-MAP IE Structure

Sun Bo, Lu Zhaohua

ZTE Corporation

# Introduction

Persistent A-MAP IE is defined in existing AWD document. Most of this IE structure is well defined but there’re still some corners are not clear.

In PA de-allocation scheme, the current AWD text is“When the Allocation Period is set to 0b00 in the DL/UL Individual Persistent A-MAP IE, the assigned persistent resource in DL/UL Individual Persistent A-MAP IE is de-allocated in referenced DL/UL subframe and the ABS terminates the persistent allocation.”，which is not clear to tell the AMS how to deal with long TTI PA resource in de-allocation, therefore we suggest to add ‘Long TTI Indicator’ field in PA de-allocation IE.

We also suggest to add ‘MAP HFA’ field in Uplink PA MAP to solve uplink PA MAP error handling problem.

This contribution proposes an improved Persistent A-MAP IE to make the structure more reasonable.

# Proposed Text

=========================Text Start=========================================

### 15.3.6.4.2.6.1 DL Individual PA A-MAP IE

The DL individual persistent A-MAP IE is specified in Table 676.

Table 676: DL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| If (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| … |  |  |

### 15.3.6.4.2.6.2 DL Composite PA A-MAP IE

The DL composite persistent A-MAP IE is specified in Table 677.

Table 677: DL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL Composite Persistent A-MAP_IE() { | - | - |
| … |  |  |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| … |  |  |

### 15.3.6.4.2.7.1 UL Individual PA A-MAP IE

The UL individual persistent A-MAP IE is specified in Table 679.

Table 668: UL Individual Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Persistent A-MAP_IE() { | - | - |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of persistent resource. 0b00: 0 frame 0b01: 2 frames 0b10: 4 frames 0b11: 6 frames |
| If (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| MCS | 4 | MCS used for burst |
| … |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| MAP HFA | [4] | MAP HARQ Feedback Allocation |
| ACID | 3 | HARQ channel identifier |
| … |  |  |

### 15.3.6.4.2.7.2 UL Composite PA A-MAP IE

The UL composite persistent A-MAP IE is specified in Table 680.

Table 680: UL Composite Persistent A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL Composite Persistent A-MAP_IE() { | - | - |
| Number of allocations | 5 | Number of allocation specified |
| RCID Type | 2 | 0b00: Normal CID 0b01: RCID11 0b10: RCID7 0b11: RCID3 |
| For (j=0;j<Number of allocations; j++) { |  | For loop where each loop element specifies information for one allocation. |
| Persistent Flag | 1 | 0 = non-persistent 1 = persistent |
| RCID | variable | Specifies the station ID in RCID format, type defined by RCID Type |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| MAP HFA | [4] | MAP HARQ Feedback Allocation |
| } |  |  |
| } |  |  |
| If (Persistent Flag != 1) or （Allocation Period ！= ‘00’） |  |  |
| { |  |  |
| Allocation MCS indicator | 1 | If Allocation MCS Indicator is 1, it indicates that MCS is explicitly assigned for this allocation. Otherwise, this allocation will use the same MCS as the previous subburst. If j is 0 then this indicator shall be 1. |
| if (Allocation MCS indicator == 1) { |  |  |
| MCS | 4 | Depends on supported modes, 16 modes assumed as baseline |
| } |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| if(MEF == 0b1){ |  | Paraneters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b11: 2 streams 0b00: 3 stream 0b11: 4 streams |
| if(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 0b01){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else if( TNS == 0b10 or TNS==0b11){ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding using the precoder of rank Mt of MS’s choice |
| Resource Allocation Indicator | 1 | If Resource Allocation Indicator is 1, it indicates that Duration is explicitly assigned for this subburst (allocation or deallocation). Otherwise, this subburst (allocation or deallocation) will use the same Duration as the previous subburst. If j is 1 then this indicator shall be 1. |
| if (Resource Allocation Indicator ==1) { |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| }else if (Nsubframe, A-MAP == 1 and DL:UL == 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every subframes (Nsubframe, A-MAP =1) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first UL subframe and the next two A-MAP regions are relevant to two UL subframes each. 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| HFA | [4] | TBD HARQ Feedback Allocation |
| if (Persistent Flag == 1) { |  |  |
| Allocation Period | 2 | Period of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| if (Allocation Period==0b00){ |  |  |
| Resource Allocation | Variable | Variable number of bits – depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) Allocation size |
| HFA | [4] | TBD HARQ Feedback Allocation |
| } else if (Allocation Period != 0b00){ |  |  |
| Allocation Period and ACID Indicator | 1 | If Allocation Period and ACID Indicator is 1, it indicates that allocation information (allocation period, Number of ACID (ACID) is explicitly assigned for this allocation. Otherwise, this allocation will use the same allocation period as the previous allocation. If j is 0 then this indicator shall be 1. |
| if (Allocation Period and ACID Indicator == 1) { | - | - |
| Allocation Periodicity | 2 | Periodicity of persistent allocation If (Allocation Period==0b00), it indicates the deallocation of a persistently allocated resource. 0b00: deallocation 0b01: 2 frames 0b10: 4 frames 0b11: 8 frames |
| Allocation Period (AP) | 5 | Period of the persistent allocation is this field value plus 1 (unit is sub-frame/frame TBD) |
| ACID | 4 | Number of HARQ channels associated with this persistent assignment is this field value plus 1 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

==============================Text End===============================