| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | (AWD-15.2.3.2.1) An Extended Header for ARQ Retransmission |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jaesun Cha, Kwangjae Lim, Sungcheol Chang, Chulsik Yoon ETRI Eunjong Lee, Jeongki Kim and Kiseon Ryu LG Electronics Youngbin Chang, Anil Agiwal Samsung Electronics | jscha@etri.re.kr, mehappy3@lge.com yb.chang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comment / Area: Chapter 15.2.3.1 (MAC header formats) “Call for comments on Project 802.16m Amendment Working Document (AWD) Content”. |  |
| Abstract | This contribution proposes a new Extended Header for ARQ retransmission to be included in the 802.16m amendment working document. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

An Extended Header for ARQ Retransmission

Jaesun Cha, Kwangjae Lim, Sungcheol Chang, Chulsik Yoon

ETRI

Eunjong Lee, Jeongki Kim and Kiseon Ryu

LG Electronics

Youngbin Chang, Anil Agiwal

Samsung Electronics

# Introduction

According to the ARQ procedure defined in 15.2.13 of AWD [1], ARQ feedback IE, ARQ feedback poll and RFPEH (Rearrangement Fragmentation and Packing Extended Subheader) are used for ARQ retransmission. But, the detailed method and format are not defined yet.

In this contribution, we propose the structure of RFPEH and RMEHB (Rearrangement Multiplexing Extended Header Block) which are used for ARQ retransmission with rearrangement instead of FPEH and MEHB, respectively.

In addition, we propose to add some parameters into FPEH and MEH for ARQ feedback operation.

# Summary of the Proposed Modifications

In this contribution, we have modified two extended headers and one extended header block; FPEH, MEH and MEHB. In addition to the modification, we have defined one extended header and one extended header block for ARQ retransmission; RFPEH and RMEHB.

## Modification of the existing extended header and extended header block

The applied modification is summarized in Table 1.

Table 1 – the summary of the modification

| Header | Added parameters (size in unit of bit) | Overhead |
| --- | --- | --- |
| FPEH | RI(1), AFI(1), AFP(1) | No change. There is no change in total length of the FPEH because the FPEH always needs 3-bit or 7-bit reserved field for byte alignment. |
| MEH | AFI(1) | If a MEH contains NMEHB MEHBs, the increased overhead is (1 + NMEHB *2) bits. But, the total length of the MEH depends on how many MEHBs are included in the MEH and how many SDU/SDU fragments are included in each MEHB. |
| MEHB | RI(1), AFP(1) |  |

- RI (Re-arrangement header indicator) indicates whether this PDU contains ARQ retransmission information or not. By using the value of the RI, the receiver distinguishes FPEH and RFPEH or MEHB and RMEHB.

0 = Indicating this ARQ block is firstly transmitted or retransmitted without rearrangement

1 = Indicating this ARQ block is retransmitted with rearrangement

- AFI (ARQ Feedback IE indicator) indicates whether an ARQ feedback IE presents in a MAC PDU or not. In case of MEH, we put the AFI right after ‘Type’ field because ARQ feedback information for multiple ARQ connections can be packed into a single MAC PDU.

- AFP (ARQ Feedback Poll indicator) indicates that the ARQ transmitter requests the transmission of ARQ feedback IE to the ARQ receiver. If this field is set to ‘1’, the receiver transmits the ARQ feedback IE for this connection to the transmitter.

## Definition of new extended header and extended header block for ARQ retransmission

In this contribution, we have defined RFPEH and RMEHB. They are used for ARQ retransmission with rearrangement instead of FPEH and MEHB, respectively.

Basically, the format of the RFPEH and RMEHB is inherited from FPEH and MEHB, respectively. In addition to that, some parameters are added to represent ARQ retransmission information and they are summarized in Table 2.

Table 1 – newly added parameters

| Parameters | Size (bit) | Scope | Description |
| --- | --- | --- | --- |
| RI | 1 | RFPEH, RMEHB | Refer to 2.1 of this contribution |
| AFI | 1 | RFPEH | Refer to 2.1 of this contribution |
| AFP | 1 | RFPEH, RMEHB | Refer to 2.1 of this contribution |
| LSI | 1 | RFPEH, RMEHB | LSI (Last Sub-block indicator) indicates whether the last ARQ sub-block from a single ARQ block is included in the MAC PDU or not. 0 = Indicating the last ARQ sub-block from a single ARQ block is not included in this MAC PDU 1 = Indicating the last ARQ sub-block from a single ARQ block is included in this MAC PDU |
| SSN | TBD | RFPEH, RMEHB | SUB-SN of the first ARQ sub-block |

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify section 15.2.3.2.1 as follows]

15.2.3.2.1 Fragmentation and Packing Extended Header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in GMH set to '1' or after the GMH if 'EH' in GMH set to '0'. The FPEH format is defined in Table 7— and its fields are defined in Table 8—.

Table 7 – FPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FPEH () { |  |  |
| RI | 1 | Re-arrangement header indicator Always set to ‘0’ |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation control (see tableTable 9) |
| AFI | 1 | ARQ feedback IE indicator |
| AFP | 1 | ARQ feedback poll indicator |
| Do { |  |  |
| End | 1 |  |
| If (End == 0) { |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } |  |  |
| } while (!End) |  |  |
| Reserved | variable |  |
| } |  |  |

Table 8 – FPEH fields

| Names | Size (bit) | Description |
| --- | --- | --- |
| RI | 1 | Re-arrangement header indicator 0 = FPEH format follows after RI bit 1 = RFPEH format follows after RI bit |
| SN | 10 | Payload sequence number |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator. 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. |
| Rsvd | variable | Reserved bits are added at the end of FPEH for byte alignment. |

Table 9 – Encoding of FC field

| FC | Meaning | Examples |
| --- | --- | --- |
| 00 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | - One or Multiple Full SDUs packed in an MPDU |
| 01 | The first byte of data in the MPDU payload is the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | - MPDU with only First fragment of an SDU - MPDU with one or more unfragmented SDUs, followed by first fragment of subsequent SDU |
| 10 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is the last byte of a MAC SDU. | - MPDU with only Last fragment of an SDU - MPDU with Last fragment of an SDU, followed by one or more unfragmented subsequent SDUs |
| 11 | The first byte of data in the MPDU payload is not the first byte of a MAC SDU. The last byte of data in the MPDU payload is not the last byte of a MAC SDU. | - a) MPDU with only middle fragment of an SDU - MPDU with Last fragment of an SDU, followed by zero or more unfragmented SDUs, followed by first fragment of a subsequent SDU |

15.2.3.2.2 Rearrangement Fragmentation and Packing Extended Header (RFPEH)

RFPEH shall be used only for ARQ connection. It is used when a MAC PDU contains ARQ sub-blocks from a single transport connection for retransmission. The location of this header exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in GMH set to '1' or after the GMH if 'EH' in GMH set to '0'. The RFPEH format is defined in Table xa and its fields are defined in Table xb.

Table xa – RFPEH format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| RFPEH () { |  |  |
| RI | 1 | Re-arrangement header Indicator Always set to ‘1’ |
| SN | 10 | ARQ block sequence number |
| FC | 2 | Fragmentation control (see Table 9) |
| AFI | 1 | ARQ feedback IE Indicator. |
| AFP | 1 | ARQ Feedback poll indicator. |
| LSI | 1 | Last ARQ sub-block indicator |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| Do { |  |  |
| End | 1 |  |
| If (End == 0) { |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } |  |  |
| }While (!End) |  |  |
| Reserved | variable |  |
| } |  |  |

Table xb – RFPEH fields

| Names | Size (bit) | Description |
| --- | --- | --- |
| RI | 1 | Re-arrangement header indicator 0 = FPEH format follows after RI bit. 1 = RFPEH format follows after RI bit |
| SN | 10 | ARQ block sequence number Sequence number of the ARQ block which was fragmented into ARQ sub-blocks |
| FC | 2 | Fragmentation Control bits (encoding shown in Table 9) |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator. 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in GMH |
| LSI | 1 | Last ARQ sub-block indication 0 = Indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = Indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present in FPEH. The length of the first SDU or SDU fragment in the payload is indicated by the ‘Length’ field in GMH. |
| Rsvd | variable | Reserved bits are added at the end of FPEH for byte alignment. |

15.2.3.2.23 Fragmentation extended header (FEH)

The FEH shall be used when MAC PDU contains payload from single management connection. The FEH format is defined in Table 10—.

Table 10 – FEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| FEH () { |  |  |
| LAST | 1 | Always set to ’1’ |
| Type | TBD | Type of extended header |
| SN | 8 | Type dependent content |
| FC | 2 |  |
| } |  |  |

[Modify section 15.2.3.2.3 as follows]

15.2.3.2.34 Multiplexing extended header (MEH)

The format of MEH is defined in Table 11–. It is used when SDUs/SDUs fragments from different connections are included in the same MPDU. Each MEH contains multiple Multiplexing Extended Header Blocks (MEHBs) or Rearrangement Multiplexing Extended Header Blocks (RMEHBs). The SDUs or SDU fragments belonging to the same connection are packed together and the information related to these SDUs or SDU fragments is included in one MEHB or RMEHB. The fields of MEH are defined in Table 12. The M bit in MEHB or RMEHB indicates if there is more MEHB or RMEHB followed.

Table 11 – Multiplexing Extended Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEH () { |  |  |
| LAST | 1 | 0 = Another extended header follows MEH 1 = another extended header does not follow MEH |
| Type | TBD | MEH type |
| AFI | 1 | ARQ feedback IE indicator |
| Do { |  |  |
| MEHB (M) | 1 | Multiplexing extended header block |
| } while (!M) |  |  |
| Reserved | variable |  |
| } |  |  |

Table 12 – Multiplexing Extended Header Fields

| Names | Length (bits) | Description |
| --- | --- | --- |
| LAST | 1 | Last Extended Header Indicator (always set to 1) |
| Type | TBD | Extended Header Type |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after MEH |
| MEHB | variable | Multiplexing extended header block (format shown in) |

The format of MEHB is shown in Table 13—, except the first MEHB. The format of the first MEHB is shown in Table 14—. The first MEHB doesn't contain the Flow ID and the length for the first SDU/SDU fragment associated with the Flow ID. The Flow ID and the Length fields in the generic MAC header represent the flow ID and the length of the first SDU/SDU fragment associated with the first MEHB. The fields of MEHB are defined in Table 15—.

Table 13 – MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEHB () { |  |  |
| M | 1 | 0 = Another MEHB follows the MEHB 1 = This is the last MEHB in MEH |
| RI | 1 | Re-arrangement header Indicator Always set to ‘0’ |
| FlowID | 4 | Flow Identifier |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| AFP | 1 | ARQ feedback poll indicator |
| Do { |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| End | 1 |  |
| } while (!End) |  |  |
| } |  |  |

Table 14 – First MEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MEHB () { |  |  |
| M | 1 | 0 = Another MEHB follows the MEHB 1 = This is the last MEHB in MEH |
| RI | 1 | Re-arrangement header Indicator Always set to ‘0’ |
| FC | 2 | Fragmentation Control |
| SN | 10 |  |
| AFP | 1 | ARQ Feedback poll indicator. |
| Do { |  |  |
| End | 1 |  |
| If (End == 0) |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } |  |  |
| } while (!End) |  |  |
| } |  |  |

Table 15 – MEHB fields

| Names | Length (bits) | Description |
| --- | --- | --- |
| M | 1 | Indication of more MEHB 0 = no more MEHB follows the current MEHB 1 = one or more MEHB follows the current MEHB |
| RI | 1 | Re-arrangement header indicator 0 = MEHB format follows after RI bit. 1 = RMEHB format follows after RI bit |
| Flow ID | 4 | Flow ID of the SDU/SDUs fragment identified in MEHB |
| FC | 2 | Fragmentation Control bits (encoding shown in Table x.9) |
| SN | 10 | Sequence number for the payload identified in the MEHB |
| AFP | 1 | ARQ feedback poll indicator. 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in MEHB. In case of First MEHB, ARQ feedback poll for the connection indicated in GMH. |
| Length | 11 | Length of the correspondent SDU/SDU fragment This field indicates the length of SDU or SDU fragment. In case of First MEHB, if a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present. In case of MEHB, if a payload consists of ‘N’ SDU/SDU fragments, N ‘Length’ fields are present. |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |

The format of RMEHB is shown in Table xc, except the first RMEHB. The format of the first RMEHB is shown in Table xd. In addition to the information in MEHB, the RMEHB contains additional information on ARQ retransmission. The fields of RMEHB are defined in Table xe.

Table xc – RMEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| RMEHB () { |  |  |
| M | 1 | 0 = Another RMEHB follows the RMEHB 1 = This is the last RMEHB in MEH |
| RI | 1 | Re-arrangement header Indicator Always set to ‘1’ |
| FlowID | 4 | Flow Identifier |
| FC | 2 | Fragmentation control |
| SN | 10 | ARQ block sequence number |
| AFP | 1 | ARQ Feedback poll indicator |
| LSI | 1 | Last ARQ sub-block indicator |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| Do { |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| End | 1 |  |
| } while (!End) |  |  |
| } |  |  |

Table xd – First RMEHB format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| RMEHB () { |  |  |
| M | 1 | 0 = Another MEHB follows the MEHB 1 = This is the last MEHB in MEH |
| RI | 1 | Re-arrangement header Indicator Always set to ‘1’ |
| FC | 2 | Fragmentation control |
| SN | 10 | ARQ block sequence number |
| AFR | 1 | ARQ Feedback poll indicator |
| LSI | 1 | Last ARQ sub-block indication |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| Do { |  |  |
| End | 1 |  |
| If (End == 0) |  |  |
| Length | 11 | Length of SDU or SDU fragment |
| } |  |  |
| } while (!End) |  |  |
| } |  |  |

Table xe – RMEHB fields

| Names | Length (bits) | Description |
| --- | --- | --- |
| M | 1 | Indication of more MEHB 0 = no more MEHB follows the current MEHB 1 = one or more MEHB follows the current MEHB |
| RI | 1 | Re-arrangement header indicator 0 = MEHB format follows after RI bit. 1 = RMEHB format follows after RI bit |
| Flow ID | 4 | Flow ID of the SDU/SDUs fragment identified in RMEHB |
| FC | 2 | Fragmentation Control bits (encoding shown in Table 9) |
| SN | 10 | ARQ block sequence number Sequence number of the ARQ block which was fragmented into ARQ sub-blocks |
| AFR | 1 | ARQ feedback poll indicator. 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in MEHB. In case of First MEHB, ARQ feedback poll for the connection indicated in GMH. |
| LSI | 1 | Last ARQ sub-block indication 0 = Indicating the last ARQ sub-block from a single ARQ block is not included in this MAC PDU 1 = Indicating the last ARQ sub-block from a single ARQ block is included in this MAC PDU |
| SSN | TBD | SUB-SN of the first ARQ sub-block |
| Length | 11 | This field indicates the length of SDU or SDU fragment. In case of First RMEHB, If a payload consists of ‘N’ SDU/SDU fragments, N-1 ‘Length’ fields are present. In case of RMEHB, If a payload consists of ‘N’ SDU/SDU fragments, N ‘Length’ fields are present. |
| End | 1 | Indication of more information 0 = Indicating another “Length” and “End” fields are followed 1 = Indicating no more “Length” and “End” fields are followed |

---------------------------------------------------------End of the Text----------------------------------------------------------

# Reference

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document (AWD)”