| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL power control scheme for sounding channel (AWD-15.3.9) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Alexei Davydov Yang-seok Choi Intel Corporation | alexei.davydov@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.9 (UL-CTRL) “Comments on AWD 15.3.9 (UL-CTRL)” |  |
| Abstract | The contribution proposes power control scheme for sounding channel |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# UL power control for sounding channel

Alexei Davydov, Yang-seok Choi

Intel Corporation

Intel Corporation

# Introduction

Support of open loop UL power control scheme for sounding channel is adopted in the AWD text. Further details describing operation of control algorithms are proposed.

# Proposed text

Add field to the table 730 on page 308, line 10 in the section 15.3.9.4.1 UL Open-Loop Power Control.

-------------------------------------------------- Start Text Proposal #1 --------------------------------------------------

Table 730—Normalized C/N per modulation

| Modulation/FEC rate | Required C/N |
| --- | --- |
| … | … |
| Sounding channel | 10 |
| … | … |

----------------------------------------------- End of Text Proposal #1 -------------------------------------------------

Modify text on page 310 in the section 15.3.9.4.4 Sounding Channel Power Control

-------------------------------------------------- Start Text Proposal #2 --------------------------------------------------

        - Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target CINR value. The power per subcarrier shall be maintained for the UL sounding transmission in accordance to the equation (245) in section 15.3.9.4.1.

as follows:

(249)

PTX(dBm) is the transmit power per subcarrier and per transmit antenna power level.

PL is the estimated average pathloss.

CINRtarget is the target sounding CINR required at ABS.

is the estimated average power level (dBm) of the noise and interference per subcarrier at ABS on the sounding channel.

OffsetAMSperAMS is the correction term for AMS-specific power offset controlled by AMS.

OffsetABSperAMS is the correction term for AMS-specific power offset controlled by ABS.

In Equation (2495), CSINRtarget is the sounding channel target CSINR, which is set according to the DL CSINR of AMS. In order to maintain the UL sounding quality, the different target CINR values are assigned according to DL CSINR of each AMS; the AMS with high DL CSINR applies relatively high target CSINR and the AMS with low DL CSINR applies relatively low target CSINR.

The SINRtarget for sounding channel shall be calculated from equation (246) using the following parameter settings: SINRMIN is the minimum SINR requirement expected by ABS which is set to 0dB, γIoT = 1.2 is the fairness and IoT control factor for sounding channel, SINRDL is the ratio of the downlink signal vs. noise and interference power, measured by the AMS, α and β parameters shall be set to 0.

----------------------------------------------- End of Text Proposal #2 -------------------------------------------------