| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | UL sounding text cleanup (AWD-15.3.6) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Alexei Davydov Yang-seok Choi Intel Corporation | alexei.davydov@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 (DL-CTRL)” |  |
| Abstract | The contribution proposes sounding text cleanup for the 802.16m |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# UL sounding text cleanup

Alexei Davydov, Yang-seok Choi

Intel Corporation

# Proposed text

Add the fields to the Table 665, page 197, line 44 in the section 15.3.6.5.1.2

-------------------------------------------------- Start Text Proposal #1 --------------------------------------------------

Table 665 – S-SFH SP3 IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE () { |  |  |
| … |  |  |
| Uplink subframe bitmap | 8 | Uplink subframe index with sounding symbol |
| Sounding Multiplexing Type (SMT) | 1 | 0b0: Frequency division multiplexing (FDM) 0b1: Code division multiplexing (CDM) |
| If (SMT == 0) { |  |  |
| Decimation value D | 2 | 0b00: 4 0b01: 8 0b10: 16 (or 18 TBD) 0b11: 32 |
| } else { |  |  |
| Max Cyclic Shift Index P | 2 | 0b00: 4 0b01: 8 0b10: 16 (or 18 TBD) 0b11: 32 |
| } |  |  |
| … |  |  |
| } |  |  |

----------------------------------------------- End of Text Proposal #1 -------------------------------------------------

Modify table 684 and text in the section 15.3.6.5.2.11 UL Sounding Command A-MAP IE

-------------------------------------------------- Start Text Proposal #2 ------------------------------------------------

Table 684 - UL Sounding Command A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| UL Sounding Command IE format() { | - | - |
| A-MAP IE type | 4 |  |
| Sounding subframe | 3 | Indicates the sounding subframe [TBD] |
| Sounding subband bitmap | Variable [max. 12] | [TBD] FFT size dependant TBD |
| If (Multiplexing type == 0) { |  |  |
| Decimation offset d | 4 | Unique decimation offset |
| } else { |  |  |
| Cyclic time shift n | 4 | Unique cyclic shift [TBD] |
| } |  |  |
| Periodicity (n) | 3 | 0b000 = Single command, not periodic, or terminate the periodicy. Otherwise, repeat sounding once per 2(n-1) frames, where n is decimal value of the periodicy field |
| Multi-antenna sounding | 1 | 0b0: sounding with antenna switching 0b1: sounding from multiple antennas |
| MCRC | 16 | 16 bit CRC masked by Station ID |
| } |  |  |

Table 684 specifies the fields of UL Sounding Command A-MAP IE used by the ABS to request sounding transmission by the AMS. Decimal equivalent of the sounding subframe indicates the uplink subframe with sounding symbol. The sounding subband bitmap filed is used to indicate the sounding subbands used in the sounding allocation. For that purpose, the Nused contiguous subcarriers are further subdivided into sounding subbands, where each sounding subband compromises N1 * Nsc adjacent subcarriers with Nsc = 18 for NFFT = 512, 1024 and Nsc = 36 for NFFT = 2048. For multiplexing type equal to 0 the first subcarrier index (actual decimation offset) of ith transmit antenna within sounding allocation is determined from g = F(d , k, Frame Index) g = PermSeq(d + i - 1), where function F() is TBD PermSeq is permutation sequence of length D specified in section 15.3.5.3.3 and is determined by SEED={IDCell*1367}mod210 TBD (option 2, g = d + i - 1). The three periodicy bits are used to indicate the MS to periodically repeat the sounding transmission. Setting periodicy bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed. For multi-antenna flag equals to 0 the MS sounds with antenna switching, while for multi-antenna flag equals to 1 the MS sounds all transmit antennas. If multi-antenna sounding field equals to 1 then the ith antenna of the MS corresponds to index = d + i - 1 for multiplexing type 0 or to n + i - 1 for multiplexing type 1.

----------------------------------------------- End of Text Proposal #2 -------------------------------------------------