| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MIMO midamble sequence proposal (AWD-15.3.5) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Alexei Davydov Huaning Niu Yuan Zhu Jiacheng Wang Yuval Lomnitz Ayelet Doron Gregory Morozov Yang-seok Choi Guangjie Li Qinghua Li Intel Corporation | alexei.davydov@intel.com huaning.nui@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.5 (DL-PHY) “Comments on AWD 15.3.5 (DL-PHY)” |  |
| Abstract | The contribution proposes further details of DL MIMO midamble |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MIMO midamble proposal

Alexei Davydov, Huaning Niu, Yuan Zhu, Jiacheng Wang, Guangjie Li, Qinghua Li, Yang-seok Choi

Intel Corporation

- Motivation

Some details of MIMO midamble physical structure has been adopted in [1]. In accordance to the design [1] MIMO midamble occupies single OFDMA symbol in the DL subframe. Further details such as MIMO midamble sequence, subcarrier mapping rule and MIMO midamble symbol location in the DL frame are not specified. In this contribution we propose missing details of the MIMO midamble physical structure.

- Text proposal for inclusion in the 802.16m AWD

- Add text on page 151 line 54 of section 15.3.4.2 MIMO midamble as shown below

-------------------------------  Text Start  ---------------------------------------------------

The MIMO midamble signal transmitted by the ABS antenna is defined by:

k≠Nused−12k=Nused−1

where bk is a complex coefficients modulating subcarriers in the midamble symbol defined from equation below

bk=1−2⋅G([k+u+offsetD(fft)]modfft),k≠Nused−12,kmodNt=g+k−sN1∗NscmodNt

where k is the subcarrier index (0 ≤k ≤ Nused -1), Nused is the number of used subcarriers, G(x) is the Golay sequence defined in Table 1 (0 ≤ x ≤ 2047). fft is the FFT size used, u is a shift value (0 ≤ u ≤ 127), where the actual value of u is derived from u = mod(BSID, 128), offsetD(fft) is an FFT size specific offset as defined in Table 2, g is a ABS transmit antenna index, Nt is number of ABS transmit antennas, parameter s = 0, for k≤(Nused - 1)/2 and s = 1, for k>(Nused - 1)/2.

Example of physical structure of MIMO midamble is shown in Figure x for 4TX antenna.

Figure X. MIMO midamble physical structure for 4TX antennas

MIMO midamble shall be transmitted with boosting of 2dB (TBD).

| Table 1 – Golay sequence of length 2048 bits |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0x121D | 0x12E2 | 0xEDE2 | 0x12E2 | 0x121D | 0x12E2 | 0x121D | 0xED1D |
| 0xEDE2 | 0xED1D | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0xEDE2 | 0x12E2 | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |
| 0x121D | 0x12E2 | 0x121D | 0xED1D | 0xEDE2 | 0xED1D | 0x121D | 0xED1D |  |  |  |  |

| Table 2 – Offsets in the Golay sequence |  |
| --- | --- |
| FFT size | Offset |
| 2048 | 30 |
| 1024 | 60 |
| 512 | 40 |

-------------------------------  Text End  ---------------------------------------------------

- References

[1] IEEE 802.16m Amendment Working Document (AWD), IEEE 802.16m-09/0010r2