| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Deallocation Confirmation in Feedback Allocation A-MAP Information Elements for UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Yuan Zhu, Qinghua Li, Huaning Niu, Yi Hsuan, Hongmei Sun, Guangjie Li, Rath Vannithamby, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan, Intel Corporation | {yuan.y.zhu, qinghua.li, huaning.niu, jong-kae.fwu}@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” (Chapter 15.3.6 DL control, Chapter 15.3. 9 UL-CTRL) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Deallocation Confirmation in Feedback Allocation A-MAP Information Elements in the 802.16m AWD

Yuan Zhu, Qinghua Li, Huaning Niu, Yi Hsuan Hongmei Sun, Guangjie Li, Rath Vannithamby, Jong-Kae (JK) Fwu, Feng Zhou, Hujun Yin, Jeong Julie Lee, Yang-seok Choi, Roshni M Srinivasan

Intel Corporation

- Introduction

This contribution provides further amendment text for feedback allocation A-MAP IEs to be included in Section 15.3.6.5 in the 802.16m Amendment Working Document [1]. Since feedback channel allocation is performed similarly as persistent allocation, deallocation functioning is needed in case BS may want to early release feedback channel when it is not needed any more. In case when feedback allocation A-MAP IE is sent to deallocate a feedback channel, there is chance that the A-MAP IE was lost in the air due to poor channel condition. In such cases, MS will keep sending feedback on the deallocated channel, and it will cause collision and inter-cell interference rise without being known by BS. Though blindly detection feedback allocation A-MAP IE loss at BS side is possible, it may take longer time to have a confident detection and it also rely on a good BS implementation. Thus a confirmation step is required to make feedback deallocation procedure more robust. When doing feedback channel deallocation, it is proposed for the BS to explicitly allocate one UL ACK channel for MS to send an ACK signal when deallocation A-MAP is received.

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

# Proposed AWD text Change

# 15.3.6.5.2.9 Feedback Allocation A-MAP IE

Table 1 describes the fields in a Feedback Allocation A-MAP IE used for dynamically allocating or de-allocating UL fast feedback control channels (including both PFBCH and SFBCH) to an AMS.

Definitions of the fields in the Feedback Allocation A-MAP IE are listed below in Table 1.

Table 1 Feedback Allocation A-MAP IE

Note: 1. Need to differentiate it is in OL region or not. In OL region, MaxMt is not required

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { | - | - |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable | - Feedback channel index within the UL fast feedback control resource region - (Dependent on LFB,FPi defined in section 15.3.8.3.3.2) |
| Allocation Period (p) | [3] | A feedback is transmitted on the FBCH every 2pframes |
| Long-term Allocation Period (TBD) |  | TBD |
| Frame offset (TBD) |  | Depends on the decision of A-MAP allocation IE |
| Allocation Duration(d) | [3] | 0b00 is reserved for feedback deallocation |
| ACK Allocation Flag | [1] | Indicate if one ACK channel is allocated |
| If (ACK Allocation Flag == 0b1){ |  |  |
| HFA |  | HARQ feedback allocation |
| } |  |  |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2: - 0b0: 1 - 0b1: 2 - If Nt=4: - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO) - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8: (MU-MIMO) - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| … |  |  |

Channel Index: Uniquely identifies a fast feedback channel on which an AMS can transmit fast feedback information. With this allocation, a one-to-one relationship is established between Channel Index and the AMS. In case of multiple SFBCH are allocated for an AMS, it is assumed that multiple SFBCH are allocated contiguously and this field specifies the index of the first one.

ACK Allocation Flag: BS may set ACK Allocation Flag to 0b1 if Allocation Duration equals to 0b00. MS may set ACK Allocation Flag to 0b1 if Allocation Duration doesn’t equal to 0b00 and deallocation is implicitly made to the FBCH allocated by the last Feedback Allocation IE.