| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for differentiating bandwidth request based on service class and retry attempts |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Abhay Karandikar Srinadh B, Prateek Kapadia, Harshad Maral Kiran Kuchi, J Klutto Milleth ¦ CEWiT, India | E-mail: karandi@ieee.org (+91 9820347269) kkuchi@cewit.org.in (+91 9962045186) klutto@cewit.org.in (+91 9444107970) |
| Re: | Call for Comments on IEEE 802.16m-09/0010r2 IEEE 802.16m Amendment text/Area: Chapter 15.2.11.1.1. (Bandwidth Request) |  |
| Abstract | Proposal for differentiating bandwidth request based on service class and retry attempts |  |
| Purpose | For inclusion in AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

###### Proposal for differentiating bandwidth request based on service class and retry attempts

Abhay Karandikar, Srinadh B, Prateek Kapadia, Harshad Maral

Telecom Centre of Excellence (TCOE), IIT-Bombay

Kiran Kuchi, J Klutto Milleth

CEWiT, India

# Introduction:

The current procedure for differentiation in the bandwidth request contention process is not defined in the standard. This procedure must be BS controlled so that access control classification between different QoS and priority users is maintained. Also, values for the backoff timers in case of retry must be defined as part of the MAC protocol. The following text proposes a mechanism for the BS to differentiate bandwidth contention using the Qo

## ……………………..Start of Text……………………………………………………….

## ……………………..End of Text……………………………………………………….