| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Yi Hsuan Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Category: AWD comment / Section: (AWD - 15.3.6.5.2.2 ) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2)

Roshni Srinivasan, Hujun Yin, Yi Hsuan

Intel Corporation

- Introduction

Currently 4 bits are allocated to signal the DL/UL assignment A-MAP IE type in the 802.16m AWD [1] in tables 668 and 670. It is proposed that 3 bits are allocated to signal the type of assignment A-MAP IE. These 3 bits cover the following A-MAP IE types

- Two basic assignment A-MAP IEs are defined for resource allocation in the DL and the UL. These IEs are expected to be the most frequently used IE types and cover a broad set of features supported by 802.16m including

  - Support for 2, 4 and 8 tx antennas on the DL and support for 2 and 4 tx antennas on the UL

  - All open and closed loop, single user and multi-user MIMO modes

  - HARQ IR

  - Resource indexing for 5,10 and 20 MHz system bandwidths

- Two extended IE types for the DL and the UL, which used when the size of the IE is either larger or smaller than the basic. Extended IEs are further classified into multiple types, which are identified by the first field in the extended IE. The length of the IE and the corresponding fields are derived from the extended IE type.

IEs used for control signaling such as the feedback allocation IE, sounding allocation IE, group resource allocation and group configuration IEs are included in the extended IE category

- Two IE types for DL and UL persistent IEs.

- Two IE types are also reserved to specify information elements that may be required in the future.

- Instructions to editor:

- The following changes have been included in the text proposal

- Change size of A-MAP IE Type field in DL basic assignment A-MAP IE to 3 bits in Table 668

- Replace Table 669 with the updated table in the proposed text

- Change size of A-MAP IE Type field in UL basic assignment A-MAP IE to 3 bits in Table 670

Proposed AWD text has been underlined in red, AWD text that has been deleted has been struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal ---------------------------------------------------

# DL Basic Assignment A-MAP IE

Table 668 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL.

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 668.

Table 668- DL basic assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 3 | DL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| . . . . . |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between UL/DL basic/extended IEs. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format

PSI: Allocated pilot stream index for horizontal encoding

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

RA: Resource Allocation information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

Long TTI Indicator: Indicator to signal allocations span multiple subframes in time.

Allocation Relevance: Subframe index corresponding to the A-MAP region in which the resource is allocated

HFA: TBD allocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.

Table 669 describes Assignment A-MAP IE Types

Table 669: A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b000 | DL Basic Assignment A-MAP IE |
| 0b001 | UL Basic Assignment A-MAP IE |
| 0b010 | DL Extended Assignment A-MAP IE |
| 0b011 | UL Extended Assignment A-MAP IE |
| 0b100 | DL Persistent A-MAP IE |
| 0b101 | UL Persistent A-MAP IE |
| 0b110 | Reserved |
| 0b111 | Reserved |

Table 669- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Reserved |
| 0b0011 | Reserved |
| 0b0100 | Reserved |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

- Change size of A-MAP IE Type field in DL basic assignment A-MAP IE to 3 bits from 4 bits

# UL Basic Assignment A-MAP IE

Table 670 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 670 UL basic assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 3 | UL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| . . . . . |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM

- PF: Precoding flag to indicate adaptive or non-adsptive precoding

- PMI: Precoding matrix index

-------------------------------  End Text Proposal ---------------------------------------------------