| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2) |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Yi Hsuan Intel Corporation Hyunkyu Yu Samsung Electronics Yanfeng Guan ZTE Corporation | roshni.m.srinivasan@intel.com hk.yu@samsung.com yanfeng.guan@zte.com.cn |
| Re: | Category: AWD comment / Section: (AWD – 15.3.6.5.2.2 ) |  |
| Abstract | The contributionon proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the DL and UL A-MAP Information Element Types in the 802.16m AWD (AWD- 15.3.6.5.2.2)

Roshni Srinivasan, Hujun Yin, Yi Hsuan, Intel Corporation

Hyunkyu Yu, Samsung Electronics

Yanfeng Guan, ZTE Corporation

- Introduction

Currently 4 bits are allocated to signal the DL/UL assignment A-MAP IE type in the 802.16m AWD [1]. Table 669 was defined to enumerate the different IE types that can be identified by these 4 bits. The table currently contains only two entries related to the DL and UL Basic Assignment IEs.

The proposed text includes A-MAP IE types that are currently defined in the AWD as well as additional IE types required for system operation.

- The DL and UL Basic Assignment IEs are used to assign resources span one or more contiguous LRUs.

- The DL and UL Subband Assignment IEs are defined to specify resource indices that cover non-contiguous allocations in one or more subbands

- The Access A-MAP IE type is a placeholder for IEs that cover initial, periodic and bandwidth request ranging

- The remaining IE types that are listed are currently included in the AWD [1]

Proposed AWD text has been underlined in red, AWD text that has been deleted has been struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal ---------------------------------------------------

# DL Basic Assignment A-MAP IE

.

.

.

Table 669 describes Assignment A-MAP IE Types

Table 669- A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Reserved DL Subband Resource Assignment A-MAP IE |
| 0b0011 | Reserved UL Subband Assignment A-MAP IE |
| 0b0100 | Reserved Feedback Allocation A-MAP IE |
| 0b0101 | Reserved UL Sounding Command A-MAP IE |
| 0b0110 | Reserved Access A-MAP IE |
| 0b0111 | Reserved DL Persistent A-MAP IE |
| 0b1000 | Reserved UL Persistent A-MAP IE |
| 0b1001 | Reserved DL Group Configuration A-MAP IE |
| 0b1010 | Reserved UL Group Configuration A-MAP IE |
| 0b1011 | Reserved DL Group Resource Allocation A-MAP IE |
| 0b1100 | Reserved UL Group Resource Allocation A-MAP IE |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-------------------------------  End Text Proposal ---------------------------------------------------