| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to MIMO Signaling in the 802.16m UL Basic Assignment A-MAP Information Element (AWD - 15.3.6.5.2.3) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Huaning Niu, Alexei Davydov Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Category: AWD comment / Section: (AWD - 15.3.6.5.2.3) |  |
| Abstract | The contribution proposes changes to the UL A-MAP IE in the 802.16m AWD for more efficient MIMO signaling |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to MIMO Signaling in the 802.16m UL Basic Assignment A-MAP Information Element (AWD - 15.3.6.5.2.3)

Roshni Srinivasan, Hujun Yin, Huaning Niu, Alexei Davydov

Intel Corporation

- Introduction

This contribution proposes changes to the Basic UL Assignment A-MAP IE currently defined in the 802.16m AWD [1] to include more efficient signaling for MIMO operation. The proposed modifications reduce 3 bits of signaling overhead while supporting all possible MIMO formats currently described in the AWD for 2 and 4 tx antennas on the uplink.

The Basic UL A-MAP IE currently supports all UL MIMO modes defined in the 802.16m AWD [1]. Based on the fields included in the IE, it can be seen that total size of the fields associated with MIMO operation in the Basic UL A-MAP IE is largest when in the 4tx antenna case when 4-stream Collaborative Spatial Multiplexing (CSM) and 6 bit Precoding Matrix Index (PMI) are signaled by the ABS. In order to reduce the signaling overhead associated with this mode, it is proposed that redundant signaling in the IE is eliminated. The following considerations have been taken into account in the proposed update:

- The introduction of a flag for CSM-PMI (right-most branch in flow chart) enables separation of the format with the highest signaling overhead from the other cases. In order to reduce signaling overhead when both PMI and CSM are signaled, TNS, Mt and SI fields can be combined to cover all possible combinations required using 4 bits instead of using 6 bits (2 bits each for TNS, Mt and SI).

- The MIMO Encoder Format (MEF) field and CSM flags are currently signaled for all MIMO transmission formats used in the UL.

  - Since vertical encoding is always used with CSM, the MEF need not be signaled.

  - The CSM flag is only required to distinguish between SU-MIMO and CSM when the PMI is not explicitly signaled. It need not be signaled for other transmission formats used in the UL.

  - The MEF (SFBC or vertical encoding) needs to be signaled only when neither CSM nor PMI is signaled.

- The precoding flag (PF) need not be signaled always. It is only used when the PMI is not signaled and the type of precoding used needs to be identified.

Non-adaptive precoding shall be used for SFBC. For CSM and SU-MIMO with Vertical Encoding, the PF is used to identify the type of precoding used.

  - When the PF is set to 0b0, the MS uses non-adaptive precoding

  - When PF is set to 0b1, the MS uses adaptive codebook precoding. This is done using the precoder of rank Mt of MS’s choice.

Figure 1 describes the parsing of information fields in the basic UL A-MAP IE to determine the MEF and associated parameters used for transmission on the UL.

Figure 1: MIMO Signaling in the UL Basic Assignment A-MAP IE

Since the IE fields have been reorganized to reflect the flow of information in the above figure, it is proposed that MIMO signaling in Table 670 is replaced by fields in the table below.

The Proposed AWD text has been underlined in red, AWD text that has been deleted has been struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal ---------------------------------------------------

# UL Basic Assignment A-MAP IE

Table 670 describes the fields in a UL Basic Assignment A-MAP IE used for resource assignment in the UL.

Table 670 - UL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| UL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | UL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| CSM-PMI Indicator | 1 | Flag to indicate if both CSM and PMI are signaled 0b0: SU-MIMO with or without PMI indication , or CSM without PMI indication 0b1: CSM with PMI indication |
| if (CSM-PMI Indicator == 0b0){ |  |  |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ), up to 4 streams per AMS supported 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| SU-PMI Indicator | 1 | Flag to indicate if both SU-MIMO and PMI are signaled 0b0: SU MIMO without PMI indication, or CSM without PMI indication 0b1: SU MIMO with PMI indication |
| if (SU-PMI Indicator == 0b0){ |  |  |
| CSM | 1 | Flag to indicate CSM 0b0: CSM 0b1: SU-MIMO |
| if (CSM == 0b0){ |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding - Non-adaptive precoding shall be used at the MS with SFBC. |
| if (MEF == 0b1){ |  |  |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| } |  |  |
| }else if (CSM == 0b1){ |  | Parameters for CSM with adaptive precoding |
| PF | 1 | Precoding flag for SU-MIMO when PMI is not signaled 0b0: Non-adaptive precoding 0b1: Adaptive codebook precoding using the precoder of rank Mt of MS's choice |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | 1 | First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | 2 | First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| }else if (SU-PMI Indicator == 0b1){ |  | Parameters for SU-MIMO with PMI ( adaptive precoding) |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| }else if (CSM-PMI Indicator == 0b1){ |  | Parameters for CSM with PMI (adaptive precoding) |
| CSM Format | 4 | For non CSM, modes, TNS = Mt. For CSM modes the following combinations apply for upto 4 streams 0b0000: TNS = 2, Mt=1, SI =1 0b0001: TNS = 2, Mt=1, SI =2 0b0010: TNS = 3, Mt=1, SI =1 0b0011: TNS = 3, Mt=1, SI =2 0b0100: TNS = 3, Mt=1, SI =3 0b0101: TNS = 3, Mt=2, SI =1 0b0110: TNS = 3, Mt=2, SI =2 0b0111: TNS = 4, Mt=1, SI =1 0b1000: TNS = 4, Mt=1, SI =2 0b1001: TNS = 4, Mt=1, SI =3 0b1010: TNS = 4, Mt=1, SI =4 0b1011: TNS = 4, Mt=2, SI =1 0b1100: TNS = 4, Mt=2, SI =2 0b1101: TNS = 4, Mt=2, SI =3 0b1110: TNS = 4, Mt=3, SI =1 0b1111: TNS = 4, Mt=3, SI =2 |
| if(Nt == 2){ |  |  |
| PMI | 3 | 3 bits PMI for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | 6 | 6 bits PMI for Nt = 4 |
| } |  |  |
| } |  |  |
| MEF | 1 | MIMO encoder format - 0b0: SFBC - 0b1: Vertical encoding |
| CSM | 1 | - 0b0: CSM enabled 0b1: No CSM enabled |
| If(MEF == 0b1){ |  | Parameters for vertical encoding |
| Mt | 2 | Number of streams in transmission (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| If(CSM == 0b1){ |  |  |
| TNS | 2 | - Total number of streams in the LRU for CSM 0b00: reserved 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| if(TNS == 2){ |  |  |
| SI | - 1 | - First pilot index for CSM with TNS = 2 |
| } else{ |  |  |
| SI | - 2 | - First pilot index for CSM with TNS = 3,4 |
| } |  |  |
| } |  |  |
| PF | 1 | - Precoding Flag - 0b0: non adaptive precoding 0b1: adaptive codebook precoding |
| If(PF == 0b1){ |  |  |
| PMI Indicator | 1 | - PMI Indicator - 0b0: the MS shall use the precoder of rank Mt of its choice - 0b1: the indicated PMI of rank Mt shall be used by the MS for precoding |
| if (PMI Indicator == 0b1){ |  |  |
| if(Nt == 2){ |  |  |
| PMI | - 3 | - Precoding Matrix Index for Nt = 2 |
| } else if (Nt == 4){ |  |  |
| PMI | - 6 | - Precoding Matrix Index for Nt = 4 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 UL subframes for FDD or all UL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL subframe relevant to an A-MAP region 0b1: Allocation in the second UL subframe relevant to an A-MAP region |
| }else if (DL:UL == 3:5){ |  |  |
| Allocation Relevance | 2 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) and DL:UL subframe ratio is 3:5 i.e., the first A-MAP region is relevant to the first two UL subframes and the second A-MAP region is relevant to the last 3 UL subframes 0b00: Allocation in the first UL subframe relevant to an A-MAP region 0b01: Allocation in the second UL subframe relevant to an A-MAP region 0b10: Allocation in the third UL subframe relevant to an A-MAP region 0b11: reserved |
| } |  |  |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 3 | HARQ channel identifier |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

- TNS: Total number of streams in the LRU for CSM

- SI: First pilot index for CSM

CSM-PMI Indicator: Flag to indicate if both CSM and PMI are signaled

SU-PMI Indicator: Flag to indicate if both SU-MIMO and PMI are signaled

CSM: Flag to indicate if CSM is signaled

- PF: Precoding flag to indicate adaptive or non-adsptive adaptive precoding

- PMI: Precoding matrix index

-------------------------------  End Text Proposal---------------------------------------------------