| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes to the DL Basic Assignment A-MAP Information Element in the 802.16m AWD to support MLD (AWD - 15.3.6.5.2.2) |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Roshni Srinivasan, Alexei Davydov, Yang-seok Choi, Hujun Yin Intel Corporation David Mazzarese, Hyunkyu Yu Samsung Electronics Narayan Prasad, Amir Khojastepour, Meilong Jiang, Sampath Rangarajan, Linghang Fan, Tetsu Ikeda, Mohammad Madihian NEC Kishigami Takaaki, Isamu Yoshii Panasonic | roshni.m.srinivasan@intel.com d.mazzarese@samsung.com prasad@nec-labs.com kishigami.takaaki@jp.panasonic.com |
| Re: | Category: AWD comment / Section: (AWD – 15.3.6.5.2.2) |  |
| Abstract | The contribution proposes text for the DL/UL A-MAP IEs to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes to the DL Basic Assignment A-MAP Information Element in the 802.16m AWD to support MLD                                  (AWD - 15.3.6.5.2.2)

Roshni Srinivasan, Alexei Davydov, Yang-seok Choi, Hujun Yin

Intel Corporation

David Mazzarese, Hyunkyu Yu

Samsung Electronics

Narayan Prasad, Amir Khojastepour, Meilong Jiang, Sampath Rangarajan, Linghang Fan, Tetsu Ikeda, Mohammad Madihian

NEC

Kishigami Takaaki, Isamu Yoshii

Panasonic

- Introduction

This contribution proposes changes to the Basic DL A-MAP IE currently defined in the 802.16m AWD [1] to include support for maximum likelihood demodulation (MLD) in DL MU-MIMO operation when 2 streams are transmitted. A new two bit field denoted by Mp is defined for horizontal encoding with Nt=2, which is used to indicate the modulation constellation of the paired AMS. A new 4 bit field, Si is introduced in place of Mt and PSI for Nt= 4 or 8, to signal the combination of the number of streams in transmission, the allocated pilot stream index and the modulation constellation of paired user in the case of 2 stream transmission.

The key features of the Basic DL A-MAP with the proposed new MEF are:

- All features currently supported in the Basic DL A-MAP are retained without increasing the size (in bits) of the Basic DL A-MAP IE. Note that for horizontal encoding when MEF is 0b10, and the number of transmit antennas, Nt is either 4 or 8, 2 bits each are required to signal the number of streams in the transmission (Mt) and the pilot stream index (PSI) of the AMS to which the A-MAP IE is transmitted.

  - Since the number of streams is limited to 2 when Nt =2, only 1 bit for Mt and 1 bit for PSI are needed to identify the pilot positions corresponding to the intended AMS and the other co-scheduled AMS. The two bits that become available with Nt = 2 can therefore be used to convey the three possible options for the modulation constellation of the paired AMS without changing the A-MAP IE size.

  - When Nt = 4 or 8, the number of streams in  transmission (1,2,3 or 4) and the allocated pilot stream index (1,2,3 or 4) need to be signaled. If two bits are allocated to signal each of these fields, the modulation constellation for the paired MU-MIMO user cannot be signaled without increasing the size of the current A-MAP IE. A new 4 bit field, Si is introduced in place of Mt and PSI for Nt= 4 or 8, to signal the combination of  the number of streams in transmission and allocated pilot stream index, and the modulation constellation of paired user in the case of 2 stream transmission. Specifically the values of Si are specified as follows:

| Value of Si | Number of streams in transmission | Allocated pilot stream index | Modulation constellation of paired user |
| --- | --- | --- | --- |
| 0b0000 | 2 | 1 | QPSK |
| 0b0001 | 2 | 1 | 16QAM |
| 0b0010 | 2 | 1 | 64QAM |
| 0b0011 | 2 | 1 | not available |
| 0b0100 | 2 | 2 | QPSK |
| 0b0101 | 2 | 2 | 16QAM |
| 0b0110 | 2 | 2 | 64QAM |
| 0b0111 | 2 | 2 | not available |
| 0b1000 | 3 | 1 | n/a |
| 0b1001 | 3 | 2 | n/a |
| 0b1010 | 3 | 3 | n/a |
| 0b1011 | 4 | 1 | n/a |
| 0b1100 | 4 | 2 | n/a |
| 0b1101 | 4 | 3 | n/a |
| 0b1110 | 4 | 4 | n/a |
| 0b1111 | n/a | n/a | n/a |

- The proposed modification facilitates enhanced DL MU-MIMO operation in which two AMSs can be paired and modulation information of the paired user is signaled in the A-MAP IE allowing the AMS to perform MLD. The performance improvement resulting from the use of MLD over MMSE in DL MU-MIMO with AMS pairing has been demonstrated in [2] and [3]. It must be noted that MLD-type receivers (especially for two-stream transmission) are currently implemented for SU-MIMO operation. Consequently, the available hardware can be leveraged to obtain better performance for MU-MIMO operation without increasing the cost.

- For 2 stream transmissions, the option of horizontal encoding without modulation indication is also available to the ABS. Thus, if the ABS chooses not to provide the scheduled AMS with the modulation information of a paired AMS, it may do so by scheduling the AMS with Mp set to 0b11 with Nt =2 or by setting with Si to 0b0100 or 0b1000 for Nt = 4 or 8.

- If an AMS with only an MMSE implementation receives information on the modulation constellation of the paired user, it can simply disregard this information. The number of streams in the transmission and the allocated pilot stream index can be clearly identified from every value assigned to Si

Proposed AWD text has been underlined in red, AWD text that has been deleted has been struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

[2] Alexei Davydov, Intel Corporation, IEEE C802.16m-09/1312, July 2009.

[3] M. A. Khojastepour, NEC, “MU-MIMO: Demodulation at the Mobile Station” IEEE C802.16m-09/1014, April 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal  ---------------------------------------------------

# DL Basic Assignment A-MAP IE

Table 668 describes the fields in a DL Basic Assignment A-MAP IE used for resource assignment in the DL .

Definitions of the fields in the DL Basic Assignment A-MAP IE are listed following Table 668.

Table 668: DL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a |
| if (MEF == 0b01){ |  | Parameters for vertical encoding |
| if(Nt == 2){ |  |  |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| }else if(Nt == 4){ |  |  |
| Mt | 2 | Number of streams in transmission for Nt = 4 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| }else if(Nt == 8){ |  |  |
| Mt | 3 | Number of streams in transmission for Nt = 8 (Mt <= Nt ) 0b000: 1 stream 0b001: 2 streams 0b010: 3 streams 0b011: 4 streams 0b100: 5 streams 0b101: 6 streams 0b110: 7 streams 0b111: 8 streams |
| } else if(MEF == 0b10){ |  | Parameters for horizontal encoding |
| If(Nt == 2){ |  |  |
| PSI | 1 | Allocated pilot stream index for Nt = 2 0b0: #1 stream 0b1: #2 stream |
| Mt | 1 | Number of streams in transmission for Nt = 2 (Mt <= Nt ) 0b0: 1 stream 0b1: 2 streams |
| Mp | 2 | Modulation constellation of the paired user 0b00: QPSK 0b01: 16 QAM 0b10: 64 QAM 0b11: other modulation information not available. |
| } else { |  |  |
| PSI | 2 | Allocated pilot stream index for Nt = 4 or 8 0b00: #1 stream 0b01: #2 stream 0b10: #3 stream 0b11: #4 stream |
| Mt | 2 | Number of streams in transmission for Nt = 4 or 8 (Mt <= Nt ) 0b00: 1 stream 0b01: 2 streams 0b10: 3 streams 0b11: 4 streams |
| Si | 4 | Index used when Nt= 4 or 8, to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission 0b0000: 2 streams with PSI=stream1 and other modulation =QPSK 0b0001: 2 streams with PSI=stream1 and other modulation =16QAM 0b0010: 2 streams with PSI=stream1 and other modulation =64QAM 0b0011: 2 streams with PSI=stream1 and other modulation information not available 0b0100: 2 streams with PSI=stream2 and other modulation =QPSK 0b0101: 2 streams with PSI=stream2 and other modulation =16QAM 0b0110: 2 streams with PSI=stream2 and other modulation =64QAM 0b0111: 2 streams with PSI=stream2 and other modulation information not available 0b1000: 3 streams with PSI=stream1 0b1001: 3 streams with PSI=stream2 0b1010: 3 streams with PSI=stream3 0b1011: 4 streams with PSI=stream1 0b1100: 4 stream with PSI=stream2 0b1101: 4 streams with PSI=stream3 0b1110: 4 streams with PSI=stream4 0b1111: n/a |
| } |  |  |
| } |  |  |
| Resource Allocation | Variable | Variable number of bits - depends on system bandwidth. Information may include: - Type of resource unit (DRU/CRU) - Location (start/end) - Allocation size |
| Long TTI Indicator | 1 | Indicates number of subframes spanned by the allocated resource. 0b0: 1 subframe (default) 0b1: 4 DL subframes for FDD or all DL subframes for TDD |
| if (Nsubframe, A-MAP == 2){ |  |  |
| Allocation Relevance | 1 | Subframe index when an A-MAP region occurs every 2 subframes (Nsubframe, A-MAP =2) 0b0: Allocation in the first DL subframe relevant to an A-MAP region 0b1: Allocation in the second DL subframe relevant to an A-MAP region |
| } |  |  |
| HFA | [4] | TBD HARQ Feedback Allocation |
| AI_SN | 1 | HARQ identifier sequence number |
| ACID | 4 | HARQ channel identifier |
| SPID/CoRe Version | [3] | HARQ subpacket identifier for IR and Constellation Rearrangement version |
| Reserved | TBD | Reserved bits |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } | - | - |

A-MAP IE Type: Defines the structure of the A-MAP IE for the bits in the A-MAP IE following the A-MAP IE type field. A-MAP IE Type distinguishes between UL/DL basic/extended IEs. Additional IE types are reserved for future use.

MEF: MIMO Encoder Format

PSI: Allocated pilot stream index for horizontal encoding

Mt: Number of streams in transmission. The DL pilot pattern with Mt streams shall be used in the allocated resource.

Mp: Modulation constellation of the paired user for 2 stream MU-MIMO operation with Nt = 2

Si: Index used when Nt= 4 or 8, to identify the combination of the number of streams and the allocated pilot stream index in a transmission with MU-MIMO , and the modulation constellation of paired user in the case of 2 stream transmission

RA: Resource Allocation information is used to signal the type of resource unit allocated (DRU/CRU), the location (start/end) and allocation size.

Long TTI Indicator: Indicator to signal allocations span multiple subframes in time.

Allocation Relevance: Subframe index corresponding to the A-MAP region in which the resource is allocated

HFA: TBD allocation for HARQ feedback.

SPID/CoRe Version: Signaling for HARQ IR including HARQ subpacket identifier for IR and Constellation Rearrangement version.

MCRC: 16 bit CRC masked by Station ID/Flow ID.