| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD Proposal for DL/UL Resource Indexing in IEEE 802.16m (AWD - 15.3.6.2.2.2) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Roshni Srinivasan, Hujun Yin, Ping Wang, Huaning Niu Intel Corporation | roshni.m.srinivasan@intel.com |
| Re: | Category: AWD comment / Section: (AWD - 15.3.6.2.2.2) |  |
| Abstract | The contribution proposes text a resource indexing scheme for the DL and UL and proposed text to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AWD Proposal for DL/UL Resource Indexing in IEEE 802.16m         (AWD - 15.3.6.2.2.2)

Roshni Srinivasan, Hujun Yin, Ping Wang, Huaning Niu

Intel Corporation

- 1. Introduction

This contribution proposes an indexing scheme to allocate resources in the DL/UL Basic A-MAP IE currently defined in the 802.16m AWD [1]. The following design considerations were taken into account in the proposed scheme.

- Support for a large range of allocation sizes: As defined in Table 742 in [1], allocations can be made in integral multiples of 1 LRU, and the size can vary from 1 to Nmax, the maximum number of LRUS in a given frequency partition.

- Fine granularity in resource allocation for small allocation sizes to facilitate support for applications like VoIP.

- Support for allocation sizes at the high end of the range of assignable resources to facilitate transmissions at the peak rate.

- No constraints related to the location of the allocated resource, i.e., for a supported allocation size, the scheduler has complete flexibility in allocating the resource anywhere in the DL/UL subframe.

- Support for allocations in multiple system bandwidths: The resource indexing scheme proposed in this contribution can be used to allocate resources of any allowable size in a 5/10/20 MHz system.

- Reduced signaling overhead in the A-MAP IE by trading off flexibility in allocating larger allocation sizes without impacting link adaptation

For a given system bandwidth with Nmax LRUs, , the total number of assignable resource sizes is Nmax. For each assignable resource size, resources can be allocated in different locations in the subframe. If all possible combinations of resource size and location are assigned unique indices, then the total number of indices required is Nmax * (Nmax +1) /2. For 5, 10 and 20 MHz system bandwidths in an IEEE 802.16m system, such an unconstrained resource indexing scheme requires 9, 11 and 13 bits respectively.

A resource indexing scheme that uses 11 bits for 20 MHz is proposed in this contribution. The indexing scheme is extended to derive resource indices for allocations in 5MHz and 10MHz with full flexibility.

Based on the analysis in section 4, it is proposed that at least 11 bits are used for resource indexing for a 20 MHz system bandwidth. Since 9 and 11 bits are required for full flexibility in resource indexing in 5 and 10MHz system bandwidths respectively, reserving 11 bits in the DL and UL assignment A-MAP IEs would cover the requirements for 5, 10 and 20 MHz.

- 2. Assignable resources with 11 bit resource indexing for a 20MHz system bandwidth

Ideally, a resource indexing scheme should support indexing of the complete range of assignable resource sizes in a subframe, where each resource comprises of a set of contiguous LRUs. If the required number of bits for full flexibility in resource indexing exceeds the target number of bits reserved for signaling resource allocation in the A-MAP IE, then some resource sizes and/or locations can be eliminated from the set of assignable resource sizes and locations. In order to reduce scheduler complexity related to searching for assignable indices, the indexing scheme should not impose any constraints on location for a given resource size and the entire set of indices corresponding to a particular resource size should be eliminated.

It is proposed that resource indexing for 5 and 10MHz system bandwidths is designed for full flexibility i.e., 9 or 11 bits are allocated for resource indexing in the DL/UL A-MAP IEs for 5 MHz and 10 MHz system bandwidths respectively.

A resource indexing scheme for a 20 MHz system bandwidth that requires 11 bits is proposed. As explained earlier, the number of bits required for indexing is reduced at the expense of resource indexing flexibility. However, the penalty is minimized by taking into account the following considerations:

- For an assignable resource size, there are no constraints on location.

- All resource sizes in increments of 1 LRU are assignable in the range of1 to 12 LRUs with full flexibility in order to support scheduling of small allocations for applications such as VoIP.

- Resource sizes are indexed uniformly within a given range to minimize the impact on link adaptation.

  - Only even resource sizes are assignable in increments of 2 LRUs from sizes of 14 to 24 LRUs (Nmax = 24 for a 5 MHz system bandwidth)

  - Resource sizes are assignable in increments of 4 LRUs for sizes 28 to 48 LRUs (Nmax = 48 for a 10 MHz system bandwidth)

  - Resource sizes are assignable in increments of 8 LRUs for sizes 56 to 88 LRUs (Nmax = 96 for a 20 MHz system bandwidth)

- All resource sizes from 92 to 96 LRUs are indexed with no constraints on location in order to allow full flexibility in scheduling large allocations transmitted at the peak data rate. The starting point (92) for this range of resource sizes is chosen such that all the bits available for resource indexing with 11 bits are fully utilized.

The assignable resource sizes and corresponding LRUs indexed for the proposed scheme are listed in Table 1.

Table 1: Assignable Resources with 11-bit Resource Indexing for 20MHz

| Assignable Resource Sizes | Contiguous LRUs Indexed | Number of Indices |
| --- | --- | --- |
| 1 | {1},{2},{3},{4},{5},{6},{7},{8},{9},{10},{11},{12},{13},{14},{15},{16},{17}, {18}, {19}, {20}, {21}, {22}, {23}, {24}, {25}, {26}, {27}, {28}, {29}, {30}, {31}, {32}, {33}, {34}, {35}, {36}, {37}, {38}, {39}, {40}, {41}, {42}, {43}, {44}, {45}, {46}, {47},{48},{49},{50},{51}, {52},{53},{54},{55}, {56},{57},{58},{59},{60},{61},{62},{63},{64}, {65}, {66}, {67}, {68}, {69}, {70}, {71}, {72}, {73}, {74}, {75}, {76}, {77}, {78}, {79}, {80}, {81}, {82}, {83}, {84}, {85}, {86}, {87}, {88}, {89}, {90}, {91}, {92}, {93}, {94}, {95}, {96} | 96 |
| 2 | {1~2},{2~3},{3~4},{4~5},{5~6},{6~7},{7~8},{8~9},{9~10},{10~11},{11~12},{12~13},{13~14},{14~15},{15~16},{16~17},{17~18},{18~19},{19~20},{20~21},{21~22},{22~23},{23~24},{24~25},{25~26},{26~27},{27~28},{28~29},{29~30},{30~31},{31~32},{32~33},{33~34},{34~35},{35~36},{36~37},{37~38},{38~39},{39~40},{40~41},{41~42},{42~43},{43~44},{44~45},{45~46},{46~47},{47~48},{48~49},{49~50},{50~51},{51~52},{52~53},{53~54},{54~55},{55~56},{56~57},{57~58},{58~59},{59~60},{60~61},{61~62},{62~63},{63~64},{64~65},{65~66},{66~67},{67~68},{68~69},{69~70},{70~71},{71~72},{72~73},{73~74},{74~75},{75~76},{76~77},{77~78},{78~79},{79~80},{80~81},{81~82},{82~83},{83~84},{84~85},{85~86},{86~87},{87~88},{88~89},{89~90},{90~91},{91~92},{92~93},{93~94},{94~95},{95~96} | 95 |
| 3 | {1~3},{2~4},{3~5},{4~6},{5~7},{6~8},{7~9},{8~10},{9~11},{10~12},{11~13},{12~14},{13~15},{14~16},{15~17},{16~18},{17~19},{18~20},{19~21},{20~22},{21~23},{22~24},{23~25},{24~26},{25~27},{26~28},{27~29},{28~30},{29~31},{30~32},{31~33},{32~34},{33~35},{34~36},{35~37},{36~38},{37~39},{38~40},{39~41},{40~42},{41~43},{42~44},{43~45},{44~46},{45~47},{46~48},{47~49},{48~50},{49~51},{50~52},{51~53},{52~54},{53~55},{54~56},{55~57},{56~58},{57~59},{58~60},{59~61},{60~62},{61~63},{62~64},{63~65},{64~66},{65~67},{66~68},{67~69},{68~70},{69~71},{70~72},{71~73},{72~74},{73~75},{74~76},{75~77},{76~78},{77~79},{78~80},{79~81},{80~82},{81~83},{82~84},{83~85},{84~86},{85~87},{86~88},{87~89},{88~90},{89~91},{90~92},{91~93},{92~94},{93~95},{94~96} | 94 |
| 4 | {1~4},{2~5},{3~6},{4~7},{5~8},{6~9},{7~10},{8~11},{9~12},{10~13},{11~14},{12~15},{13~16},{14~17},{15~18},{16~19},{17~20},{18~21},{19~22},{20~23},{21~24},{22~25},{23~26},{24~27},{25~28},{26~29},{27~30},{28~31},{29~32},{30~33},{31~34},{32~35},{33~36},{34~37},{35~38},{36~39},{37~40},{38~41},{39~42},{40~43},{41~44},{42~45},{43~46},{44~47},{45~48},{46~49},{47~50},{48~51},{49~52},{50~53},{51~54},{52~55},{53~56},{54~57},{55~58},{56~59},{57~60},{58~61},{59~62},{60~63},{61~64},{62~65},{63~66},{64~67},{65~68},{66~69},{67~70},{68~71},{69~72},{70~73},{71~74},{72~75},{73~76},{74~77},{75~78},{76~79},{77~80},{78~81},{79~82},{80~83},{81~84},{82~85},{83~86},{84~87},{85~88},{86~89},{87~90},{88~91},{89~92},{90~93},{91~94},{92~95},{93~96} | 93 |
| 5 | {1~5},{2~6},{3~7},{4~8},{5~9},{6~10},{7~11},{8~12},{9~13},{10~14},{11~15},{12~16},{13~17},{14~18},{15~19},{16~20},{17~21},{18~22},{19~23},{20~24},{21~25},{22~26},{23~27},{24~28},{25~29},{26~30},{27~31},{28~32},{29~33},{30~34},{31~35},{32~36},{33~37},{34~38},{35~39},{36~40},{37~41},{38~42},{39~43},{40~44},{41~45},{42~46},{43~47},{44~48},{45~49},{46~50},{47~51},{48~52},{49~53},{50~54},{51~55},{52~56},{53~57},{54~58},{55~59},{56~60},{57~61},{58~62},{59~63},{60~64},{61~65},{62~66},{63~67},{64~68},{65~69},{66~70},{67~71},{68~72},{69~73},{70~74},{71~75},{72~76},{73~77},{74~78},{75~79},{76~80},{77~81},{78~82},{79~83},{80~84},{81~85},{82~86},{83~87},{84~88},{85~89},{86~90},{87~91},{88~92},{89~93},{90~94},{91~95},{92~96} | 92 |
| 6 | {1~6},{2~7},{3~8},{4~9},{5~10},{6~11},{7~12},{8~13},{9~14},{10~15},{11~16},{12~17},{13~18},{14~19},{15~20},{16~21},{17~22},{18~23},{19~24},{20~25},{21~26},{22~27},{23~28},{24~29},{25~30},{26~31},{27~32},{28~33},{29~34},{30~35},{31~36},{32~37},{33~38},{34~39},{35~40},{36~41},{37~42},{38~43},{39~44},{40~45},{41~46},{42~47},{43~48},{44~49},{45~50},{46~51},{47~52},{48~53},{49~54},{50~55},{51~56},{52~57},{53~58},{54~59},{55~60},{56~61},{57~62},{58~63},{59~64},{60~65},{61~66},{62~67},{63~68},{64~69},{65~70},{66~71},{67~72},{68~73},{69~74},{70~75},{71~76},{72~77},{73~78},{74~79},{75~80},{76~81},{77~82},{78~83},{79~84},{80~85},{81~86},{82~87},{83~88},{84~89},{85~90},{86~91},{87~92},{88~93},{89~94},{90~95},{91~96} | 91 |
| 7 | {1~7},{2~8},{3~9},{4~10},{5~11},{6~12},{7~13},{8~14},{9~15},{10~16},{11~17},{12~18},{13~19},{14~20},{15~21},{16~22},{17~23},{18~24},{19~25},{20~26},{21~27},{22~28},{23~29},{24~30},{25~31},{26~32},{27~33},{28~34},{29~35},{30~36},{31~37},{32~38},{33~39},{34~40},{35~41},{36~42},{37~43},{38~44},{39~45},{40~46},{41~47},{42~48},{43~49},{44~50},{45~51},{46~52},{47~53},{48~54},{49~55},{50~56},{51~57},{52~58},{53~59},{54~60},{55~61},{56~62},{57~63},{58~64},{59~65},{60~66},{61~67},{62~68},{63~69},{64~70},{65~71},{66~72},{67~73},{68~74},{69~75},{70~76},{71~77},{72~78},{73~79},{74~80},{75~81},{76~82},{77~83},{78~84},{79~85},{80~86},{81~87},{82~88},{83~89},{84~90},{85~91},{86~92},{87~93},{88~94},{89~95},{90~96} | 90 |
| 8 | {1~8},{2~9},{3~10},{4~11},{5~12},{6~13},{7~14},{8~15},{9~16},{10~17},{11~18},{12~19},{13~20},{14~21},{15~22},{16~23},{17~24},{18~25},{19~26},{20~27},{21~28},{22~29},{23~30},{24~31},{25~32},{26~33},{27~34},{28~35},{29~36},{30~37},{31~38},{32~39},{33~40},{34~41},{35~42},{36~43},{37~44},{38~45},{39~46},{40~47},{41~48},{42~49},{43~50},{44~51},{45~52},{46~53},{47~54},{48~55},{49~56},{50~57},{51~58},{52~59},{53~60},{54~61},{55~62},{56~63},{57~64},{58~65},{59~66},{60~67},{61~68},{62~69},{63~70},{64~71},{65~72},{66~73},{67~74},{68~75},{69~76},{70~77},{71~78},{72~79},{73~80},{74~81},{75~82},{76~83},{77~84},{78~85},{79~86},{80~87},{81~88},{82~89},{83~90},{84~91},{85~92},{86~93},{87~94},{88~95},{89~96} | 89 |
| 9 | {1~9},{2~10},{3~11},{4~12},{5~13},{6~14},{7~15},{8~16},{9~17},{10~18},{11~19},{12~20},{13~21},{14~22},{15~23},{16~24},{17~25},{18~26},{19~27},{20~28},{21~29},{22~30},{23~31},{24~32},{25~33},{26~34},{27~35},{28~36},{29~37},{30~38},{31~39},{32~40},{33~41},{34~42},{35~43},{36~44},{37~45},{38~46},{39~47},{40~48},{41~49},{42~50},{43~51},{44~52},{45~53},{46~54},{47~55},{48~56},{49~57},{50~58},{51~59},{52~60},{53~61},{54~62},{55~63},{56~64},{57~65},{58~66},{59~67},{60~68},{61~69},{62~70},{63~71},{64~72},{65~73},{66~74},{67~75},{68~76},{69~77},{70~78},{71~79},{72~80},{73~81},{74~82},{75~83},{76~84},{77~85},{78~86},{79~87},{80~88},{81~89},{82~90},{83~91},{84~92},{85~93},{86~94},{87~95},{88~96} | 88 |
| 10 | {1~10},{2~11},{3~12},{4~13},{5~14},{6~15},{7~16},{8~17},{9~18},{10~19},{11~20},{12~21},{13~22},{14~23},{15~24},{16~25},{17~26},{18~27},{19~28},{20~29},{21~30},{22~31},{23~32,{24~33},{25~34},{26~35},{27~36},{28~37},{29~38},{30~39},{31~40},{32~41},{33~42},{34~43},{35~44},{36~45},{37~46},{38~47},{39~48},{40~49},{41~50},{42~51},{43~52},{44~53},{45~54},{46~55},{47~56},{48~57},{49~58},{50~59},{51~60},{52~61},{53~62},{54~63},{55~64},{56~65},{57~66},{58~67},{59~68},{60~69},{61~70},{62~71},{63~72},{64~73},{65~74},{66~75},{67~76},{68~77},{69~78},{70~79},{71~80},{72~81},{73~82},{74~83},{75~84},{76~85},{77~86},{78~87},{79~88},{80~89},{81~90},{82~91},{83~92},{84~93},{85~94},{86~95},{87~96} | 87 |
| 11 | {1~11},{2~12},{3~13},{4~14},{5~15},{6~16},{7~17},{8~18},{9~19},{10~20},{11~21},{12~22},{13~23},{14~24},{15~25},{16~26},{17~27},{18~28},{19~29},{20~30},{21~31},{22~32},{23~33},{24~34},{25~35},{26~36},{27~37},{28~38},{29~39},{30~40},{31~41},{32~42},{33~43},{34~44},{35~45},{36~46},{37~47},{38~48},{39~49},{40~50},{41~51},{42~52},{43~53},{44~54},{45~55},{46~56},{47~57},{48~58},{49~59},{50~60},{51~61},{52~62},{53~63},{54~64},{55~65},{56~66},{57~67},{58~68},{59~69},{60~70},{61~71},{62~72},{63~73},{64~74},{65~75},{66~76},{67~77},{68~78},{69~79},{70~80},{71~81},{72~82},{73~83},{74~84},{75~85},{76~86},{77~87},{78~88},{79~89},{80~90},{81~91},{82~92},{83~93},{84~94},{85~95},{86~96} | 86 |
| 12 | {1~12},{2~13},{3~14},{4~15},{5~16},{6~17},{7~18},{8~19},{9~20},{10~21},{11~22},{12~23},{13~24},{14~25},{15~26},{16~27},{17~28},{18~29},{19~30},{20~31},{21~32},{22~33},{23~34},{24~35},{25~36},{26~37},{27~38},{28~39},{29~40},{30~41},{31~42},{32~43},{33~44},{34~45},{35~46},{36~47},{37~48},{38~49},{39~50},{40~51},{41~52},{42~53},{43~54},{44~55},{45~56},{46~57},{47~58},{48~59},{49~60},{50~61},{51~62},{52~63},{53~64},{54~65},{55~66},{56~67},{57~68},{58~69},{59~70},{60~71},{61~72},{62~73},{63~74},{64~75},{65~76},{66~77},{67~78},{68~79},{69~80},{70~81},{71~82},{72~83},{73~84},{74~85},{75~86},{76~87},{77~88},{78~89},{79~90},{80~91},{81~92},{82~93},{83~94},{84~95},{85~96} | 85 |
| 14 | {1~14},{2~15},{3~16},{4~17},{5~18},{6~19},{7~20},{8~21},{9~22},{10~23},{11~24},{12~25},{13~26},{14~27},{15~28},{16~29},{17~30},{18~31},{19~32},{20~33},{21~34},{22~35},{23~36},{24~37},{25~38},{26~39},{27~40},{28~41},{29~42},{30~43},{31~44},{32~45},{33~46},{34~47},{35~48},{36~49},{37~50},{38~51},{39~52},{40~53},{41~54},{42~55},{43~56},{44~57},{45~58},{46~59},{47~60},{48~61},{49~62},{50~63},{51~64},{52~65},{53~66},{54~67},{55~68},{56~69},{57~70},{58~71},{59~72},{60~73},{61~74},{62~75},{63~76},{64~77},{65~78},{66~79},{67~80},{68~81},{69~82},{70~83},{71~84},{72~85},{73~86},{74~87},{75~88},{76~89},{77~90},{78~91},{79~92},{80~93},{81~94},{82~95},{83~96} | 83 |
| 16 | {1~16},{2~17},{3~18},{4~19},{5~20},{6~21},{7~22},{8~23},{9~24},{10~25},{11~26},{12~27},{13~28},{14~29},{15~30},{16~31},{17~32},{18~33},{19~34},{20~35},{21~36},{22~37},{23~38},{24~39},{25~40},{26~41},{27~42},{28~43},{29~44},{30~45},{31~46},{32~47},{33~48},{34~49},{35~50},{36~51},{37~52},{38~53},{39~54},{40~55},{41~56},{42~57},{43~58},{44~59},{45~60},{46~61},{47~62},{48~63},{49~64},{50~65},{51~66},{52~67},{53~68},{54~69},{55~70},{56~71},{57~72},{58~73},{59~74},{60~75},{61~76},{62~77},{63~78},{64~79},{65~80},{66~81},{67~82},{68~83},{69~84},{70~85},{71~86},{72~87},{73~88},{74~89},{75~90},{76~91},{77~92},{78~93},{79~94},{80~95},{81~96} | 81 |
| 18 | {1~18},{2~19},{3~20},{4~21},{5~22},{6~23},{7~24},{8~25},{9~26},{10~27},{11~28},{12~29},{13~30},{14~31},{15~32},{16~33},{17~34},{18~35},{19~36},{20~37},{21~38},{22~39},{23~40},{24~41},{25~42},{26~43},{27~44},{28~45},{29~46},{30~47},{31~48},{32~49},{33~50},{34~51},{35~52},{36~53},{37~54},{38~55},{39~56},{40~57},{41~58},{42~59},{43~60},{44~61},{45~62},{46~63},{47~64},{48~65},{49~66},{50~67},{51~68},{52~69},{53~70},{54~71},{55~72},{56~73},{57~74},{58~75},{59~76},{60~77},{61~78},{62~79},{63~80},{64~81},{65~82},{66~83},{67~84},{68~85},{69~86},{70~87},{71~88},{72~89},{73~90},{74~91},{75~92},{76~93},{77~94},{78~95},{79~96} | 79 |
| 20 | {1~20},{2~21},{3~22},{4~23},{5~24},{6~25},{7~26},{8~27},{9~28},{10~29},{11~30},{12~31},{13~32},{14~33},{15~34},{16~35},{17~36},{18~37},{19~38},{20~39},{21~40},{22~41},{23~42},{24~43},{25~44},{26~45},{27~46},{28~47},{29~48},{30~49},{31~50},{32~51},{33~52},{34~53},{35~54},{36~55},{37~56},{38~57},{39~58},{40~59},{41~60},{42~61},{43~62},{44~63},{45~64},{46~65},{47~66},{48~67},{49~68},{50~69},{51~70},{52~71},{53~72},{54~73},{55~74},{56~75},{57~76},{58~77},{59~78},{60~79},{61~80},{62~81},{63~82},{64~83},{65~84},{66~85},{67~86},{68~87},{69~88},{70~89},{71~90},{72~91},{73~92},{74~93},{75~94},{76~95},{77~96} | 77 |
| 22 | {1~22},{2~23},{3~24},{4~25},{5~26},{6~27},{7~28},{8~29},{9~30},{10~31},{11~32},{12~33},{13~34},{14~35},{15~36},{16~37},{17~38},{18~39},{19~40},{20~41},{21~42},{22~43},{23~44},{24~45},{25~46},{26~47},{27~48},{28~49},{29~50},{30~51},{31~52},{32~53},{33~54},{34~55},{35~56},{36~57},{37~58},{38~59},{39~60},{40~61},{41~62},{42~63},{43~64},{44~65},{45~66},{46~67},{47~68},{48~69},{49~70},{50~71},{51~72},{52~73},{53~74},{54~75},{55~76},{56~77},{57~78},{58~79},{59~80},{60~81},{61~82},{62~83},{63~84},{64~85},{65~86},{66~87},{67~88},{68~89},{69~90},{70~91},{71~92},{72~93},{73~94},{74~95},{75~96} | 75 |
| 24 | {1~24},{2~25},{3~26},{4~27},{5~28},{6~29},{7~30},{8~31},{9~32},{10~33},{11~34},{12~35},{13~36},{14~37},{15~38},{16~39},{17~40},{18~41},{19~42},{20~43},{21~44},{22~45},{23~46},{24~47},{25~48},{26~49},{27~50},{28~51},{29~52},{30~53},{31~54},{32~55},{33~56},{34~57},{35~58},{36~59},{37~60},{38~61},{39~62},{40~63},{41~64},{42~65},{43~66},{44~67},{45~68},{46~69},{47~70},{48~71},{49~72},{50~73},{51~74},{52~75},{53~76},{54~77},{55~78},{56~79},{57~80},{58~81},{59~82},{60~83},{61~84},{62~85},{63~86},{64~87},{65~88},{66~89},{67~90},{68~91},{69~92},{70~93},{71~94},{72~95},{73~96} | 73 |
| 28 | {1~28},{2~29},{3~30},{4~31},{5~32},{6~33},{7~34},{8~35},{9~36},{10~37},{11~38},{12~39},{13~40},{14~41},{15~42},{16~43},{17~44},{18~45},{19~46},{20~47},{21~48},{22~49},{23~50},{24~51},{25~52},{26~53},{27~54},{28~55},{29~56},{30~57},{31~58},{32~59},{33~60},{34~61},{35~62},{36~63},{37~64},{38~65},{39~66},{40~67},{41~68},{42~69},{43~70},{44~71},{45~72},{46~73},{47~74},{48~75},{49~76},{50~77},{51~78},{52~79},{53~80},{54~81},{55~82},{56~83},{57~84},{58~85},{59~86},{60~87},{61~88},{62~89},{63~90},{64~91},{65~92},{66~93},{67~94},{68~95},{69~96} | 69 |
| 32 | {1~32},{2~33},{3~34},{4~35},{5~36},{6~37},{7~38},{8~39},{9~40},{10~41},{11~42},{12~43},{13~44},{14~45},{15~46},{16~47},{17~48},{18~49},{19~50},{20~51},{21~52},{22~53},{23~54},{24~55},{25~56},{26~57},{27~58},{28~59},{29~60},{30~61},{31~62},{32~63},{33~64},{34~65},{35~66},{36~67},{37~68},{38~69},{39~70},{40~71},{41~72},{42~73},{43~74},{44~75},{45~76},{46~77},{47~78},{48~79},{49~80},{50~81},{51~82},{52~83},{53~84},{54~85},{55~86},{56~87},{57~88},{58~89},{59~90},{60~91},{61~92},{62~93},{63~94},{64~95},{65~96} | 65 |
| 36 | {1~36},{2~37},{3~38},{4~39},{5~40},{6~41},{7~42},{8~43},{9~44},{10~45},{11~46},{12~47},{13~48},{14~49},{15~50},{16~51},{17~52},{18~53},{19~54},{20~55},{21~56},{22~57},{23~58},{24~59},{25~60},{26~61},{27~62},{28~63},{29~64},{30~65},{31~66},{32~67},{33~68},{34~69},{35~70},{36~71},{37~72},{38~73},{39~74},{40~75},{41~76},{42~77},{43~78},{44~79},{45~80},{46~81},{47~82},{48~83},{49~84},{50~85},{51~86},{52~87},{53~88},{54~89},{55~90},{56~91},{57~92},{58~93},{59~94},{60~95},{61~96} | 61 |
| 40 | {1~40},{2~41},{3~42},{4~43},{5~44},{6~45},{7~46},{8~47},{9~48},{10~49},{11~50},{12~51},{13~52},{14~53},{15~54},{16~55},{17~56},{18~57},{19~58},{20~59},{21~60},{22~61},{23~62},{24~63},{25~64},{26~65},{27~66},{28~67},{29~68},{30~69},{31~70},{32~71},{33~72},{34~73},{35~74},{36~75},{37~76},{38~77},{39~78},{40~79},{41~80},{42~81},{43~82},{44~83},{45~84},{46~85},{47~86},{48~87},{49~88},{50~89},{51~90},{52~91},{53~92},{54~93},{55~94},{56~95},{57~96} | 57 |
| 44 | {1~44},{2~45},{3~46},{4~47},{5~48},{6~49},{7~50},{8~51},{9~52},{10~53},{11~54},{12~55},{13~56},{14~57},{15~58},{16~59},{17~60},{18~61},{19~62},{20~63},{21~64},{22~65},{23~66},{24~67},{25~68},{26~69},{27~70},{28~71},{29~72},{30~73},{31~74},{32~75},{33~76},{34~77},{35~78},{36~79},{37~80},{38~81},{39~82},{40~83},{41~84},{42~85},{43~86},{44~87},{45~88},{46~89},{47~90},{48~91},{49~92},{50~93},{51~94},{52~95},{53~96} | 53 |
| 48 | {1~48},{2~49},{3~50},{4~51},{5~52},{6~53},{7~54},{8~55},{9~56},{10~57},{11~58},{12~59},{13~60},{14~61},{15~62},{16~63},{17~64},{18~65},{19~66},{20~67},{21~68},{22~69},{23~70},{24~71},{25~72},{26~73},{27~74},{28~75},{29~76},{30~77},{31~78},{32~79},{33~80},{34~81},{35~82},{36~83},{37~84},{38~85},{39~86},{40~87},{41~88},{42~89},{43~90},{44~91},{45~92},{46~93},{47~94},{48~95},{49~96} | 49 |
| 56 | {1~56},{2~57},{3~58},{4~59},{5~60},{6~61},{7~62},{8~63},{9~64},{10~65},{11~66},{12~67},{13~68},{14~69},{15~70},{16~71},{17~72},{18~73},{19~74},{20~75},{21~76},{22~77},{23~78},{24~79},{25~80},{26~81},{27~82},{28~83},{29~84},{30~85},{31~86},{32~87},{33~88},{34~89},{35~90},{36~91},{37~92},{38~93},{39~94},{40~95},{41~96} | 41 |
| 64 | {1~64},{2~65},{3~66},{4~67},{5~68},{6~69},{7~70},{8~71},{9~72},{10~73},{11~74},{12~75},{13~76},{14~77},{15~78},{16~79},{17~80},{18~81},{19~82},{20~83},{21~84},{22~85},{23~86},{24~87},{25~88},{26~89},{27~90},{28~91},{29~92},{30~93},{31~94},{32~95},{33~96} | 33 |
| 72 | {1~72},{2~73},{3~74},{4~75},{5~76},{6~77},{7~78},{8~79},{9~80},{10~81},{11~82},{12~83},{13~84},{14~85},{15~86},{16~87},{17~88},{18~89},{19~90},{20~91},{21~92},{22~93},{23~94},{24~95},{25~96} | 25 |
| 80 | {1~80},{2~81},{3~82},{4~83},{5~84},{6~85},{7~86},{8~87},{9~88},{9~89},{9~90},{9~91},{10~92},{11~93},{12~94},{13~95},{14~96} | 17 |
| 88 | {1~88},{2~89},{3~90},{4~91},{5~92},{6~93},{7~94},{8~95},{9~96} | 9 |
| 92 | {1~92},{2~93},{3~94},{4~95},{5~96} | 5 |
| 93 | {1~93},{2~94},{3~95},{4~96} | 4 |
| 94 | {1~94},{2~95},{3~96} | 3 |
| 95 | {1~95},{2~96} | 2 |
| 96 | {1~96} | 1 |
| Total | log2(96*97/2-2608) = log2(2048) = 11 bits | 2048 |

- 3. Resource Indexing with 11 bits for a 20MHz system bandwidth

Figure 1: Assignment structure for 11-bit Resource Indexing for 20MHz

- As can be seen from Table 1 and Figure 1, in order to determine the resource index RI for an allocation, the ABS requires the size S and index of the LRU from where the scheduled allocation begins, L.

- The ABS also maintains a vector Ia of length Nmax ,in which the non-zero entries contain the starting index for each of the K=34 assignable resource sizes listed in Table 1. The 11 bit resource indices in Ia run from 0 through 2047 and the entries in Ia are calculated as 0, (0+96-1), (0+96+95-1), (0+96+95+94-1), ….., 0, 0, (2047- 2-3-4-9-17-25), 0, 0, 0, 0, 0, 0, 0, (2047-2-3-4-9-17), 0, 0, 0, 0, 0, 0, 0, (2047-2-3-4-9), 0, 0, 0, (2047-2-3-4-5), (2047-2-3-4), (2047-2-3), (2047-2), 2047.

- Specifically, the ith element of Ia,  for 11 bit resource indexing for 20 MHz is defined as

- The resource index RI for an allocation of size S LRUs beginning at LRU L is computed as

- The ABS first determines if the required resource size is assignable by checking if the (S-1)th element in Ia has a non-zero value. If the size S is assignable, then the 11 bit resource index is then determined by simply adding L to the value of the Sth element in Ia. If the required resource is not assignable, the next higher or lower non-zero element in Ia is selected based on the link adaptation scheme employed.

- Note that the same scheme can be applied to the 5 and 10 Mhz system bandwidths where the length of Ia is 24 and 48 respectively and all values in Ia correspond to non-zero indices for every allocation size.

- The ith element of Ia, for 9 bit resource indexing for 5 MHz is defined as:

- The resource index RI for an allocation of size S LRUs beginning at LRU L is computed as

- Similarly, the ith element of Ia, for 11 bit resource indexing for 10 MHz are defined as:

- At the receiver the 11 bit index, RI in the A-MAP is decoded as follows. The value of element S in Ia with the smallest value equal to or less than the index is used to determine the assigned resource size S. Let this value be Ia (i).

- The starting LRU for the allocation is determined by subtracting the value of the Sth element in Ia from the index in the A-MAP, i.e.,

- L=RI- Ia (i)

- 4. Analysis of the impact on link adaptation with resource indexing with 12 bits for a 20MHz system bandwidth

In IEEE 802.16m, the effective code rate Re is determined by the burst size L (bytes) and resource size K (LRUs):

Re =L/K, where  depends on the subframe type and pilot pattern.

When the ABS receives an MPDU of size L and CQI feedback MCS level R0, it is desirable for BS to allocate right amount of resource to accommodate the MPDU at the desired MCS level R0. The effective code rate Re may not exactly match the desired code rate R0 because the allocated resource needs to be an integer multiple of LRUs. However, it is desirable to keep Re as close to R0 as possible, i.e.

Re−R0=mini∈[0,15]Re−MCSi

Table 2 lists the 16 levels of MCS feedback currently supported in IEEE 802.16m. It must be noted that the minimum step of code rate change is 34/256 (between MCS #0 and #1). Most steps are above 1/6 except MCS #0  #1.

If the minimum step between MCS levels that are fed back is denoted as:

Δmin=mini∈[0,14]MCSi+1−MCSi

from Table 2, it can be seen that

Table 2: Nominal MCS levels for 802.16m

| MCS index | Nominal code rate (R) | ΔR | MCS index | Nominal code rate (R) | ΔR |
| --- | --- | --- | --- | --- | --- |
| 0 | 62/256 |  | 8 | 620/256 | 108/256 |
| 1 | 96/256 | 34/256 | 9 | 740/256 | 120/256 |
| 2 | 142/256 | 46/256 | 10 | 810/256 | 70/256 |
| 3 | 202/256 | 60/256 | 11 | 942/256 | 132/256 |
| 4 | 270/256 | 68/256 | 12 | 1086/256 | 144/256 |
| 5 | 342/256 | 72/256 | 13 | 1230/256 | 146/256 |
| 6 | 408/256 | 66/256 | 14 | 1350/256 | 120/256 |
| 7 | 512/256 | 104/256 | 15 | 1422/256 | 72/256 |

Consider two types of resource indexing schemes (A) Full indexing, where all possible resource sizes N=[1,⋯,Nmax] are supported; (B) Partial indexing, where only a subset of the resource sizes are supported M⊂N.

Assume with full indexing (scheme A), Ni resource units can be allocated to best match the effective code rate Ri =L/Ni to the code rate R0 that is fed back. With scheme B, effective code rate Ri may not be supported because source size Ni is not supported. The scheduler may have to allocate next available higher or lower resource sizes (Mj-1 or Mj as shown in the figure below) which results in effective code rate Rj-1 =L/Mj-1 or Rj =L/Mj.

Let   and . Let ΔNi=Mj−Mj−1 then

Since and , we can approximateas

From the figure, it can be seen that either Rj-1 or Rj will approximate the desired code rate Ro if

Therefore, we have

ΔNiNi≤ΔminMCS15

From Table 2, we see that

In the proposed resource indexing scheme with 11 bits,

- In the ranges of 1 to 12 LRUs and 92 to 96 LRUs, there is full flexibility. In these ranges, the proposed indexing scheme does as well as the indexing scheme with full flexibility since allocations can be made in increments of 1 LRU.

- In the remaining ranges of allocation sizes, a comparison of the partial indexing scheme with the scheme with full flexibility shows that link adaptation is not adversely affected, since the requirement is met for every assignable size, Ni in the specified ranges.

  - In the range of 12 to 24 LRUs,  in the worst case.

  - In the range of 24 to 48 LRUs,  in the worst case.

  - In the range of 48 to 92 LRUs, partial indexing does not impact link adaptation since the requirement is met for every assignable size, Ni in the given range. In the worst case,

It can be seen that for all ranges of allocation sizes, thereby ensuring that link adaptation with the granularity of feedback MCS levels as defined in the 802.16m is not adversely affected by the proposed reduction in assignable resource indices.

Conversely it can be seen that any further reduction in the number of bits for resource indexing will increase the separation between assignable indices in some ranges and consequently have an adverse impact on link adaptation.

It is therefore recommended that at least 11 bits should be used for resource indexing for 20 MHz.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 2009.

- Amendment text proposal for inclusion in 802.16m-09/0010r2

- Instructions to Editor

- Section 15.3.6.2.2.2 in the AWD describes the Assignment A-MAP. It is proposed that a subsection ( 7 level heading?) on Resource Indexing is added under Section 15.3.6.2.2.2 to include the proposed text.

- -------------------------------  Start Text Proposal ---------------------------------------------------

- 15.3.6.2.2.2.1 Resource Indexing in the Assignment A-MAP

- The resource index RI for an allocation of contiguous LRUs in a DL/UL subframe contains information about the size (S) of the allocated resource in number of contiguous LRUs and index (L) of the LRU from where the scheduled allocation begins.

- Determining the Resource Index for the Scheduled Resource at the ABS

- For a given system bandwidth, the ABS maintains a vector Ia of length Nmax ,in which the non-zero entries contain the starting index for each of the assignable resource sizes.

- The ith element of Ia,  for 11 bit resource indexing in a DL/UL subframe for a 20 MHz system bandwidth is defined as

- The resource index RI for an allocation of size S LRUs beginning at LRU L is computed as

- The ABS first determines if the required resource size is assignable by checking if the (S-1)th element in Ia has a non-zero value. If the size S is assignable, then the 11 bit resource index is then determined by simply adding L to the value of the Sth element in Ia. If the required resource is not assignable, the next higher or lower non-zero element in Ia is selected based on the link adaptation scheme employed.

- The ith element of Ia, , for 11 bit resource indexing in a DL/UL subframe for a 10 MHz system bandwidth is defined as:

- The ith element of Ia,  for 9 bit resource indexing in a DL/UL subframe for a 5 MHz system bandwidth is defined as:

- The resource index RI for an allocation of size S LRUs beginning at LRU L is computed as

- Determining the Scheduled Resource from the Resource Index at the Receiver

- At the receiver the 11 bit index, RI in the A-MAP is decoded as follows. The value of element S in Ia with the smallest value equal to or less than the index is used to determine the assigned resource size S. Let this value be Ia (i).

- The starting LRU for the allocation is determined by subtracting the value of the Sth element in Ia from the index in the A-MAP, i.e.,

- L=RI- Ia (i)

-------------------------------  End Text Proposal ---------------------------------------------------