| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | aGPS BR indicator on FFB in 802.16m |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jie Hui Xiangying Yang Muthaiah Venkatachalam Intel Corporation Kiseon Ryu Ronny Yongho Kim Jeongki Kim LG Electronics | jie.hui@intel.com ksryu@lge.com chegal@lge.com |
| Re: |  |  |
| Abstract | This proposal describes stage-3 text for section 15.2.11.1.4 Bandwidth Request using FFB for the IEEE 802.16m Amendment. |  |
| Purpose | For discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

aGPS BR Indicator on FFB

for the IEEE 802.16m Amendment

Jie Hui, Xiangying Yang, Muthaiah Venkatachalam

Intel Corporation

Kiseon Ryu, Ronny Yongho Kim, Jeongki Kim

LG Electronics

- Introduction

The contribution proposes two usages of primary feedback channel (PFBCH) for bandwidth request, to be included in the section “15.2.11.1.4 Bandwidth Request using FFB” of 802.16m amendment. These two usages are to support the newly adopted Adaptive Granting and Polling Service (aGPS) in AWD.

- Text proposal for inclusion in the 802.16m amendment

-------------------------------------------------  Text Start  -------------------------------------------------

15.2.11.1.4 Bandwidth Request using FFB

15.2.11.1.4.1 aGPS Primary BR indicator

If an AMS has aGPS connections, the AMS may inform the serving ABS of the existence of pending aGPS data related to primary QoS parameters. The aGPS primary BR indicator (TBD: 0b111100) is used for that purpose. If the ABS receives this codeword on PFBCH from the AMS, the ABS should start allocation for the AMS UL bursts corresponding to the Primary service flow QoS parameter set of the AMS's aGPS UL service flow, which has the largest Grant_Size_primary among all aGPS service flows.

aGPS Primary BR indicator is also used for AMS to inform the serving ABS of the existence of pending ertPS data.

In order to maintain operation of link adaptation mechanisms at the ABS and adequate CINR reporting, the AMS shall not transmit the aGPS Primary BR indicator on two consecutive PFBCH allocations that are allocated to it.

15.2.11.1.4.2 aGPS Secondary BR indicator

If an AMS has aGPS connections, the AMS may inform the serving ABS of the existence of pending aGPS data related to secondary QoS parameters. The aGPS secondary BR indicator (TBD: 0b111101) is used for that purpose. If the ABS receives this codeword on PFBCH from the AMS, the ABS should start allocation for the AMS UL bursts corresponding to the Secondary service flow QoS parameter set of the AMS's aGPS UL service flow, which has the largest Grant_Size_secondary among all aGPS service flows.

In order to maintain operation of link adaptation mechanisms at the ABS and adequate CINR reporting, the AMS shall not transmit the aGPS Secondary BR indicator on two consecutive PFBCH allocations that are allocated to it.

-------------------------------------------------  Text end  -------------------------------------------------