| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for PFBCH contents for UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi Intel Corporation | huaning.niu@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” (Chapter 15.3.6 DL control, Chapter 15.3. 9 UL-CTRL) |  |
| Abstract | The contribution proposes text for the PFBCH content to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for PFBCH Content in the 802.16m AWD

Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron,

Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

This contribution provides amendment text for primary fast feedback information content to be included in Section 15.3.9.3.1.1 in the 802.16m Amendment Working Document [1].

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

# 15.3.9.3.1.1 Primary fast feedback control channel

The UL PFBCH carries 4 to 6 bits of information, providing wideband channel quality feedback and MIMO feedback.

Table 725 PFBCH Feedback Content

| Index | Content (Value) | Description/Notes |
| --- | --- | --- |
| 0 | Rank=1, MCS=0000 |  |
| 1 | Rank=1, MCS=0001 |  |
| 2 | Rank=1, MCS=0010 |  |
| 3 | Rank=1, MCS=0011 |  |
| 4 | Rank=1, MCS=0100 |  |
| 5 | Rank=1, MCS=0101 |  |
| 6 | Rank=1, MCS=0110 |  |
| 7 | Rank=1, MCS=0111 |  |
| 8 | Rank=1, MCS=1000 |  |
| 9 | Rank=1, MCS=1001 |  |
| 10 | Rank=1, MCS=1010 |  |
| 11 | Rank=1, MCS=1011 |  |
| 12 | Rank=1, MCS=1100 |  |
| 13 | Rank=1, MCS=1101 |  |
| 14 | Rank=1, MCS=1110 |  |
| 15 | Rank=1, MCS=1111 |  |
| 16 | Rank=2, MCS=0000 |  |
| 17 | Rank=2, MCS=0001 |  |
| 18 | Rank=2, MCS=0010 |  |
| 19 | Rank=2, MCS=0011 |  |
| 20 | Rank=2, MCS=0100 |  |
| 21 | Rank=2, MCS=0101 |  |
| 22 | Rank=2, MCS=0110 |  |
| 23 | Rank=2, MCS=0111 |  |
| 24 | Rank=2, MCS=1000 |  |
| 25 | Rank=2, MCS=1001 |  |
| 26 | Rank=2, MCS=1010 |  |
| 27 | Rank=2, MCS=1011 |  |
| 28 | Rank=2, MCS=1100 |  |
| 29 | Rank=2, MCS=1101 |  |
| 30 | Rank=2, MCS=1110 |  |
| 31 | Rank=2, MCS=1111 |  |
| 32 | Rank=3, MCS=0100 |  |
| 33 | Rank=3, MCS=0101 |  |
| 34 | Rank=3, MCS=0110 |  |
| 35 | Rank=3, MCS=0111 |  |
| 36 | Rank=3, MCS=1000 |  |
| 37 | Rank=3, MCS=1001 |  |
| 38 | Rank=3, MCS=1010 |  |
| 39 | Rank=3, MCS=1011 |  |
| 40 | Rank=3, MCS=1100 |  |
| 41 | Rank=3, MCS=1101 |  |
| 42 | Rank=3, MCS=1110 |  |
| 43 | Rank=3, MCS=1111 |  |
| 44 | Rank=4, MCS=0110 |  |
| 45 | Rank=4, MCS=0111 |  |
| 46 | Rank=4, MCS=1000 |  |
| 47 | Rank=4, MCS=1001 |  |
| 48 | Rank=4, MCS=1010 |  |
| 49 | Rank=4, MCS=1011 |  |
| 50 | Rank=4, MCS=1100 |  |
| 51 | Rank=4, MCS=1101 |  |
| 52 | Rank=4, MCS=1110 |  |
| 53 | Rank=4, MCS=1111 |  |
| 54 | CQI for frequency partition 0 (reuse-1) | AMS informs BS periodically the frequency partition of the reported CQI (for DL diversity mode) |
| 55 | CQI for frequency partition 1 (reuse-3) |  |
| 56 | CQI for frequency partition 2 (reuse-3) |  |
| 57 | CQI for frequency partition 3 (reuse-3) |  |
| 58 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 1) |  |
| 59 | Event-driven Indicator (EDI) for Bandwidth Request Indicator (sequence 2) |  |
| 60 | Event-driven Indicator (EDI) for Buffer management (80% full) |  |
| 61 | Event-driven Indicator (EDI) for Buffer management (full) |  |
| 62 | Reserve |  |
| 63 | Reserve |  |

The MCS index is defined in Table 744.