| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Format for UL Control in the 802.16m AWD |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi Intel Corporation | huaning.niu@intel.com |
| Re: | IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content” (Chapter 15.3.6 DL control, Chapter 15.3. 9 UL-CTRL) |  |
| Abstract | The contribution proposes text for the Feedback format to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Feedback Format in the 802.16m AWD

Huaning Niu, Hongmei Sun, Guangjie Li, Yuan Zhu, Qinghua Li, Ayelet Doron, Jong-Kae (JK) Fwu, Hujun Yin, Yang-seok Choi

Intel Corporation

- Introduction

This contribution provides amendment text for feedback format in fast feedback channel to be included in Section 15.3.9.3.1.1 in the 802.16m Amendment Working Document [1].

Proposed AWD text has been underlined in blue, AWD text that has been deleted has been marked in red and struck through and existing AWD text is shown in black.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r1a, March 2009.

[  ------------------- Proposed Text Starts --------------------]

# 15.3.9.3.1.4 Feedback format

# 15.3.9.3.1.4.1 Feedback format for MFM 0,1,4,7

Table xxx  Feedback formats for MIMO feedback mode 0,1,4,7

| MFM | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0,1 | PFBCH | 1 | W-CQI and STC rate |  | 6 | Joint coding of CQI and STC rate |
| 4,7 | PFBCH | 1 | Short term | W-CQI | 6 | Long term beamforming. For 4-Tx antennas, PMI indicates two codewords one in 6-bit base codebook and the other in 4-bit differential codebook, respectively. |
|  |  | 2 | Long term | W-PMI | 5: Nt=2 10: Nt=4 12: Nt=8 |  |

# 15.3.9.3.1.4.2 Feedback format for MFM 2

The detailed format is listed in Table 727. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] long term reporting period as defined in feedback allocation A-MAP IE.

Table 727  Feedback formats for MIMO feedback mode 2

| Feedback Format | FBCH | Number of reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M=1) | SFBCH | 1 | Short term | Rank Indication+CQI | 5-7 | See Error: Reference source not found MaxMt decides the maximum rank |
|  |  |  |  | Subband Selection | 5: 20MHz 4: 10MHz 3: 5MHz | Log2(N) |
| 1 (M=2) | SFBCH | 2 | Short term | Average CQI + Diff CQI | 8 | Log2N2 |
|  |  |  | Long term | Subband selection | 9: 20MHz 7: 10MHz 4: 5MHz |  |
|  |  |  |  | Rank Indication | 1-3 |  |
| 2 (M=3) | SFBCH | 2 | Short term | Average CQI + Diff CQI | 4+6=10 |  |
|  |  |  | Long term | Subband selection | 11: 20MHz 8: 10MHz 5: 5MHz | Log2N3 |
|  |  |  |  | Rank Indication | 1-3 | MaxMt decides the maximum rank |
| 3 (M =5) (TBD) | SFBCH | 2 | Long term | Subband Selection | 16: 20MHz | Log2N5. M=5 for 20MHz only |
|  |  |  |  | Rank Indication | 1-3 | MaxMt decides the maximum rank |
|  |  |  | Short term | Ave_CQI + Diff CQI | 4+5*2 = 14 |  |

# 15.3.9.3.1.4.3 Feedback format for MFM 3 and 6

The detailed format is listed in Tables 728. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] long term reporting period as defined in feedback allocation A-MAP IE. Multiple, consecutive, short term reports may be punctured by one long term report to send the report payload when CM=0b10.

Table 728. Feedback formats for MIMO feedback mode 3 and 6 in base mode and adaptive mode, i.e. CM=0b00 and 0b01.

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short term | CQI | 4 |  |
|  |  |  |  | PMI | 3-6 | Nt=2: 3bit Nt=4 and subset disabled: 6 bit Nt=4 and subset enabled: 4 bit |
|  |  |  | Long term | Subband selection | 5: 20MHz 4: 10MHz 3: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the maximum rank |
| 1 (M =2) | SFBCH | 2 | Short term | Diff CQI | 4 |  |
|  |  |  |  | PMI | 6-12 |  |
|  |  |  | Long term | Subband selection | 9: 20MHz 7: 10MHz 4: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the maximum rank |
|  |  |  |  | Average CQI | 4 |  |
| 2 (M =3) | SFBCH | 2 | Short term | Diff CQI | 3*2 = 6 |  |
|  |  |  |  | PMI | 9-18 |  |
|  |  |  | Long term | Subband selection | 10: 20MHz 8: 10MHz 5: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the rank |
|  |  |  |  | Average CQI | 4 |  |
| 3 (M =5) (TBD) | SFBCH | 3 | Short term | Diff CQI | 2*5 = 10 |  |
|  |  |  |  | PMI | 15-30 |  |
|  |  |  | Long term | Subband selection | 16: 20MHz | Log2N5. M=5 for 20MHz only |
|  |  |  |  | Rank | 1-3 | MaxMt decides the rank |
|  |  |  |  | Average CQI | 4 |  |

Table xx. Feedback formats for MIMO feedback mode 3 and 6 in differential mode, i.e. CM=0b10.

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 2 | Short term | CQI | 4 |  |
|  |  |  |  | Differential PMI | 2-4 | Nt=2: 2 bit Nt=4: 4 bit Nt=8: 4 bit |
|  |  |  | Long term | Subband selection | 5: 20MHz 4: 10MHz 3: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the maximum rank |
|  |  |  |  | Base PMI | 3-6 | PMI for reset. |
| 1 (M=2) | SFBCH | 2 | Short term | Diff CQI | 2*2 = 4 |  |
|  |  |  |  | Differential PMI | 2×2-2×4=4-8 | Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband |
|  |  |  | Long term | Subband selection | 9: 20MHz 7: 10MHz 4: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the rank |
|  |  |  |  | Average CQI | 4 |  |
|  |  |  |  | Base PMI | 2×3-2×6=6-12 |  |
| 2 (M =3) | SFBCH | 2 | Short term | Diff CQI | 3*2 = 6 |  |
|  |  |  |  | Differential PMI | 3×2-3×4=6-12 | Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband |
|  |  |  | Long term | Subband selection | 10: 20MHz 8: 10MHz 5: 5MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the rank |
|  |  |  |  | Average CQI | 4 |  |
|  |  |  |  | Base PMI | 3×3-3×6=9-18 |  |
| 3 (M =5) (TBD) | SFBCH | 3 | Short term | Diff CQI | 5×3=15 |  |
|  |  |  |  | Differential PMI | 5×2-5×4=10-20 | Nt=2: 2 bits per subband Nt=4: 4 bits per subband Nt=8: 4 bits per subband |
|  |  |  | Long term | Subband selection | 16: 20MHz |  |
|  |  |  |  | Rank | 1-3 | MaxMt decides the rank |
|  |  |  |  | Average CQI | 4 |  |
|  |  |  |  | Base PMI | 5×3-5×6=15-30 | Nt=2: 3 bits per subband Nt=4: 4 or 6 bits per subband Nt=8: 4 bits per subband |

# 15.3.9.3.1.4.4 Feedback format for MFM 5

The detailed format is listed inTable 729. Short term report happens in every reporting period as defined in feedback allocation A-MAP IE, long term report will puncture short term report in every [4n-th TBD] long term reporting period as defined in feedback allocation A-MAP IE.

.

Table 729 Feedback formats for MIMO feedback mode 5

| Feedback Format | FBCH | Number of Reports | Feedback Fields |  | Size in bits | Description/Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 0 (M =1) | SFBCH | 1 | Short term | Sub-band selection | 5: 20MHz 4: 10MHz 3: 5MHz |  |
|  |  |  |  | CQI | 4 bit |  |
|  |  |  |  | Stream Index | 1-2 |  |
| 1 (M=2) | SFBCH | 2 | Long term | Subband selection | 9: 20MHz 7: 10MHz 4: 5MHz |  |
|  |  |  | Short term | Ave_CQI | 4 |  |
|  |  |  |  | Diff CQI | 2*2=4 |  |
|  |  |  |  | Stream Index | 2*(1-2) |  |
| 2 (M =3) | SFBCH | 2 | Long term | Sub-band Selection | 10: 20MHz 8: 10MHz 5: 5MHz |  |
|  |  |  | Short term | Ave_CQI | 4 |  |
|  |  |  |  | Diff CQI | 3*2=6 |  |
|  |  |  |  | Stream Index | 3*(1-2) |  |
| 3 (M =5) (TBD) | SFBCH | 3 | Long term | Sub-band Selection | 16: 20MHz |  |
|  |  |  | Short term | Ave_CQI | 4 |  |
|  |  |  |  | Diff CQI | 6*2=12 |  |
|  |  |  |  | Stream Index | 6*(1-2) |  |