| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Update for the Draft P802.16m Amendment on the OL region |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Huaning Niu, Jong-Kae (JK) Fwu, Guangjie Li, Ayelet Doron, Hujun Yin, Yang-seok Choi Intel Corporation | E-mail: huaning.niu@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.9 (UL-CTRL) “Comments on AWD 15.3.9 UL-CTRL” 802.16m amendment working document |  |
| Abstract | Proposed text for the PHY structure for UL control in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

IEEE 802.16m Amendment Text Proposal for OL Region

Huaning Niu, Jong-Kae (JK) Fwu, Guangjie Li, Ayelet Doron, Hujun Yin, Yang-seok Choi

Intel Corporation

Introduction

This contribution provides updated amendment text for OL region signaling in the Advanced Air Interface to be included in Section 15 of the IEEE 802.16m Amendment Working Document [1]. The proposed update is developed so that it can be readily combined with IEEE 802.16m-09/0010r2 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

Concept of OL region

The OL region is the predefined zone in reuse-1 frequency partition dedicated for rank-1 and/or rank-2 OL MIMO transmission. OL region resources allocation is allocated using logical LRUs for OL MIMO. The remaining resources may be used for the normal region (or called non-OL region) where any MIMO mode can be supported. The benefit to define the OL region is to maintain the static interference for OL MIMO, improve accuracy of the CQI estimation and covariance matrix of interference.

Two kinds of OL regions are defined: rank-1 OL region and rank-2 OL region.

Rank-1 OL region supports rank-1 precoder, using mini-band CRU and sub-band CRU using 1-stream pilot. Rank-2 OL region supports OL SU SFBC/SM and OL MU MIMO using 2-stream pilot. DRU, mini-band CRU and sub-band CRU are supported in rank-2 OL region.

In OL region, the CQI can be estimated using precoded pilots only without using midamble. For OL SU MIMO mode, the STBC and SM can be flexibly adapted. For OL MU MIMO, full rank transmission is always used (no adaptation to SU MIMO rank-1 precoder). OL region configuration is transmitted in SFH SP3 which includes the following fields: (1) subband size (4-bit) for rank-2 OL region (in reuse-1 frequency partition, i.e., FP0); (2) miniband size (4-bit) for rank-2 OL region (in reuse-1 frequency partition, i.e., FP0); (3) combination of sub-band and mini-band in rank-1 OL (3-bit). All DRU are in rank-1 OL region.

References

[1] IEEE 802.16m-09/0010r2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” March. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r8, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

Proposed Amendment Text

The following text is proposed for inclusion in Section 15 of the IEEE 802.16m Amendment Working Document [1].

Table 692—DL MIMO control parameters

| Parameters | Description | Value | Control Channel (IE) | Notes |
| --- | --- | --- | --- | --- |
| - Broadcast Information |  |  |  |  |
| Nt | Number of transmit antennas at the BS | 0b00: 2 0b01: 4 0b10: 8 | SFH (system information) | Nt must be known before decoding the DL A-MAP IE |
| OL_Region[TBD] | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | TBD SB-OL-Rank2-Size (4 bit) : signal the number of sub-bands in rank-2 OL region. MB-OL-Rank2-Size (4 bit) : to signal the size of mini-band in rank-2 OL region. OL-Rank1-Config (3 bit): to signal the combination of sub-band and mini-band in rank-1 OL region. Refer to table TBD. | Broadcast information | Two kinds of OL region are defined: rank-1 OL region and rank-2 OL region. Rank-1 OL region supports rank-1 precoder, using mini-band CRU and sub-band CRU. Rank-2 OL region supports OL SU STBC/SM and OL MU MIMO. DRU, mini-band CRU and sub-band CRU are supported in rank-2 OL region. Rank-1 and rank-2 OL region formed using logical LRUs. OL region signaling is SFH-SP3 |
| SU_CT | SU base codebook type |  | Broadcast information | SU base codebook subset indication |
| MU_CT (TBD) | MU base codebook type |  | Broadcast information | MU base codebook subset indication |
| BC_SI | Rank-1 base codebook subset indication | BitMAP (Same size as rate-1 codebook for each number of transmit antenna) | Broadcast information | Rate-1 codebook element restriction/recommendation information It shall be ignored if CCE = 0b0 |
| MaxMt (TBD | Maximum number of streams | 0b00: 2 0b01: 3 0b10: 4 0b11: reserved | Broadcast information | If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| Nt |  |  |  |  |
| OL_Region[TBD] | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | TBD | Broadcast information |  |
| SU_CT | SU base codebook type |  | Broadcast information | SU base codebook subset indication |
| MU_CT (TBD) | MU base codebook type |  | Broadcast information | MU base codebook subset indication |
| BC_SI | Rank-1 base codebook subset indication | BitMAP (Same size as rate-1 codebook for each number of transmit antenna) | Broadcast information | Rate-1 codebook element restriction/recommendation information It shall be ignored if CCE = 0b0 |
| MaxMt (TBD | Maximum number of streams | 0b00: 2 0b01: 3 0b10: 4 0b11: reserved | Broadcast information | If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |

[  ------------------- Proposed Text Ends --------------------]