| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Improved Rank-1 8-Antenna Codebook |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Ron Porat, Phil Orlik Mitsubishi Electric Research Labs Toshiyuki Kuze Mitsubishi Electric Corporation 5-1-1 Ofuna Kamakura, Kanagawa 2478501, JAPAN | Voice: E-mail: poratron@yahoo.com; E-mail : kuze.Toshiyuki@ah.MitsubishiElectric.co.jp *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7.2.6.6.2.3.1 “Comments on AWD 15.3.7.2.6.6.2.3.1” |  |
| Abstract | This document describes a proposal for AWD Section 15.3.7.2.6.6.2.3.1 |  |
| Purpose | To be discussed and adopted by 802.16m |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Improved Rank-1 8-Antenna Codebooks

Ron Porat

Mitsubishi Electric Research Labs

# Background

The current 802.16m codebooks include 3-bit 2-antenna, 6bit 4-antenna and 4bit 8-antenna codebooks.

Clearly the number of bits required for a codebook grows with the number of antennas as long as they are uncorrelated. It is therefore expected that for truly uncorrelated 8-antenna arrays, 12 bits will be required. In practice some correlation is expected and the number of bits can be reduced.

However, the current 8-antenna design is only satisfactory with highly correlated antennas and draws on the classical array signal processing AoD concept.

In this contribution we propose to augment the current 8-antenna codebook with two new designs that are more suitable for other array configurations.

The proposed designs utilize the current 2 and 4 antenna 16m codebooks by using them as building blocks.

# 2. Proposed Codebooks

We focus on three practical antenna configurations:

- Eight lambda/2 spaced antennas [ | | | | | | | | ] – We used a 6bit steering vector with the same structure of the 4bit steering vector in 802.16m.

- Four lambda/2 spaced cross-pols [XXXX] – We used a 4-antenna 4bit steering vector and a 2-antenna 3bit codebook.  Each co-polarized group of antennas uses one steering vector and a weight from the 2-antenna codebook in order to account for the gain and phase offset between the groups.

The rank-1 PMI has therefore the following structure  where it was assumed that the co-polarized antennas occupy the first and second halves of the singular vector.

- Two clusters of 4-lambda spaced two lambda/2 spaced cross-pols [XX  XX]  - We used the same 2-antenna steering vector for each of the semi-independent 4 groups of closely spaced co-polarized antennas. Each group uses a weight taken from the 6bit 4-antenna codebook.  The rank-1 PMI has the following structure

Three options were simulated:

- 3bit 2-antenna steering vector

- 2bit 2-antenna steering vector

- 3bit 16m 2-antenna codebook

# 3. Simulation Results

We test the proposed rank-1 codebook using MUZF simulations with the MS using an MMSE receiver. The interference is assumed to be known perfectly.

The simulation assumptions are:

- DL channel – SCM Urban Macro (15 degrees angular spread).

- DL Band Size – 4 PRB (assuming one precoder per band)

- DL speed and feedback delay – 3kmph, 5mS

- DL midamble estimation – perfect

- UL feedback channel – error free

- User selection: 4 random users with exhaustive search

The table contains the achieved throughput gain (in percentage) relative to the current 4bit codebook.

| 8-Antenna Configuration | Codebook Option | DL SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 |
| Two clusters of 4-lambda spaced two lambda/2 spaced cross-pols [XX XX] | 2bit Steering Vector | 19 | 18 | 14 | 10 |
|  | 3bit Steering Vector | 22 | 22 | 19 | 14 |
|  | 3bit 16m CB | 21 | 21 | 18 | 12 |
| Four lambda/2 spaced cross-pols [XXXX] | 16m 2-Ant | 10 | 11 | 10 | 9 |
|  | 2bit Subset | 8.5 | 9 | 9 | 7.5 |
| Eight lambda/2 spaced vertical-pols [¦¦¦¦¦¦¦¦] | 6bit Steering Vector | 2.7 | 2.5 | 4.3 | 4.5 |

# Proposed text

In section 15.3.7.2.6.6.2.3.1:

Replace formulas 213 and 214 with:

The symbol  denotes a Kronecker product.

Add section 15.3.7.2.6.6.2.5.2.2 CL SU-MIMO subset for eight transmit antennas

Codebook subset is limited to rank-1:

Add section 15.3.7.2.6.6.2.5.3.3. CL MU-MIMO subset for eight transmit antennas

The base codebook subset for MU-MIMO is the same as the rank 1 of the base codebook subset for SU-MIMO