| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Principal Singular Vector Calculation for 4xN Channels |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Ron Porat, Phil Orlik Mitsubishi Electric Research Labs Toshiyuki Kuze Mitsubishi Electric Corporation 5-1-1 Ofuna Kamakura, Kanagawa 2478501, JAPAN | Voice: E-mail: poratron@yahoo.com; porlik@merl.com E-mail : kuze.Toshiyuki@ah.MitsubishiElectric.co.jp *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | . |  |
| Abstract | This document describes a proposal for 802.16m channel feedback calculation |  |
| Purpose | To be discussed and adopted by 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Rank-1 Calculation for 4xN channels

Ron Porat

Mitsubishi Electric Research Labs

# Introduction

This contribution extends the results of a previous contribution [1] whereby the SVD of a general 2xN channel was shown to have a simple closed form solution.

We provide an extension to 4xN channels. The closed form solution is not mathematically optimal but provides practically close to optimal performance.

The idea can be used in the same way for larger size channels.

A typical usage scenario is for an MS with 4 antennas estimating a joint rank-1 precoder for multi-BS scenario.

# Rank-1 Calculation

The main idea is to concentrate on HHwhich is of size Nx4 and find the optimal gain and phase combining weights of the two right singular vectors of  and .  The optimal solution for the right singular vector of an Nx2 channel was given in [1].

Denote by  and  the two right singular vectors of the two Nx2 channels.

We would like to solve

The solution for  is straightforward and given by  where

The solution for can now be found by solving   where

and .

Differentiating  and equating to zero we can arrive at a quadratic equation in, the solution of which (skipping a few stages) is

Finally the rank-1 solution to H is given by   where .

# Simulation Results

The following plot shows the rank-1 SU-MIMO effective channel-norm loss using this formula as compared to the optimal SVD based rank-1 precoding. Uncorrelated flat fading 4x4, 4x8 and 4x12 channels were simulated.

It can be seen that the loss assuming perfect channel knowledge is minimal.

We have also simulated this formula for MU-MIMO using regularized ZF for 4x8 channel in SCM Suburban Macro.

The BS was assumed to have 4 closely spaced cross-pols XXXX or two clusters [XX  XX] and the MS 2 closely spaced cross-pols ++ (V-H).

Other simulation assumptions were 3kmph, 5mS feedback delay and 4PRB band size and perfect channel estimation and feedback.

The spectral efficiency loss as compared to perfect SVD based was found to be 0.5% in all SNR ranges

# Conclusion

The closed form formula provides practically close to perfect solution for rank-1 calculation for any 4xN channel.

# References

- C802.16m-08/522r1 – “Analog Feedback Complexity Clarification”