| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Mitigation with Beam Switching for Femto BSs |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Toshiyuki Kuze, Shigeru Uchida, Kentaro Sawa, Hiroki Iura Mitsubishi Electric Corp Philip Orlik, Zhifeng (Jeff) Tao, Ramesh Annavajjala, Jinyun Zhang Mitsubishi Electric Research Lab | Voice: +81-467-41-2885 Fax: +81-467-41-2486 Kuze.Toshiyuki@ah.MitsubishiElectric.co.jp Voice: 617-621-7557 Fax: 617-621-7550 tao@merl.com |
| Re: | “802.16m AWD”: IEEE 802.16m-09/0020, “Call for Contributions on Project 802.16m Amendment Working Document (AWD) Content”. Target topic: “Support for femto BS” |  |
| Abstract | This contribution proposes the text for Interference Mitigation with Beam Switching |  |
| Purpose | To discuss and adopt the proposed text in the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Interference Mitigation with Beam switching

Toshiyuki Kuze,

Mitsubishi Electric Corp

Philip Orlik, Zhifeng (Jeff) Tao, Ramesh Annavajjala, Jinyun Zhang

Mitsubishi Electric Research Lab

# Introduction

This contribution proposes the text for inter-cell and/or Macro-cell/femto-cell interference reduction to be included in the 802.16m amendment [1]. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D9a [2], it is compliant with the 802.16m SDD [3], and it follows the style and format guidelines in [4].

# Background

The basic principle of ABF is to form at the BS a number of beams within each sector. The beams can be created in a number of different ways; one of the simplest is the concatenation of a linear antenna array concatenated with a Butler matrix, see Fig. 1

Fig. 1 Beamforming with Butler matrix

The BS then switches through the available beams (both in uplink and downlink) in an arbitrary sequence, and communicates at each time with the MSs located in the respective beams. Due to the beamforming gain, the range of the cell is extended – which is important especially for deployment in rural areas, and during the initial deployment of a network. Furthermore, by adopting appropriate beam switching patterns, the interference situation can also be improved.

Figure 2 Transmission phases of beam switching

We emphasize that ABF is not a theoretically-optimum way of using multiple antenna elements; heterodyning all the signals to/from baseband and digitally processing the signals would allow for higher capacity. However, it does present an excellent tradeoff between performance and complexity – it requires only a single RF chain. Thus due to the simplicity of the beam generation it is reasonable to assume that Femto-cells can easily be deployed with such capability. And the cost of 4 to 8 beams is reasonable even for low cost products such as femtos. This allows the reduction of interference from the femto-cell to the corresponding macro-cell in that the signal energy from the femto-cell is targeted to the femto-subscribers

Another important point is that ABF can be combined with spatial multiplexing and other MIMO techniques. The set of N available antenna elements can be divided into K groups of M antennas each (with M*K=N), so that K beams can be formed, and in each beam M antenna elements are available for spatial multiplexing etc. Especially when dual-polarized antennas are used, it is easily possible to use K=4, N=2.

Interference reduction through beam switching

ABF can also be used to improve the interference situation. Remember that MSs in different beams are served at different times. Therefore, if two neighboring cells arrange the downlink transmission in such a way that they do not transmit to the same part of the cell-edge at the same time (and similarly for the uplink), then the interference seen by the MS is greatly reduced. Fig. 2 shows an example for such a situation; the numbers in the beams indicate the BF zone during which they are served.

Fig. 2  Interference reduction through beam switching

If the ABSs and/or the ABS and femto-cell can coordinate the beams, then interference greatly reduced.  If BSs cannot coordinate, then the sequence in which beams are served can be chosen randomly, and independently at each BS. This still leads to a stochastic reduction of the interference, similar to the reduction of interference in random frequency hopping or time-hopping impulse radio.

In the case of random beam selection, the hopping sequence can be derived at each BS based upon the base station identification (BS ID).  For example, the BS ID is used as the initial value (i.e., seed) to the feedback shift register that will generate the hopping sequence.

# Proposed Text

-----------------------------------------  Start of the Text---------------------------------------------------

[insert the following subclauses into section 20]

20.x Interference Mitigation Techniques

IEEE 802.16m femto-cells shall be capable of transmitting signals in fixed beams.

20.x.x Beam selection

During initialization of a femto-cell preamble signals are sent on each beam in consecutive fashion.  The AMS listens to all ABF preambles and reports the index of the best beam.

Alternatively, the femto-cell may request ranging or UL sounding signal as in 15.3.9.1.4.1 from the AMS while alternating its beam for the reception.  After several soundings the femto-cell has a sampling of channel qualities for each beam and can schedule the MS’s transmissions for in the best beam.

[insert the following into 15.3.6.5.2.11 UL Sounding Command A-MAP IE, page 229 line 43]

… ‘The three periodicy bits are used to indicate the MS to periodically repeat the sounding transmission. For ABS or femtocells that use fixed beams for DL transmissions the periodic sounding shall continue for at least log2(N) periods, so that all N beams may be sounded. Setting periodicy bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed. For multi-antenna flag equals to 0 the MS sounds with antenna switching, while for multi-antenna flag equals to 1 the MS sounds all transmit antennas.

[insert the following subclause 15.3.2 which describes the signaling of beam numbers in fixed beam femto-cells]

15.3.10.2.7

Each beam shall be assigned a unique MAC level identifier which is the concatenation the femto-cell’s BS-ID along with an integer Nb, were Nb denotes the beam number and may be 4 or 8.

-----------------------------------------  End of the Text---------------------------------------------------

- References

[1] IEEE 802.16m-08/010r1a, “IEEE 802.16m Amendment Working Document,” Mar. 2009

[2] IEEE P802.16 Rev2 / D9a, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,” Mar. 2009

[3] IEEE 802.16m-08/003r8, “The Draft IEEE 802.16m System Description Document,” Mar. 2009

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment,” Sep. 2008