| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling support for reporting AMS battery level for Power Management in Connected Mode |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | N. Himayat, M. Venkatchalam, S. Mohanty, Intel Corporation S. Andreev, P. Gonchukov, A. Turlikov, SUAI Ming-Hung Tao Ying-Chuan Hsiao ITRI Yih-Shen Chen MediaTek Inc. | Voice: 408-765-5043 E-mail: nageen.himayat@intel.com Email: mhtao@itri.org.tw Email: yihshen.chen@mediatek.com |
| Re: | Battery level reporting for Power Management in Connected Mode (section 15.2.20) |  |
| Abstract | Section 15.2.20 in the AWD allows for the AMS to report its battery level so that the ABS can initiate power savings at the AMS. This contribution proposes specific signaling for the client to report its battery level. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling support for reporting AMS battery level for Power Management in Connected Mode

Intel Corporation

# Introduction

Section 15.2.20 of the AWD on “Power management in connected mode” includes support for the AMS to report its battery level to the ABS. This contribution defines the information to be included as part of the battery level report, as well as the signaling mechanism enabling the reporting of this information.

# Enabling Battery Level Reporting by the AMS

Either the AMS or the ABS can initiate the AMS battery level report. The 1byte information content for this report is defined below.

Table 1: Information Content of Battery Level Report

| Battery Level | 3 | 111: AMS is plugged into a power source 110: Battery level is between 80% and 100% 101: Battery level is between 65% and 80% 100: Battery level is between 50% and 65% 011: Battery level is between 35% and 50% 010: Battery level is between 20% and 35% 001: Battery level is between 10% and 20% 000: Battery level is between 0% and 10% |
| --- | --- | --- |
| Confirmation | 1 | 0: Request 1: Confirmation |
| Reserved | 4 | Shall be set to zero. |

In addition, we propose to use an optional extended MAC header for communicating the battery level report (section 15.2.3.2, [2]). For this purpose a new extended header type, “Connected Mode Power Management Extended Header” is defined.

The specific text changes are indicated in the next section.

# Text Proposal

Insert the following text in section 15.2.20 of the Amendment working document

----------------------------------------------Start of the text proposal (section 15.2.20) ------------------------------------

### 15.2.20.1 Reporting AMS Battery Level

Either the AMS or the ABS may initiate the battery level report. The information fields for the Battery Level Report are defined below.

Table x.x.1 Battery Level Report

| Battery Level | 3 | 111: AMS is plugged into a power source 110: Battery level is between 80% and 100% 101: Battery level is between 65% and 80% 100: Battery level is between 50% and 65% 011: Battery level is between 35% and 50% 010: Battery level is between 20% and 35% 001: Battery level is between 10% and 20% 000: Battery level is between 0% and 10% |
| --- | --- | --- |
| Confirmation | 1 | 0: Request 1: Confirmation |
| Reserved | 4 | Shall be set to zero. |

The AMS may autonomously send a battery level report, indicating its current battery level to the ABS, with Confirmation field set to “0= Request”.

The ABS may also trigger a battery level report from the AMS by sending a battery level report with Confirmation field set to “0=Request”.  It will set all remaining fields to zero. The AMS will then report its current battery level to the ABS with Confirmation field set to “1=Confirm”.

The “Connected Mode Power Management Extended Header” defined in Section 15.2.3.2 should be used for sending the Battery Level Report.

----------------------------------------------End of the text proposal (section 15.2.20) ------------------------------------

Modify the text in section 15.2.3.2 of the Amendment working document

-------------------------------------------Start of the text modification (section 15.2.3.2) ---------------------------------

Table 6- Description of Extended Header Types

| Extended Header Types | Names | Description |
| --- | --- | --- |
| - | Fragmentation and packing extended header | See 15.2.3.2.1 |
| TBD | Fragmentation extended header | See 15.2.3.2.2 |
| TBD | Multiplexing extended header | See 15.2.3.2.3 |
| TBD | Connected mode power management extended header | See 15.2.3.2.x |
| TBD | Reserved |  |

-------------------------------------------End the text modification (section 15.2.3.2) ---------------------------------

Insert the following text in section 15.2.3.2 of the Amendment working document

-------------------------------------------Start of the text proposal (section 15.2.3.2.x) ---------------------------------

### 15.2.3.2.x Connected mode power management extended header  (CPMEH)

Table x: Connected mode power management extended header

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CPMEH () { |  |  |
| Type | TBD | CPMEH type |
| Battery Level Report | 8 | See 15.2.20.1 |
| Reserved | TBD |  |
| } |  |  |

----------------------------------------------End of the text proposal (section 15.2.3.2.x) ----------------------------

# References

- IEEE STD 802.16, Rev2, D9, Jan. 2009.

- IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document,” (AWD).