| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 16m HO Neighbor Advertisement message format (AWD-15.2.5) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Xiangying Yang Muthaiah Venkatachalam Etemad Kamran Intel | E-mail: xiangying.yang@intel.com |
| Re: | IEEE 802.16m-09/0028r1. ”AWD call for contribution after session #61” Category: AWD/Area: Chapter 15.2.5 (AWD-15.2.5) |  |
| Abstract | This contribution proposes text for HO network re-entry procedure in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

16m HO Neighbor Advertisement message format (AWD-15.2.5)

Xiangying Yang, Muthaiah Venkatachalam, Etemad Kamran

Intel

- Text proposal for 16m AAI_NBR-ADV

There are a few reasons that 16e MOB_NBR-ADV message format needs improvement in 16m.

- 16m use SFH to carry system information instead of TLV based DCD/UCD.

- 16m is to support deployment of multiple cell types and HO between them. 16e MOB_NBR-ADV relies on delta TLV encoding for overhead reduction, which may not be efficient when cell types are different.

In this contribution, we propose new AAI_NBR-ADV message format that is flexible enough to meet 16m deployment requirement as well as achieve good overhead reduction. A conceptual structure is illustrated in the Figure 1.

The AAI_NBR-ADV message carries system information of ABS of a particular cell type. Each AAI_NBR-ADV message contains the following information

- Segment control information (total number of cell types, cell type of this message, segment information if this message is partitioned into multiple segments)

- parameters shared by this particular cell type

- system information of each BS, with two possible encoding formats, where

  - encoding format 1 uses SFH format (except those parameters already defined in cell type parameters), with a control_bitmap indicating whether a particular subpacket is included or not (not included due to overhead or less importance). Except that the serving BS is of this cell type too, the 1st BS in the list always uses this encoding format. The control_bitmap allows serving BS to broadcast minimal system information for certain neighbor BS (e.g., 16e BS or overlay pico-cells) so that MS can obtain the information via listening to LZone or scanning.

  - encoding format 2 uses 16e-like delta TLV format. The reference BS for delta encoding is always the serving BS if they are of the same type, and if not the reference BS is the 1st BS in this cell type. In case of

Figure 1 AAI_NBR-ADV structure with two different encoding formats

Proposed text #1:

Insert the following text into section 15.2.5

-------------------------------------------text #1 start----------------------------------------------

15.2.5.x AAI_NBR-ADV

Table XXX—AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| Number of cell type | 3 |  |
| Cell type in this message | 3 |  |
| Total Number of Segments | TBD | Number of segments for this cell type |
| Segment Index | TBD | Segment index of this message in the specific cell type |
| BS number M | TBD | Total number of BSs to be included in this segment |
| FA information | TBD | //cell type info { |
| BW information | 2 | 0b00: 5M, 0b01: 10M, 0b10: 20M, 0b11: Reserved |
| MAC protocol version | TBD |  |
| Duplex Mode | 2 | TDD, FDD, HFDD}//cell type info |
| for (i=0; i<M-1; i++) |  |  |
| { |  |  |
| Index of BS | 8 | The index of the BS in the whole AAI_NBR-ADV |
| BSID | 48 |  |
| A-PREAMBLE info | TBD |  |
| SFH_encoding_format | 2 | 00: full Subpkt information 01: delta encoding with TBD format (the 1st BS in this cell type shall use full Subpkt encoding ) |
| Control_bitmap | 4 | Each bit maps to one SFH subpacket. The last bit is Multicarrier capability indication (MCI) 0: not included. 1: included. |
| If( SFH_encoding_format =00 |  | //encoding format type-1 |
| { |  |  |
| If( Control_bitmap[0] ==1 ) |  |  |
| { |  |  |
| Subpkt 1 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[1] ==1 ) |  |  |
| { |  |  |
| Subpkt2 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[2] ==1 ) |  |  |
| { |  |  |
| Subpkt3 | TBD | //exclude those fields already in cell type info |
| } |  |  |
| If( Control_bitmap[3] ==1 ) |  |  |
| { |  |  |
| Number of carriers (NC) | 3 | Multi-carrier info for fully configured carriers |
| For(i=0; i<NC; i++) |  |  |
| { |  |  |
| Carrier index | TBD | Carrier information as indicated in AAI_MC-ADV |
| } |  |  |
| } |  |  |
| } |  |  |
| If( SFH_encoding_format =00 |  |  |
| { |  |  |
| Delta TLV information |  | Delta encoding, w.r.t. the reference BS |
| } |  |  |
| } | — | — |
| } | — | — |

-------------------------------------------text #1 end----------------------------------------------