| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | 16m HO Neighbor Advertisement message format (AWD-15.2.5) |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Xiangying Yang Muthaiah Venkatachalam Etemad Kamran Intel Yih-Shen Chen, Kelvin Chou, I-Kang Fu MediaTek Inc. Jaehyuk Jang Samsung | E-mail: xiangying.yang@intel.com yihshen.chen@mediatek.com, kelvin.chou@mediatek.com jack.jang@samsung.com |
| Re: | IEEE 802.16m-09/0028r1. ”AWD call for contribution after session #61” Category: AWD/Area: Chapter 15.2.5 (AWD-15.2.5) |  |
| Abstract | This contribution proposes text for HO network re-entry procedure in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

16m HO Neighbor Advertisement message format (AWD-15.2.5)

Xiangying Yang, Muthaiah Venkatachalam, Etemad Kamran

Intel

Yih-Shen Chen, Kelvin Chou, I-Kang Fu

MediaTek Inc.

Jaehyuk Jang

Samsung

- Text proposal for 16m AAI_NBR-ADV

There are a few reasons that 16e MOB_NBR-ADV message format needs improvement in 16m.

- 16m use SFH to carry system information instead of TLV based DCD/UCD.

- 16m is to support deployment of multiple cell types and HO between them. 16e MOB_NBR-ADV relies on delta TLV encoding for overhead reduction, which may not be efficient when cell types are different.

In this contribution, we propose new AAI_NBR-ADV message format that is flexible enough to meet 16m deployment requirement as well as achieve good overhead reduction. A conceptual structure is illustrated in the Figure 1.

The AAI_NBR-ADV message carries system information of ABS of a particular cell type. Each AAI_NBR-ADV message contains the following information

- Segment control information (total number of cell types, cell type of this message, segment information if this message is partitioned into multiple segments)

- parameters shared by this particular cell type

- Network entry-related information in AAI_NBR-ADV: SFH IEs can be flexibly carried in AAI_NBR-ADV so as to shorten scanning time. If no SFH or not full SFH information is carried, MS can obtain it during scanning.

- system information of each BS, with two possible encoding formats, where

  - encoding format 1 uses SFH format (except those parameters already defined in cell type parameters), with a control_bitmap indicating whether a particular subpacket is included or not (not included due to overhead or less importance). Except that the serving BS is of this cell type too, the 1st BS in the list always uses this encoding format. The control_bitmap allows serving BS to broadcast minimal system information for certain neighbor BS (e.g., 16e BS or overlay pico-cells) so that MS can obtain the information via listening to LZone or scanning.

  - encoding format 2 uses 16e-like delta TLV format. The reference BS for delta encoding is always the serving BS if they are of the same type, and if not the reference BS is the 1st BS in this cell type. In case of

Figure 1 AAI_NBR-ADV structure with two different encoding formats

Proposed text #1:

Insert the following text into section 15.2.5

-------------------------------------------text #1 start----------------------------------------------

15.2.5.x AAI_NBR-ADV

…….

Each AAI_NBR-ADV message carries

- AAI_NBR-ADV change count

- number of total cell types

- segment information for this AAI_NBR-ADV message

- system information of ABSs from one or more cell types, which is of variable size. For each cell type, shared configuration parameters are carried as follows

- FA information

- BW information

- Duplex mode

- MAC protocol version

Within each cell type, a list of ABSs are provided and the following parameters are carried for each ABS

- Index of neighbor ABS in the whole neighbor list maintained by serving ABS

- 48bit BS-ID

- ABS A-Preamble index

- indication whether full system information or partial information is carried for this ABS, which includes

- SFH information

- ESI information

- Index to MC configuration,

where for macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type).

-------------------------------------------text #1 end----------------------------------------------