| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A-A-MAP Group Size Indication in Non-User Specific A-MAP IE (AWD-15.3.6) |  |
| Date Submitted | 2009-07-09 |  |
| Source(s) | Ping Wang Yi Hsuan Hujun Yin Intel Corporation | ping.wang@intel.com yi.hsuan@intel.com hujun.yin@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes the A-MAP group size indication in non-user specific A-MAP IE. |  |
| Purpose | To be discussed by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A-A-MAP Group Size Indication in Non-User Specific A-MAP

Ping Wang, Yi Hsuan, and Hunjun Yin

Intel Corporation

- 1. Introductions

QPSK {1/2, 1/4} or QPSK {1/2, 1/8} are accepted as A-A-MAP MCS sets in the May, 2009 IEEE 802.16m meeting. A-A-MAPs are grouped together based on MCS and A-A-MAP IE size. The size of each A-A-MAP group needs to be indicated in non-user specific A-MAP IE to facilitate AMS A-A-MAP blind detection.

This contribution proposes how to generate the A-A-MAP group size indication table in non-user specific A-MAP IE in 15.3.6.5.2.1 in the 802.16m AWD [1]. The following design considerations are taken into account in the proposed scheme.

- One A-A-MAP IE size is supported;

- Use a single table for all system bandwidths, given an MCS set and FFR configuration.

- The maximum number of used MLRUs for A-A-MAP is 48 per subframe, which is about 25% [=(48/2)/96] of subframe resource at 20MHz bandwidth.

- Four A-A-MAP group size indication tables are needed as shown in the following:

  - When QPSK (1/2, 1/4) is used in the non-FFR case, two A-A-MAP groups are needed. The minimum resource unit in each group is 1 and 2 MLRU respectively.

  - When QPSK (1/2, 1/8) is used in the non-FFR case, two A-A-MAP groups are needed. The minimum resource unit in each group is 1 and 4 MLRU respectively.

  - In case of FFR, when QPSK 1/2 is used for the FFR reuse 3 A-A-MAP and QPSK (1/2, 1/4) is used for the reuse 1 A-A-MAP, three A-A-MAP groups are needed. The minimum resource unit in each group is 1, 1, and 2 MLRUs respectively;

  - In case of FFR, when QPSK 1/2 is used for the FFR Reuse 3 A-A-MAP and QPSK (1/2, 1/8) is used for  the reuse 1 A-A-MAP, three A-A-MAP groups are needed. The minimum resource unit in each group is 1, 1, and 4 MLRUs respectively.

- We use 8-bit lookup table for A-A-MAP group size indication. However some group size combinations need to be removed, resulting the following two design considerations if the size indication is larger than the required size of a A-A-MAP group:

  - Unoccupied resource in the A-A-MAP group that is adjacent to data resource may be used for the data transmission by ABS. Therefore there is no resource waste but extra blind detections are required.

  - Unoccupied resource in the A-A-MAP group that is not adjacent to the data resource cannot be used for data transmission and is therefore wasted. The design should minimize resource waste in order to maintain system efficiency.

- 2. A-A-MAP Group Size Indication with All Group Size Combinations

Table 1 lists the number of possible group size combinations and required number of bits to signal each combination, assuming that the total number of MLRUs for A-A-MAP is 12, 24, and 48 for 5, 10, 20 MHz system bandwidth respectively.

Table 1: Signaling Overhead of Unconstrained A-A-MAP Resource Indication

| Bandwidth | 5MHz | 10MHz | 20MHz |
| --- | --- | --- | --- |
| Number of MLRUs used by A-A-MAP | 12 | 24 | 48 |
| Number of Combinations using QPSK (1/2, 1/4) for non-FFR A-A-MAP | 49 (ceil(log2(49))=6 bits) | 169 (ceil(log2(169))=8 bits) | 625 (ceil(log2(625))= 10 bits) |
| Number of Combinations using QPSK (1/2, 1/8) for non-FFR A-A-MAP | 28 (ceil(log2(28))=5 bits) | 91 (ceil(log2(91))= 7 bits) | 325 (ceil(log2(325))= 9 bits) |
| Number of Combinations using QPSK 1/2 for FFR Reuse 3 A-A-MAP and (1/2, 1/8) for Reuse 1 A-A-MAP | 152 (ceil(log2(152))=8 bits) | 861 (ceil(log2(861))= 10 bits) | 5681 (ceil(log2(5681))= 13 bits) |
| Number of Combinations using QPSK (1/2 for Reuse 3 A-A-MAP and (1/2, 1/4) for Reuse 1 A-A-MAP | 252 (ceil(log2(252))=8 bits) | 1547 (ceil(log2(1527))= 12 bits) | 10725 (ceil(log2(10725))= 14 bits) |

It can be seen that the signaling overhead of the A-A-MAP group size indication can be as large as 14 bits. It is desirable to remove some combinations and reduce the signaling overhead. .

- 3. A-A-MAP Group Size Indication with Reduced Combinations

In order to reduce the signaling overhead of the A-A-MAP group size indication in non-user specific A-MAP IE, some constraints shall be made:

- Only 8-bit signaling overhead is used for the A-A-MAP resource allocation indication in the non-user specific A-MAP IE, which means that some combinations of MLRUs for A-A-MAP have to be removed;

- Unoccupied resource not adjacent to the data transmission in the A-A-MAP region is wasted. Unoccupied resource in the A-A-MAP group that is adjacent to data resource may be used for the data transmission by ABS

## Resource allocation Indication Table of MLRUs for non-FFR A-A-MAP Using QPSK (1/2, 1/4)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(2∗N1+N2)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤24) is the number of A-A-MAPs in Group 1 using QPSK 1/4; N2 (0≤N2≤48) is the number of A-A-MAPs in Group 2 using QPSK 1/2.

Group 1 is not adjacent to data transmission and Group 2 is adjacent to data transmission. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove combinations first:

  - N1 may be [0:1:24];

  - N2 may be [0:2:48];

The total number of remaining combinations is reduced to 325, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

Δ=325(325−256)=4.71

whereΔis the removing step. From the new index 1, combinations are uniformly removed every Δ combinations.. The following are the combination indices to be removed: [1,5,10,15,20,24,29,34,39,43,48,53,58, 62,67,72,77,81,86,91,96,101,105,110,115,120,124,129,134,139,143,148,153,158,162,167,172,177,182,186,191,196,201,205,210,215,220,224,229,234,239,243,248,253,258,263,267,272,277,282,286,291,296,301,305,310, 315,320,324].

Figure 1: Average Resource Wastes and Average Extra Blind Detection

Table 2: 8-Bit Resource Allocation Indication of MLRUs for non-FFR A-A-MAP Group 1 Using QPSK 1/4 and Group 2 using QPSK 1/2 at 5, 10, and 20MHz

| Index | Group 1 Using QPSK 1/4 (Number of 2 MLRUs) | Group 2 Using QPSK 1/2 (Number of 1 MLRUs) | Total Number of MLRUs | Number of Blind-Detection on A-A-MAP |
| --- | --- | --- | --- | --- |
| 1 | 0 | 2 | 2 | 2 |
| 2 | 0 | 4 | 4 | 4 |
| 3 | 0 | 6 | 6 | 6 |
| 4 | 0 | 8 | 8 | 8 |
| 5 | 0 | 10 | 10 | 10 |
| 6 | 0 | 12 | 12 | 12 |
| 7 | 0 | 16 | 16 | 16 |
| 8 | 0 | 18 | 18 | 18 |
| 9 | 0 | 20 | 20 | 20 |
| 10 | 0 | 24 | 24 | 24 |
| 11 | 0 | 26 | 26 | 26 |
| 12 | 0 | 28 | 28 | 28 |
| 13 | 0 | 30 | 30 | 30 |
| 14 | 0 | 32 | 32 | 32 |
| 15 | 0 | 34 | 34 | 34 |
| 16 | 0 | 40 | 40 | 40 |
| 17 | 0 | 42 | 42 | 42 |
| 18 | 0 | 44 | 44 | 44 |
| 19 | 1 | 0 | 2 | 1 |
| 20 | 1 | 4 | 6 | 5 |
| 21 | 1 | 6 | 8 | 7 |
| 22 | 1 | 8 | 10 | 9 |
| 23 | 1 | 10 | 12 | 11 |
| 24 | 1 | 12 | 14 | 13 |
| 25 | 1 | 14 | 16 | 15 |
| 26 | 1 | 16 | 18 | 17 |
| 27 | 1 | 18 | 20 | 19 |
| 28 | 1 | 20 | 22 | 21 |
| 29 | 1 | 22 | 24 | 23 |
| 30 | 1 | 24 | 26 | 25 |
| 31 | 1 | 26 | 28 | 27 |
| 32 | 1 | 28 | 30 | 29 |
| 33 | 1 | 30 | 32 | 31 |
| 34 | 1 | 32 | 34 | 33 |
| 35 | 1 | 34 | 36 | 35 |
| 36 | 1 | 36 | 38 | 37 |
| 37 | 1 | 38 | 40 | 39 |
| 38 | 1 | 40 | 42 | 41 |
| 39 | 1 | 42 | 44 | 43 |
| 40 | 1 | 44 | 46 | 45 |
| 41 | 1 | 46 | 48 | 47 |
| 42 | 2 | 0 | 4 | 2 |
| 43 | 2 | 2 | 6 | 4 |
| 44 | 2 | 4 | 8 | 6 |
| 45 | 2 | 6 | 10 | 8 |
| 46 | 2 | 10 | 14 | 12 |
| 47 | 2 | 18 | 22 | 20 |
| 48 | 2 | 22 | 26 | 24 |
| 49 | 2 | 24 | 28 | 26 |
| 50 | 2 | 26 | 30 | 28 |
| 51 | 2 | 30 | 34 | 32 |
| 52 | 2 | 32 | 36 | 34 |
| 53 | 2 | 34 | 38 | 36 |
| 54 | 2 | 36 | 40 | 38 |
| 55 | 2 | 40 | 44 | 42 |
| 56 | 2 | 42 | 46 | 44 |
| 57 | 2 | 44 | 48 | 46 |
| 58 | 3 | 2 | 8 | 5 |
| 59 | 3 | 4 | 10 | 7 |
| 60 | 3 | 6 | 12 | 9 |
| 61 | 3 | 8 | 14 | 11 |
| 62 | 3 | 10 | 16 | 13 |
| 63 | 3 | 12 | 18 | 15 |
| 64 | 3 | 14 | 20 | 17 |
| 65 | 3 | 16 | 22 | 19 |
| 66 | 3 | 18 | 24 | 21 |
| 67 | 3 | 20 | 26 | 23 |
| 68 | 3 | 22 | 28 | 25 |
| 69 | 3 | 26 | 32 | 29 |
| 70 | 3 | 28 | 34 | 31 |
| 71 | 3 | 30 | 36 | 33 |
| 72 | 3 | 32 | 38 | 35 |
| 73 | 3 | 34 | 40 | 37 |
| 74 | 3 | 36 | 42 | 39 |
| 75 | 3 | 38 | 44 | 41 |
| 76 | 3 | 40 | 46 | 43 |
| 77 | 3 | 42 | 48 | 45 |
| 78 | 4 | 4 | 12 | 8 |
| 79 | 4 | 6 | 14 | 10 |
| 80 | 4 | 8 | 16 | 12 |
| 81 | 4 | 10 | 18 | 14 |
| 82 | 4 | 12 | 20 | 16 |
| 83 | 4 | 14 | 22 | 18 |
| 84 | 4 | 16 | 24 | 20 |
| 85 | 4 | 22 | 30 | 26 |
| 86 | 4 | 24 | 32 | 28 |
| 87 | 4 | 28 | 36 | 32 |
| 88 | 4 | 30 | 38 | 34 |
| 89 | 4 | 34 | 42 | 38 |
| 90 | 4 | 38 | 46 | 42 |
| 91 | 5 | 0 | 10 | 5 |
| 92 | 5 | 2 | 12 | 7 |
| 93 | 5 | 6 | 16 | 11 |
| 94 | 5 | 8 | 18 | 13 |
| 95 | 5 | 10 | 20 | 15 |
| 96 | 5 | 14 | 24 | 19 |
| 97 | 5 | 16 | 26 | 21 |
| 98 | 5 | 18 | 28 | 23 |
| 99 | 5 | 20 | 30 | 25 |
| 100 | 5 | 22 | 32 | 27 |
| 101 | 5 | 24 | 34 | 29 |
| 102 | 5 | 30 | 40 | 35 |
| 103 | 5 | 32 | 42 | 37 |
| 104 | 5 | 34 | 44 | 39 |
| 105 | 5 | 38 | 48 | 43 |
| 106 | 6 | 0 | 12 | 6 |
| 107 | 6 | 2 | 14 | 8 |
| 108 | 6 | 6 | 18 | 12 |
| 109 | 6 | 10 | 22 | 16 |
| 110 | 6 | 12 | 24 | 18 |
| 111 | 6 | 14 | 26 | 20 |
| 112 | 6 | 16 | 28 | 22 |
| 113 | 6 | 18 | 30 | 24 |
| 114 | 6 | 22 | 34 | 28 |
| 115 | 6 | 24 | 36 | 30 |
| 116 | 6 | 26 | 38 | 32 |
| 117 | 6 | 28 | 40 | 34 |
| 118 | 6 | 30 | 42 | 36 |
| 119 | 6 | 32 | 44 | 38 |
| 120 | 6 | 34 | 46 | 40 |
| 121 | 6 | 36 | 48 | 42 |
| 122 | 7 | 0 | 14 | 7 |
| 123 | 7 | 2 | 16 | 9 |
| 124 | 7 | 6 | 20 | 13 |
| 125 | 7 | 8 | 22 | 15 |
| 126 | 7 | 12 | 26 | 19 |
| 127 | 7 | 14 | 28 | 21 |
| 128 | 7 | 16 | 30 | 23 |
| 129 | 7 | 18 | 32 | 25 |
| 130 | 7 | 20 | 34 | 27 |
| 131 | 7 | 22 | 36 | 29 |
| 132 | 7 | 24 | 38 | 31 |
| 133 | 7 | 26 | 40 | 33 |
| 134 | 7 | 30 | 44 | 37 |
| 135 | 7 | 32 | 46 | 39 |
| 136 | 7 | 34 | 48 | 41 |
| 137 | 8 | 0 | 16 | 8 |
| 138 | 8 | 2 | 18 | 10 |
| 139 | 8 | 4 | 20 | 12 |
| 140 | 8 | 6 | 22 | 14 |
| 141 | 8 | 8 | 24 | 16 |
| 142 | 8 | 10 | 26 | 18 |
| 143 | 8 | 12 | 28 | 20 |
| 144 | 8 | 16 | 32 | 24 |
| 145 | 8 | 20 | 36 | 28 |
| 146 | 8 | 22 | 38 | 30 |
| 147 | 8 | 24 | 40 | 32 |
| 148 | 8 | 26 | 42 | 34 |
| 149 | 8 | 28 | 44 | 36 |
| 150 | 8 | 30 | 46 | 38 |
| 151 | 8 | 32 | 48 | 40 |
| 152 | 9 | 0 | 18 | 9 |
| 153 | 9 | 2 | 20 | 11 |
| 154 | 9 | 4 | 22 | 13 |
| 155 | 9 | 6 | 24 | 15 |
| 156 | 9 | 12 | 30 | 21 |
| 157 | 9 | 14 | 32 | 23 |
| 158 | 9 | 16 | 34 | 25 |
| 159 | 9 | 18 | 36 | 27 |
| 160 | 9 | 20 | 38 | 29 |
| 161 | 9 | 24 | 42 | 33 |
| 162 | 10 | 0 | 20 | 10 |
| 163 | 10 | 4 | 24 | 14 |
| 164 | 10 | 6 | 26 | 16 |
| 165 | 10 | 8 | 28 | 18 |
| 166 | 10 | 10 | 30 | 20 |
| 167 | 10 | 12 | 32 | 22 |
| 168 | 10 | 14 | 34 | 24 |
| 169 | 10 | 20 | 40 | 30 |
| 170 | 10 | 22 | 42 | 32 |
| 171 | 10 | 24 | 44 | 34 |
| 172 | 10 | 26 | 46 | 36 |
| 173 | 10 | 28 | 48 | 38 |
| 174 | 11 | 0 | 22 | 11 |
| 175 | 11 | 2 | 24 | 13 |
| 176 | 11 | 4 | 26 | 15 |
| 177 | 11 | 6 | 28 | 17 |
| 178 | 11 | 8 | 30 | 19 |
| 179 | 11 | 12 | 34 | 23 |
| 180 | 11 | 14 | 36 | 25 |
| 181 | 11 | 16 | 38 | 27 |
| 182 | 11 | 18 | 40 | 29 |
| 183 | 11 | 22 | 44 | 33 |
| 184 | 11 | 24 | 46 | 35 |
| 185 | 11 | 26 | 48 | 37 |
| 186 | 12 | 2 | 26 | 14 |
| 187 | 12 | 4 | 28 | 16 |
| 188 | 12 | 6 | 30 | 18 |
| 189 | 12 | 8 | 32 | 20 |
| 190 | 12 | 10 | 34 | 22 |
| 191 | 12 | 12 | 36 | 24 |
| 192 | 12 | 14 | 38 | 26 |
| 193 | 12 | 16 | 40 | 28 |
| 194 | 12 | 18 | 42 | 30 |
| 195 | 12 | 20 | 44 | 32 |
| 196 | 12 | 22 | 46 | 34 |
| 197 | 12 | 24 | 48 | 36 |
| 198 | 13 | 2 | 28 | 15 |
| 199 | 13 | 6 | 32 | 19 |
| 200 | 13 | 10 | 36 | 23 |
| 201 | 13 | 12 | 38 | 25 |
| 202 | 13 | 16 | 42 | 29 |
| 203 | 13 | 20 | 46 | 33 |
| 204 | 13 | 22 | 48 | 35 |
| 205 | 14 | 2 | 30 | 16 |
| 206 | 14 | 4 | 32 | 18 |
| 207 | 14 | 6 | 34 | 20 |
| 208 | 14 | 12 | 40 | 26 |
| 209 | 14 | 14 | 42 | 28 |
| 210 | 14 | 16 | 44 | 30 |
| 211 | 15 | 0 | 30 | 15 |
| 212 | 15 | 2 | 32 | 17 |
| 213 | 15 | 4 | 34 | 19 |
| 214 | 15 | 6 | 36 | 21 |
| 215 | 15 | 8 | 38 | 23 |
| 216 | 15 | 10 | 40 | 25 |
| 217 | 15 | 12 | 42 | 27 |
| 218 | 15 | 14 | 44 | 29 |
| 219 | 15 | 16 | 46 | 31 |
| 220 | 15 | 18 | 48 | 33 |
| 221 | 16 | 2 | 34 | 18 |
| 222 | 16 | 4 | 36 | 20 |
| 223 | 16 | 6 | 38 | 22 |
| 224 | 16 | 8 | 40 | 24 |
| 225 | 16 | 12 | 44 | 28 |
| 226 | 16 | 14 | 46 | 30 |
| 227 | 16 | 16 | 48 | 32 |
| 228 | 17 | 0 | 34 | 17 |
| 229 | 17 | 2 | 36 | 19 |
| 230 | 17 | 4 | 38 | 21 |
| 231 | 17 | 6 | 40 | 23 |
| 232 | 17 | 8 | 42 | 25 |
| 233 | 17 | 10 | 44 | 27 |
| 234 | 17 | 12 | 46 | 29 |
| 235 | 17 | 14 | 48 | 31 |
| 236 | 18 | 0 | 36 | 18 |
| 237 | 18 | 2 | 38 | 20 |
| 238 | 18 | 6 | 42 | 24 |
| 239 | 18 | 10 | 46 | 28 |
| 240 | 18 | 12 | 48 | 30 |
| 241 | 19 | 2 | 40 | 21 |
| 242 | 19 | 4 | 42 | 23 |
| 243 | 19 | 6 | 44 | 25 |
| 244 | 20 | 0 | 40 | 20 |
| 245 | 20 | 2 | 42 | 22 |
| 246 | 20 | 4 | 44 | 24 |
| 247 | 20 | 6 | 46 | 26 |
| 248 | 20 | 8 | 48 | 28 |
| 249 | 21 | 2 | 44 | 23 |
| 250 | 21 | 4 | 46 | 25 |
| 251 | 21 | 6 | 48 | 27 |
| 252 | 22 | 0 | 44 | 22 |
| 253 | 22 | 2 | 46 | 24 |
| 254 | 22 | 4 | 48 | 26 |
| 255 | 23 | 0 | 46 | 23 |
| 256 | 24 | 0 | 48 | 24 |

## Resource Allocation Indication Table of MLRUs for non-FFR A-A-MAP Using QPSK (1/2, 1/8)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal=(4∗N1+N2)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤12) is the number of 4 MLRU for A-A-MAP in Group 1 using QPSK 1/8; N2 (0≤N2≤48) is the number of 1 MLRUs for A-A-MAP in Group 2 using QPSK 1/2.

Group 1 is not adjacent to data transmission and Group 2 is adjacent to data transmission. Therefore unoccupied MLRUs in Group 1 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - N1 may be [0:1:12];

  - N2 may be [0:1:48];

The total number of remaining combinations is reduced to 325, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

Δ=325(325−256)=4.71

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δ combinations. The following are the combination indices to be removed: [1,5,10,15,20,24,29,34,39,43,48,53,58, 62,67,72,77,81,86,91,96,101,105,110,115,120,124,129,134,139,143,148,153,158,162,167,172,177,182,186,191,196,201,205,210,215,220,224,229,234,239,243,248,253,258,263,267,272,277,282,286,291,296,301,305,310, 315,320,324].

Figure 2: Average Resource Wastes and Average Extra Blind Detection

Table 3: 8-bits Resource Allocation Indication of MLRUs for non-FFR A-A-MAP Group 1 Using QPSK 1/8 and Group 2 Using QPSK 1/2 at 5, 10, 20MHz BW

| Index | Group 1 Using QPSK 1/8 (Number of 4 MLRUs) | Group 2 Using QPSK 1/2 (Number of 1 MLRUs) | Total Number of MLRUs | Number of Blind-Detection on A-A-MAP |
| --- | --- | --- | --- | --- |
| 1 | 0 | 1 | 1 | 1 |
| 2 | 0 | 2 | 2 | 2 |
| 3 | 0 | 3 | 3 | 3 |
| 4 | 0 | 5 | 5 | 5 |
| 5 | 0 | 6 | 6 | 6 |
| 6 | 0 | 7 | 7 | 7 |
| 7 | 0 | 8 | 8 | 8 |
| 8 | 0 | 9 | 9 | 9 |
| 9 | 0 | 10 | 10 | 10 |
| 10 | 0 | 11 | 11 | 11 |
| 11 | 0 | 12 | 12 | 12 |
| 12 | 0 | 14 | 14 | 14 |
| 13 | 0 | 15 | 15 | 15 |
| 14 | 0 | 16 | 16 | 16 |
| 15 | 0 | 17 | 17 | 17 |
| 16 | 0 | 18 | 18 | 18 |
| 17 | 0 | 19 | 19 | 19 |
| 18 | 0 | 20 | 20 | 20 |
| 19 | 0 | 22 | 22 | 22 |
| 20 | 0 | 23 | 23 | 23 |
| 21 | 0 | 24 | 24 | 24 |
| 22 | 0 | 25 | 25 | 25 |
| 23 | 0 | 26 | 26 | 26 |
| 24 | 0 | 27 | 27 | 27 |
| 25 | 0 | 28 | 28 | 28 |
| 26 | 0 | 29 | 29 | 29 |
| 27 | 0 | 31 | 31 | 31 |
| 28 | 0 | 32 | 32 | 32 |
| 29 | 0 | 33 | 33 | 33 |
| 30 | 0 | 34 | 34 | 34 |
| 31 | 0 | 36 | 36 | 36 |
| 32 | 0 | 39 | 39 | 39 |
| 33 | 0 | 40 | 40 | 40 |
| 34 | 0 | 41 | 41 | 41 |
| 35 | 0 | 43 | 43 | 43 |
| 36 | 0 | 44 | 44 | 44 |
| 37 | 0 | 46 | 46 | 46 |
| 38 | 0 | 48 | 48 | 48 |
| 39 | 1 | 0 | 4 | 1 |
| 40 | 1 | 1 | 5 | 2 |
| 41 | 1 | 3 | 7 | 4 |
| 42 | 1 | 4 | 8 | 5 |
| 43 | 1 | 5 | 9 | 6 |
| 44 | 1 | 7 | 11 | 8 |
| 45 | 1 | 8 | 12 | 9 |
| 46 | 1 | 9 | 13 | 10 |
| 47 | 1 | 11 | 15 | 12 |
| 48 | 1 | 12 | 16 | 13 |
| 49 | 1 | 13 | 17 | 14 |
| 50 | 1 | 14 | 18 | 15 |
| 51 | 1 | 15 | 19 | 16 |
| 52 | 1 | 17 | 21 | 18 |
| 53 | 1 | 18 | 22 | 19 |
| 54 | 1 | 19 | 23 | 20 |
| 55 | 1 | 21 | 25 | 22 |
| 56 | 1 | 22 | 26 | 23 |
| 57 | 1 | 23 | 27 | 24 |
| 58 | 1 | 24 | 28 | 25 |
| 59 | 1 | 25 | 29 | 26 |
| 60 | 1 | 26 | 30 | 27 |
| 61 | 1 | 27 | 31 | 28 |
| 62 | 1 | 28 | 32 | 29 |
| 63 | 1 | 29 | 33 | 30 |
| 64 | 1 | 30 | 34 | 31 |
| 65 | 1 | 31 | 35 | 32 |
| 66 | 1 | 33 | 37 | 34 |
| 67 | 1 | 34 | 38 | 35 |
| 68 | 1 | 35 | 39 | 36 |
| 69 | 1 | 36 | 40 | 37 |
| 70 | 1 | 37 | 41 | 38 |
| 71 | 1 | 38 | 42 | 39 |
| 72 | 1 | 39 | 43 | 40 |
| 73 | 1 | 40 | 44 | 41 |
| 74 | 1 | 41 | 45 | 42 |
| 75 | 1 | 42 | 46 | 43 |
| 76 | 1 | 43 | 47 | 44 |
| 77 | 1 | 44 | 48 | 45 |
| 78 | 2 | 1 | 9 | 3 |
| 79 | 2 | 2 | 10 | 4 |
| 80 | 2 | 4 | 12 | 6 |
| 81 | 2 | 5 | 13 | 7 |
| 82 | 2 | 6 | 14 | 8 |
| 83 | 2 | 12 | 20 | 14 |
| 84 | 2 | 13 | 21 | 15 |
| 85 | 2 | 14 | 22 | 16 |
| 86 | 2 | 16 | 24 | 18 |
| 87 | 2 | 17 | 25 | 19 |
| 88 | 2 | 19 | 27 | 21 |
| 89 | 2 | 21 | 29 | 23 |
| 90 | 2 | 22 | 30 | 24 |
| 91 | 2 | 24 | 32 | 26 |
| 92 | 2 | 25 | 33 | 27 |
| 93 | 2 | 26 | 34 | 28 |
| 94 | 2 | 27 | 35 | 29 |
| 95 | 2 | 28 | 36 | 30 |
| 96 | 2 | 29 | 37 | 31 |
| 97 | 2 | 30 | 38 | 32 |
| 98 | 2 | 31 | 39 | 33 |
| 99 | 2 | 32 | 40 | 34 |
| 100 | 2 | 34 | 42 | 36 |
| 101 | 2 | 35 | 43 | 37 |
| 102 | 2 | 37 | 45 | 39 |
| 103 | 2 | 39 | 47 | 41 |
| 104 | 3 | 0 | 12 | 3 |
| 105 | 3 | 1 | 13 | 4 |
| 106 | 3 | 2 | 14 | 5 |
| 107 | 3 | 3 | 15 | 6 |
| 108 | 3 | 4 | 16 | 7 |
| 109 | 3 | 5 | 17 | 8 |
| 110 | 3 | 6 | 18 | 9 |
| 111 | 3 | 7 | 19 | 10 |
| 112 | 3 | 8 | 20 | 11 |
| 113 | 3 | 9 | 21 | 12 |
| 114 | 3 | 10 | 22 | 13 |
| 115 | 3 | 11 | 23 | 14 |
| 116 | 3 | 12 | 24 | 15 |
| 117 | 3 | 13 | 25 | 16 |
| 118 | 3 | 14 | 26 | 17 |
| 119 | 3 | 15 | 27 | 18 |
| 120 | 3 | 16 | 28 | 19 |
| 121 | 3 | 18 | 30 | 21 |
| 122 | 3 | 19 | 31 | 22 |
| 123 | 3 | 21 | 33 | 24 |
| 124 | 3 | 22 | 34 | 25 |
| 125 | 3 | 23 | 35 | 26 |
| 126 | 3 | 24 | 36 | 27 |
| 127 | 3 | 25 | 37 | 28 |
| 128 | 3 | 26 | 38 | 29 |
| 129 | 3 | 27 | 39 | 30 |
| 130 | 3 | 29 | 41 | 32 |
| 131 | 3 | 30 | 42 | 33 |
| 132 | 3 | 31 | 43 | 34 |
| 133 | 3 | 32 | 44 | 35 |
| 134 | 3 | 33 | 45 | 36 |
| 135 | 3 | 34 | 46 | 37 |
| 136 | 3 | 35 | 47 | 38 |
| 137 | 3 | 36 | 48 | 39 |
| 138 | 4 | 0 | 16 | 4 |
| 139 | 4 | 1 | 17 | 5 |
| 140 | 4 | 2 | 18 | 6 |
| 141 | 4 | 3 | 19 | 7 |
| 142 | 4 | 4 | 20 | 8 |
| 143 | 4 | 5 | 21 | 9 |
| 144 | 4 | 7 | 23 | 11 |
| 145 | 4 | 8 | 24 | 12 |
| 146 | 4 | 10 | 26 | 14 |
| 147 | 4 | 12 | 28 | 16 |
| 148 | 4 | 13 | 29 | 17 |
| 149 | 4 | 14 | 30 | 18 |
| 150 | 4 | 15 | 31 | 19 |
| 151 | 4 | 16 | 32 | 20 |
| 152 | 4 | 19 | 35 | 23 |
| 153 | 4 | 20 | 36 | 24 |
| 154 | 4 | 21 | 37 | 25 |
| 155 | 4 | 24 | 40 | 28 |
| 156 | 4 | 25 | 41 | 29 |
| 157 | 4 | 26 | 42 | 30 |
| 158 | 4 | 28 | 44 | 32 |
| 159 | 4 | 29 | 45 | 33 |
| 160 | 4 | 30 | 46 | 34 |
| 161 | 4 | 32 | 48 | 36 |
| 162 | 5 | 0 | 20 | 5 |
| 163 | 5 | 2 | 22 | 7 |
| 164 | 5 | 3 | 23 | 8 |
| 165 | 5 | 4 | 24 | 9 |
| 166 | 5 | 5 | 25 | 10 |
| 167 | 5 | 6 | 26 | 11 |
| 168 | 5 | 7 | 27 | 12 |
| 169 | 5 | 8 | 28 | 13 |
| 170 | 5 | 9 | 29 | 14 |
| 171 | 5 | 11 | 31 | 16 |
| 172 | 5 | 12 | 32 | 17 |
| 173 | 5 | 13 | 33 | 18 |
| 174 | 5 | 14 | 34 | 19 |
| 175 | 5 | 18 | 38 | 23 |
| 176 | 5 | 19 | 39 | 24 |
| 177 | 5 | 20 | 40 | 25 |
| 178 | 5 | 21 | 41 | 26 |
| 179 | 5 | 23 | 43 | 28 |
| 180 | 5 | 24 | 44 | 29 |
| 181 | 5 | 26 | 46 | 31 |
| 182 | 5 | 27 | 47 | 32 |
| 183 | 5 | 28 | 48 | 33 |
| 184 | 6 | 1 | 25 | 7 |
| 185 | 6 | 3 | 27 | 9 |
| 186 | 6 | 4 | 28 | 10 |
| 187 | 6 | 5 | 29 | 11 |
| 188 | 6 | 6 | 30 | 12 |
| 189 | 6 | 8 | 32 | 14 |
| 190 | 6 | 9 | 33 | 15 |
| 191 | 6 | 10 | 34 | 16 |
| 192 | 6 | 11 | 35 | 17 |
| 193 | 6 | 12 | 36 | 18 |
| 194 | 6 | 13 | 37 | 19 |
| 195 | 6 | 14 | 38 | 20 |
| 196 | 6 | 15 | 39 | 21 |
| 197 | 6 | 16 | 40 | 22 |
| 198 | 6 | 17 | 41 | 23 |
| 199 | 6 | 18 | 42 | 24 |
| 200 | 6 | 19 | 43 | 25 |
| 201 | 6 | 20 | 44 | 26 |
| 202 | 6 | 21 | 45 | 27 |
| 203 | 6 | 22 | 46 | 28 |
| 204 | 6 | 23 | 47 | 29 |
| 205 | 6 | 24 | 48 | 30 |
| 206 | 7 | 1 | 29 | 8 |
| 207 | 7 | 2 | 30 | 9 |
| 208 | 7 | 3 | 31 | 10 |
| 209 | 7 | 4 | 32 | 11 |
| 210 | 7 | 5 | 33 | 12 |
| 211 | 7 | 6 | 34 | 13 |
| 212 | 7 | 7 | 35 | 14 |
| 213 | 7 | 8 | 36 | 15 |
| 214 | 7 | 9 | 37 | 16 |
| 215 | 7 | 10 | 38 | 17 |
| 216 | 7 | 11 | 39 | 18 |
| 217 | 7 | 12 | 40 | 19 |
| 218 | 7 | 14 | 42 | 21 |
| 219 | 7 | 15 | 43 | 22 |
| 220 | 7 | 17 | 45 | 24 |
| 221 | 7 | 19 | 47 | 26 |
| 222 | 8 | 2 | 34 | 10 |
| 223 | 8 | 3 | 35 | 11 |
| 224 | 8 | 4 | 36 | 12 |
| 225 | 8 | 5 | 37 | 13 |
| 226 | 8 | 6 | 38 | 14 |
| 227 | 8 | 7 | 39 | 15 |
| 228 | 8 | 9 | 41 | 17 |
| 229 | 8 | 10 | 42 | 18 |
| 230 | 8 | 11 | 43 | 19 |
| 231 | 8 | 12 | 44 | 20 |
| 232 | 8 | 13 | 45 | 21 |
| 233 | 8 | 14 | 46 | 22 |
| 234 | 8 | 15 | 47 | 23 |
| 235 | 8 | 16 | 48 | 24 |
| 236 | 9 | 0 | 36 | 9 |
| 237 | 9 | 1 | 37 | 10 |
| 238 | 9 | 4 | 40 | 13 |
| 239 | 9 | 5 | 41 | 14 |
| 240 | 9 | 6 | 42 | 15 |
| 241 | 9 | 8 | 44 | 17 |
| 242 | 9 | 10 | 46 | 19 |
| 243 | 9 | 12 | 48 | 21 |
| 244 | 10 | 0 | 40 | 10 |
| 245 | 10 | 1 | 41 | 11 |
| 246 | 10 | 3 | 43 | 13 |
| 247 | 10 | 4 | 44 | 14 |
| 248 | 10 | 5 | 45 | 15 |
| 249 | 10 | 6 | 46 | 16 |
| 250 | 10 | 7 | 47 | 17 |
| 251 | 10 | 8 | 48 | 18 |
| 252 | 11 | 0 | 44 | 11 |
| 253 | 11 | 1 | 45 | 12 |
| 254 | 11 | 2 | 46 | 13 |
| 255 | 11 | 3 | 47 | 14 |
| 256 | 12 | 0 | 48 | 12 |

## Resource allocation Indication Table of MLRUs for FFR Reuse 3 A-A-MAP Using QPSK 1/2 and for FFR Reuse 1 A-A-MAP Using QPSK (1/2, 1/8)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal;=(N1+4∗N2+N3)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤48) is the number of FFR Reuse 3 A-A-MAP in Group 1 using QPSK 1/2; N2 (0≤N2≤12) is the number of 1 MLRUs for FFR Reuse 1 A-A-MAP in Group 2 using QPSK 1/8; N3 (0≤N3≤48) is the number of FFR Reuse 1 A-A-MAP in Group 3 using QPSK 1/2.

Group 1 is adjacent to data transmission in FFR Reuse 3, Group 2 is not adjacent to data transmission in FFR Reuse 1, and Group 3 is adjacent to data transmission in FFR Reuse 1. Therefore unoccupied MLRUs in Group 2 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

  - N1 may be [0:4:48];

  - N2 may be [0:1:12];

  - N3 may be [0:4:48];

The total number of the remaining combinations is reduced to 455, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

Δ=455(455−256)=2.29

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δcombinations. The following are the combination indices to be removed: [1,3,5,7,10,12,14,17,19,21,23,26,28, 30,33,35,37,39,42,44,46,49,51,53,56,58,60,62,65,67,69,72,74,76,78,81,83,85,88,90,92,95,97,99,101,104,106, 108,111,113,115,117,120,122,124,127,129,131,133,136,138,140,143,145,147,150,152,154,156,159,161,163, 166,168,170,172,175,177,179,182,184,186,189,191,193,195,198,200,202,205,207,209,211,214,216,218,221, 223,225,227,230,232,234,237,239,241,244,246,248,250,253,255,257,260,262,264,266,269,271,273,276,278, 280,283,285,287,289,292,294,296,299,301,303,305,308,310,312,315,317,319,322,324,326,328,331,333,335, 338,340,342,344,347,349,351,354,356,358,360,363,365,367,370,372,374,377,379,381,383,386,388,390,393, 395,397,399,402,404,406,409,411,413,416,418,420,422,425,427,429,432,434,436,438,441,443,445,448,450,452,454].

Figure 3: Average Extra Blind Detection and Average Resource Wastes

Table 4: 8-bits Resource Allocation Indication of MLRUs for FFR Group 1 Using QPSK 1/2, Group 2 Using QPSK 1/8, and Group 3 Using 1/2 at 5, 10, 20MHz BW

| Index | Group 1 Using QPSK 1/2 (Number of 1 MLRU) | Group 2 Using QPSK 1/8 (Number of 4 MLRUs) | Group 3 Using QPSK 1/2 (Number of 1 MLRU ) | Total Number of MLRUs | Number of Blind-Detection on A-A-MAP |
| --- | --- | --- | --- | --- | --- |
| 1 | 4 | 0 | 0 | 4 | 4 |
| 2 | 4 | 0 | 4 | 8 | 8 |
| 3 | 4 | 0 | 8 | 12 | 12 |
| 4 | 12 | 0 | 0 | 12 | 12 |
| 5 | 0 | 0 | 16 | 16 | 16 |
| 6 | 4 | 0 | 12 | 16 | 16 |
| 7 | 12 | 0 | 4 | 16 | 16 |
| 8 | 0 | 0 | 20 | 20 | 20 |
| 9 | 4 | 0 | 16 | 20 | 20 |
| 10 | 12 | 0 | 8 | 20 | 20 |
| 11 | 20 | 0 | 0 | 20 | 20 |
| 12 | 0 | 0 | 24 | 24 | 24 |
| 13 | 8 | 0 | 16 | 24 | 24 |
| 14 | 16 | 0 | 8 | 24 | 24 |
| 15 | 24 | 0 | 0 | 24 | 24 |
| 16 | 4 | 0 | 24 | 28 | 28 |
| 17 | 12 | 0 | 16 | 28 | 28 |
| 18 | 20 | 0 | 8 | 28 | 28 |
| 19 | 24 | 0 | 4 | 28 | 28 |
| 20 | 4 | 0 | 28 | 32 | 32 |
| 21 | 12 | 0 | 20 | 32 | 32 |
| 22 | 16 | 0 | 16 | 32 | 32 |
| 23 | 24 | 0 | 8 | 32 | 32 |
| 24 | 32 | 0 | 0 | 32 | 32 |
| 25 | 4 | 0 | 32 | 36 | 36 |
| 26 | 8 | 0 | 28 | 36 | 36 |
| 27 | 16 | 0 | 20 | 36 | 36 |
| 28 | 24 | 0 | 12 | 36 | 36 |
| 29 | 32 | 0 | 4 | 36 | 36 |
| 30 | 0 | 0 | 40 | 40 | 40 |
| 31 | 8 | 0 | 32 | 40 | 40 |
| 32 | 12 | 0 | 28 | 40 | 40 |
| 33 | 20 | 0 | 20 | 40 | 40 |
| 34 | 28 | 0 | 12 | 40 | 40 |
| 35 | 36 | 0 | 4 | 40 | 40 |
| 36 | 0 | 0 | 44 | 44 | 44 |
| 37 | 4 | 0 | 40 | 44 | 44 |
| 38 | 12 | 0 | 32 | 44 | 44 |
| 39 | 20 | 0 | 24 | 44 | 44 |
| 40 | 28 | 0 | 16 | 44 | 44 |
| 41 | 32 | 0 | 12 | 44 | 44 |
| 42 | 40 | 0 | 4 | 44 | 44 |
| 43 | 4 | 0 | 44 | 48 | 48 |
| 44 | 8 | 0 | 40 | 48 | 48 |
| 45 | 16 | 0 | 32 | 48 | 48 |
| 46 | 24 | 0 | 24 | 48 | 48 |
| 47 | 32 | 0 | 16 | 48 | 48 |
| 48 | 36 | 0 | 12 | 48 | 48 |
| 49 | 44 | 0 | 4 | 48 | 48 |
| 50 | 0 | 1 | 0 | 4 | 1 |
| 51 | 0 | 1 | 4 | 8 | 5 |
| 52 | 4 | 1 | 0 | 8 | 5 |
| 53 | 4 | 1 | 4 | 12 | 9 |
| 54 | 8 | 1 | 0 | 12 | 9 |
| 55 | 0 | 1 | 12 | 16 | 13 |
| 56 | 8 | 1 | 4 | 16 | 13 |
| 57 | 4 | 1 | 12 | 20 | 17 |
| 58 | 12 | 1 | 4 | 20 | 17 |
| 59 | 4 | 1 | 16 | 24 | 21 |
| 60 | 12 | 1 | 8 | 24 | 21 |
| 61 | 20 | 1 | 0 | 24 | 21 |
| 62 | 4 | 1 | 20 | 28 | 25 |
| 63 | 12 | 1 | 12 | 28 | 25 |
| 64 | 20 | 1 | 4 | 28 | 25 |
| 65 | 24 | 1 | 0 | 28 | 25 |
| 66 | 0 | 1 | 28 | 32 | 29 |
| 67 | 8 | 1 | 20 | 32 | 29 |
| 68 | 12 | 1 | 16 | 32 | 29 |
| 69 | 20 | 1 | 8 | 32 | 29 |
| 70 | 28 | 1 | 0 | 32 | 29 |
| 71 | 4 | 1 | 28 | 36 | 33 |
| 72 | 12 | 1 | 20 | 36 | 33 |
| 73 | 16 | 1 | 16 | 36 | 33 |
| 74 | 24 | 1 | 8 | 36 | 33 |
| 75 | 32 | 1 | 0 | 36 | 33 |
| 76 | 4 | 1 | 32 | 40 | 37 |
| 77 | 12 | 1 | 24 | 40 | 37 |
| 78 | 20 | 1 | 16 | 40 | 37 |
| 79 | 24 | 1 | 12 | 40 | 37 |
| 80 | 32 | 1 | 4 | 40 | 37 |
| 81 | 0 | 1 | 40 | 44 | 41 |
| 82 | 8 | 1 | 32 | 44 | 41 |
| 83 | 12 | 1 | 28 | 44 | 41 |
| 84 | 20 | 1 | 20 | 44 | 41 |
| 85 | 28 | 1 | 12 | 44 | 41 |
| 86 | 36 | 1 | 4 | 44 | 41 |
| 87 | 0 | 1 | 44 | 48 | 45 |
| 88 | 8 | 1 | 36 | 48 | 45 |
| 89 | 16 | 1 | 28 | 48 | 45 |
| 90 | 20 | 1 | 24 | 48 | 45 |
| 91 | 28 | 1 | 16 | 48 | 45 |
| 92 | 36 | 1 | 8 | 48 | 45 |
| 93 | 44 | 1 | 0 | 48 | 45 |
| 94 | 4 | 2 | 0 | 12 | 6 |
| 95 | 0 | 2 | 8 | 16 | 10 |
| 96 | 4 | 2 | 4 | 16 | 10 |
| 97 | 0 | 2 | 12 | 20 | 14 |
| 98 | 8 | 2 | 4 | 20 | 14 |
| 99 | 12 | 2 | 0 | 20 | 14 |
| 100 | 4 | 2 | 12 | 24 | 18 |
| 101 | 12 | 2 | 4 | 24 | 18 |
| 102 | 16 | 2 | 0 | 24 | 18 |
| 103 | 0 | 2 | 20 | 28 | 22 |
| 104 | 8 | 2 | 12 | 28 | 22 |
| 105 | 12 | 2 | 8 | 28 | 22 |
| 106 | 20 | 2 | 0 | 28 | 22 |
| 107 | 0 | 2 | 24 | 32 | 26 |
| 108 | 8 | 2 | 16 | 32 | 26 |
| 109 | 12 | 2 | 12 | 32 | 26 |
| 110 | 20 | 2 | 4 | 32 | 26 |
| 111 | 0 | 2 | 28 | 36 | 30 |
| 112 | 8 | 2 | 20 | 36 | 30 |
| 113 | 16 | 2 | 12 | 36 | 30 |
| 114 | 20 | 2 | 8 | 36 | 30 |
| 115 | 28 | 2 | 0 | 36 | 30 |
| 116 | 4 | 2 | 28 | 40 | 34 |
| 117 | 12 | 2 | 20 | 40 | 34 |
| 118 | 20 | 2 | 12 | 40 | 34 |
| 119 | 28 | 2 | 4 | 40 | 34 |
| 120 | 32 | 2 | 0 | 40 | 34 |
| 121 | 4 | 2 | 32 | 44 | 38 |
| 122 | 12 | 2 | 24 | 44 | 38 |
| 123 | 20 | 2 | 16 | 44 | 38 |
| 124 | 24 | 2 | 12 | 44 | 38 |
| 125 | 32 | 2 | 4 | 44 | 38 |
| 126 | 0 | 2 | 40 | 48 | 42 |
| 127 | 8 | 2 | 32 | 48 | 42 |
| 128 | 16 | 2 | 24 | 48 | 42 |
| 129 | 24 | 2 | 16 | 48 | 42 |
| 130 | 32 | 2 | 8 | 48 | 42 |
| 131 | 36 | 2 | 4 | 48 | 42 |
| 132 | 0 | 3 | 0 | 12 | 3 |
| 133 | 0 | 3 | 4 | 16 | 7 |
| 134 | 4 | 3 | 4 | 20 | 11 |
| 135 | 4 | 3 | 8 | 24 | 15 |
| 136 | 12 | 3 | 0 | 24 | 15 |
| 137 | 0 | 3 | 16 | 28 | 19 |
| 138 | 4 | 3 | 12 | 28 | 19 |
| 139 | 12 | 3 | 4 | 28 | 19 |
| 140 | 4 | 3 | 16 | 32 | 23 |
| 141 | 8 | 3 | 12 | 32 | 23 |
| 142 | 16 | 3 | 4 | 32 | 23 |
| 143 | 0 | 3 | 24 | 36 | 27 |
| 144 | 8 | 3 | 16 | 36 | 27 |
| 145 | 12 | 3 | 12 | 36 | 27 |
| 146 | 20 | 3 | 4 | 36 | 27 |
| 147 | 0 | 3 | 28 | 40 | 31 |
| 148 | 8 | 3 | 20 | 40 | 31 |
| 149 | 16 | 3 | 12 | 40 | 31 |
| 150 | 24 | 3 | 4 | 40 | 31 |
| 151 | 28 | 3 | 0 | 40 | 31 |
| 152 | 4 | 3 | 28 | 44 | 35 |
| 153 | 12 | 3 | 20 | 44 | 35 |
| 154 | 20 | 3 | 12 | 44 | 35 |
| 155 | 28 | 3 | 4 | 44 | 35 |
| 156 | 32 | 3 | 0 | 44 | 35 |
| 157 | 0 | 3 | 36 | 48 | 39 |
| 158 | 8 | 3 | 28 | 48 | 39 |
| 159 | 12 | 3 | 24 | 48 | 39 |
| 160 | 20 | 3 | 16 | 48 | 39 |
| 161 | 28 | 3 | 8 | 48 | 39 |
| 162 | 36 | 3 | 0 | 48 | 39 |
| 163 | 0 | 4 | 4 | 20 | 8 |
| 164 | 4 | 4 | 0 | 20 | 8 |
| 165 | 4 | 4 | 4 | 24 | 12 |
| 166 | 8 | 4 | 0 | 24 | 12 |
| 167 | 0 | 4 | 12 | 28 | 16 |
| 168 | 8 | 4 | 4 | 28 | 16 |
| 169 | 0 | 4 | 16 | 32 | 20 |
| 170 | 8 | 4 | 8 | 32 | 20 |
| 171 | 16 | 4 | 0 | 32 | 20 |
| 172 | 4 | 4 | 16 | 36 | 24 |
| 173 | 8 | 4 | 12 | 36 | 24 |
| 174 | 16 | 4 | 4 | 36 | 24 |
| 175 | 0 | 4 | 24 | 40 | 28 |
| 176 | 8 | 4 | 16 | 40 | 28 |
| 177 | 16 | 4 | 8 | 40 | 28 |
| 178 | 24 | 4 | 0 | 40 | 28 |
| 179 | 0 | 4 | 28 | 44 | 32 |
| 180 | 8 | 4 | 20 | 44 | 32 |
| 181 | 16 | 4 | 12 | 44 | 32 |
| 182 | 24 | 4 | 4 | 44 | 32 |
| 183 | 28 | 4 | 0 | 44 | 32 |
| 184 | 0 | 4 | 32 | 48 | 36 |
| 185 | 8 | 4 | 24 | 48 | 36 |
| 186 | 16 | 4 | 16 | 48 | 36 |
| 187 | 20 | 4 | 12 | 48 | 36 |
| 188 | 28 | 4 | 4 | 48 | 36 |
| 189 | 4 | 5 | 0 | 24 | 9 |
| 190 | 4 | 5 | 4 | 28 | 13 |
| 191 | 4 | 5 | 8 | 32 | 17 |
| 192 | 8 | 5 | 4 | 32 | 17 |
| 193 | 0 | 5 | 16 | 36 | 21 |
| 194 | 8 | 5 | 8 | 36 | 21 |
| 195 | 16 | 5 | 0 | 36 | 21 |
| 196 | 4 | 5 | 16 | 40 | 25 |
| 197 | 12 | 5 | 8 | 40 | 25 |
| 198 | 16 | 5 | 4 | 40 | 25 |
| 199 | 0 | 5 | 24 | 44 | 29 |
| 200 | 8 | 5 | 16 | 44 | 29 |
| 201 | 16 | 5 | 8 | 44 | 29 |
| 202 | 24 | 5 | 0 | 44 | 29 |
| 203 | 4 | 5 | 24 | 48 | 33 |
| 204 | 12 | 5 | 16 | 48 | 33 |
| 205 | 16 | 5 | 12 | 48 | 33 |
| 206 | 24 | 5 | 4 | 48 | 33 |
| 207 | 0 | 6 | 0 | 24 | 6 |
| 208 | 0 | 6 | 4 | 28 | 10 |
| 209 | 4 | 6 | 0 | 28 | 10 |
| 210 | 0 | 6 | 8 | 32 | 14 |
| 211 | 8 | 6 | 0 | 32 | 14 |
| 212 | 4 | 6 | 8 | 36 | 18 |
| 213 | 8 | 6 | 4 | 36 | 18 |
| 214 | 4 | 6 | 12 | 40 | 22 |
| 215 | 8 | 6 | 8 | 40 | 22 |
| 216 | 16 | 6 | 0 | 40 | 22 |
| 217 | 4 | 6 | 16 | 44 | 26 |
| 218 | 12 | 6 | 8 | 44 | 26 |
| 219 | 16 | 6 | 4 | 44 | 26 |
| 220 | 4 | 6 | 20 | 48 | 30 |
| 221 | 12 | 6 | 12 | 48 | 30 |
| 222 | 16 | 6 | 8 | 48 | 30 |
| 223 | 24 | 6 | 0 | 48 | 30 |
| 224 | 0 | 7 | 4 | 32 | 11 |
| 225 | 0 | 7 | 8 | 36 | 15 |
| 226 | 8 | 7 | 0 | 36 | 15 |
| 227 | 4 | 7 | 8 | 40 | 19 |
| 228 | 12 | 7 | 0 | 40 | 19 |
| 229 | 4 | 7 | 12 | 44 | 23 |
| 230 | 8 | 7 | 8 | 44 | 23 |
| 231 | 16 | 7 | 0 | 44 | 23 |
| 232 | 0 | 7 | 20 | 48 | 27 |
| 233 | 4 | 7 | 16 | 48 | 27 |
| 234 | 12 | 7 | 8 | 48 | 27 |
| 235 | 20 | 7 | 0 | 48 | 27 |
| 236 | 0 | 8 | 0 | 32 | 8 |
| 237 | 0 | 8 | 4 | 36 | 12 |
| 238 | 4 | 8 | 4 | 40 | 16 |
| 239 | 8 | 8 | 0 | 40 | 16 |
| 240 | 4 | 8 | 8 | 44 | 20 |
| 241 | 12 | 8 | 0 | 44 | 20 |
| 242 | 4 | 8 | 12 | 48 | 24 |
| 243 | 12 | 8 | 4 | 48 | 24 |
| 244 | 0 | 9 | 0 | 36 | 9 |
| 245 | 4 | 9 | 0 | 40 | 13 |
| 246 | 0 | 9 | 8 | 44 | 17 |
| 247 | 8 | 9 | 0 | 44 | 17 |
| 248 | 0 | 9 | 12 | 48 | 21 |
| 249 | 8 | 9 | 4 | 48 | 21 |
| 250 | 12 | 9 | 0 | 48 | 21 |
| 251 | 0 | 10 | 0 | 40 | 10 |
| 252 | 4 | 10 | 0 | 44 | 14 |
| 253 | 4 | 10 | 4 | 48 | 18 |
| 254 | 0 | 11 | 0 | 44 | 11 |
| 255 | 4 | 11 | 0 | 48 | 15 |
| 256 | 0 | 12 | 0 | 48 | 12 |

## Resource allocation Indication Table of MLRUs for FFR Reuse 3 A-A-MAP Using QPSK 1/2 and for FFR Reuse 1 A-A-MAP Using QPSK (1/2, 1/4)

Below describes how to generate the resource allocation indication for A-A-MAP to meet the resource waste as few as possible:

Ntotal;=(N1+2∗N2+N3)≤48

wherein Ntotal (0≤Ntotal≤48) is the total number of MLRUs for A-A-MAP; N1 (0≤N1≤48) is the number of FFR A-A-MAP Reuse 3 in Group 1 using QPSK 1/2; N2 (0≤N2≤24) is the number of FFR A-A-MAP Reuse 1 in Group 2 using QPSK 1/4; N3 (0≤N3≤48) is the number of FFR A-A-MAP Reuse 1 in Group 3 using QPSK 1/2.

Group 1 is adjacent to data transmission in FFR Reuse 3, Group 2 is not adjacent to the data transmission in FFR Reuse 1, and Group 3 is adjacent to the data transmission in FFR reuse 1. Therefore unoccupied MLRUs in Group 2 are wasted. In order to reduce the waste as much as possible, the following rules are applied to remove the combinations first:

- N1 may be [0:5:48];

- N2 may be [0:2:24];

- N3 may be [0:4:48];

The total number of remaining combination is reduced to 346, which is still larger than the requirement of 8-bit signaling overhead. The following is the second combination removing operation:

Δ=346(346−256)=3.84

whereinΔis the removing step. From the new index 1, the combinations are uniformly removed every Δcombinations. The following are the combination indices to be removed: [1,4,8,12,16,20,24,28,32,35,39,43, 47,51,55,59,63,66,70,74,78,82,86,90,94,97,101,105,109,113,117,121,125,128,132,136,140,144,148,152,156, 159,163,167,171,175,179,183,187,190,194,198,202,206,210,214,218,221,225,229,233,237,241,245,249,252, 256,260,264,268,272,276,280,283,287,291,295,299,303,307,311,314,318,322,326,330,334,338,342,345].

Figure 4: Average Extra Blind Detection and Average Resource Wastes

Table 5: 8-bits Resource Allocation Indication of MLRUs for FFR Group 1 Using QPSK 1/2, Group 2 Using QPSK 1/4, and Group 3 Using QPSK 1/2 at 5, 10, 20MHz BW

| Index | Group 1 Using QPSK 1/2 (Number of 1 MLRU) | Group 2 Using QPSK 1/4 (Number of 2 MLRUs) | Group 3 Using QPSK 1/2 (Number of 1 MLRU) | Total Number of MLRUs | Number of Blind-Detection on A-A-MAP |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 4 | 4 | 4 |
| 2 | 0 | 0 | 8 | 8 | 8 |
| 3 | 10 | 0 | 0 | 10 | 10 |
| 4 | 0 | 0 | 12 | 12 | 12 |
| 5 | 5 | 0 | 8 | 13 | 13 |
| 6 | 10 | 0 | 4 | 14 | 14 |
| 7 | 0 | 0 | 16 | 16 | 16 |
| 8 | 5 | 0 | 12 | 17 | 17 |
| 9 | 10 | 0 | 8 | 18 | 18 |
| 10 | 15 | 0 | 4 | 19 | 19 |
| 11 | 0 | 0 | 20 | 20 | 20 |
| 12 | 5 | 0 | 16 | 21 | 21 |
| 13 | 20 | 0 | 4 | 24 | 24 |
| 14 | 5 | 0 | 20 | 25 | 25 |
| 15 | 15 | 0 | 12 | 27 | 27 |
| 16 | 0 | 0 | 28 | 28 | 28 |
| 17 | 20 | 0 | 8 | 28 | 28 |
| 18 | 5 | 0 | 24 | 29 | 29 |
| 19 | 10 | 0 | 20 | 30 | 30 |
| 20 | 30 | 0 | 0 | 30 | 30 |
| 21 | 15 | 0 | 16 | 31 | 31 |
| 22 | 0 | 0 | 32 | 32 | 32 |
| 23 | 20 | 0 | 12 | 32 | 32 |
| 24 | 25 | 0 | 8 | 33 | 33 |
| 25 | 30 | 0 | 4 | 34 | 34 |
| 26 | 15 | 0 | 20 | 35 | 35 |
| 27 | 20 | 0 | 16 | 36 | 36 |
| 28 | 25 | 0 | 12 | 37 | 37 |
| 29 | 10 | 0 | 28 | 38 | 38 |
| 30 | 15 | 0 | 24 | 39 | 39 |
| 31 | 0 | 0 | 40 | 40 | 40 |
| 32 | 20 | 0 | 20 | 40 | 40 |
| 33 | 40 | 0 | 0 | 40 | 40 |
| 34 | 25 | 0 | 16 | 41 | 41 |
| 35 | 30 | 0 | 12 | 42 | 42 |
| 36 | 15 | 0 | 28 | 43 | 43 |
| 37 | 0 | 0 | 44 | 44 | 44 |
| 38 | 20 | 0 | 24 | 44 | 44 |
| 39 | 5 | 0 | 40 | 45 | 45 |
| 40 | 25 | 0 | 20 | 45 | 45 |
| 41 | 45 | 0 | 0 | 45 | 45 |
| 42 | 10 | 0 | 36 | 46 | 46 |
| 43 | 35 | 0 | 12 | 47 | 47 |
| 44 | 0 | 0 | 48 | 48 | 48 |
| 45 | 20 | 0 | 28 | 48 | 48 |
| 46 | 40 | 0 | 8 | 48 | 48 |
| 47 | 0 | 2 | 0 | 4 | 2 |
| 48 | 0 | 2 | 4 | 8 | 6 |
| 49 | 5 | 2 | 0 | 9 | 7 |
| 50 | 10 | 2 | 0 | 14 | 12 |
| 51 | 0 | 2 | 12 | 16 | 14 |
| 52 | 5 | 2 | 8 | 17 | 15 |
| 53 | 10 | 2 | 4 | 18 | 16 |
| 54 | 15 | 2 | 0 | 19 | 17 |
| 55 | 0 | 2 | 16 | 20 | 18 |
| 56 | 10 | 2 | 8 | 22 | 20 |
| 57 | 15 | 2 | 4 | 23 | 21 |
| 58 | 0 | 2 | 20 | 24 | 22 |
| 59 | 20 | 2 | 0 | 24 | 22 |
| 60 | 5 | 2 | 16 | 25 | 23 |
| 61 | 10 | 2 | 12 | 26 | 24 |
| 62 | 15 | 2 | 8 | 27 | 25 |
| 63 | 20 | 2 | 4 | 28 | 26 |
| 64 | 5 | 2 | 20 | 29 | 27 |
| 65 | 25 | 2 | 0 | 29 | 27 |
| 66 | 15 | 2 | 12 | 31 | 29 |
| 67 | 0 | 2 | 28 | 32 | 30 |
| 68 | 5 | 2 | 24 | 33 | 31 |
| 69 | 25 | 2 | 4 | 33 | 31 |
| 70 | 10 | 2 | 20 | 34 | 32 |
| 71 | 30 | 2 | 0 | 34 | 32 |
| 72 | 15 | 2 | 16 | 35 | 33 |
| 73 | 0 | 2 | 32 | 36 | 34 |
| 74 | 20 | 2 | 12 | 36 | 34 |
| 75 | 5 | 2 | 28 | 37 | 35 |
| 76 | 25 | 2 | 8 | 37 | 35 |
| 77 | 10 | 2 | 24 | 38 | 36 |
| 78 | 30 | 2 | 4 | 38 | 36 |
| 79 | 15 | 2 | 20 | 39 | 37 |
| 80 | 35 | 2 | 0 | 39 | 37 |
| 81 | 0 | 2 | 36 | 40 | 38 |
| 82 | 5 | 2 | 32 | 41 | 39 |
| 83 | 25 | 2 | 12 | 41 | 39 |
| 84 | 10 | 2 | 28 | 42 | 40 |
| 85 | 30 | 2 | 8 | 42 | 40 |
| 86 | 15 | 2 | 24 | 43 | 41 |
| 87 | 35 | 2 | 4 | 43 | 41 |
| 88 | 0 | 2 | 40 | 44 | 42 |
| 89 | 40 | 2 | 0 | 44 | 42 |
| 90 | 5 | 2 | 36 | 45 | 43 |
| 91 | 10 | 2 | 32 | 46 | 44 |
| 92 | 30 | 2 | 12 | 46 | 44 |
| 93 | 15 | 2 | 28 | 47 | 45 |
| 94 | 35 | 2 | 8 | 47 | 45 |
| 95 | 0 | 2 | 44 | 48 | 46 |
| 96 | 20 | 2 | 24 | 48 | 46 |
| 97 | 0 | 4 | 0 | 8 | 4 |
| 98 | 0 | 4 | 4 | 12 | 8 |
| 99 | 5 | 4 | 0 | 13 | 9 |
| 100 | 0 | 4 | 8 | 16 | 12 |
| 101 | 0 | 4 | 12 | 20 | 16 |
| 102 | 5 | 4 | 8 | 21 | 17 |
| 103 | 10 | 4 | 4 | 22 | 18 |
| 104 | 15 | 4 | 0 | 23 | 19 |
| 105 | 0 | 4 | 16 | 24 | 20 |
| 106 | 10 | 4 | 8 | 26 | 22 |
| 107 | 15 | 4 | 4 | 27 | 23 |
| 108 | 0 | 4 | 20 | 28 | 24 |
| 109 | 20 | 4 | 0 | 28 | 24 |
| 110 | 10 | 4 | 12 | 30 | 26 |
| 111 | 15 | 4 | 8 | 31 | 27 |
| 112 | 0 | 4 | 24 | 32 | 28 |
| 113 | 20 | 4 | 4 | 32 | 28 |
| 114 | 5 | 4 | 20 | 33 | 29 |
| 115 | 10 | 4 | 16 | 34 | 30 |
| 116 | 15 | 4 | 12 | 35 | 31 |
| 117 | 20 | 4 | 8 | 36 | 32 |
| 118 | 25 | 4 | 4 | 37 | 33 |
| 119 | 10 | 4 | 20 | 38 | 34 |
| 120 | 0 | 4 | 32 | 40 | 36 |
| 121 | 20 | 4 | 12 | 40 | 36 |
| 122 | 5 | 4 | 28 | 41 | 37 |
| 123 | 30 | 4 | 4 | 42 | 38 |
| 124 | 15 | 4 | 20 | 43 | 39 |
| 125 | 0 | 4 | 36 | 44 | 40 |
| 126 | 20 | 4 | 16 | 44 | 40 |
| 127 | 25 | 4 | 12 | 45 | 41 |
| 128 | 10 | 4 | 28 | 46 | 42 |
| 129 | 15 | 4 | 24 | 47 | 43 |
| 130 | 0 | 4 | 40 | 48 | 44 |
| 131 | 40 | 4 | 0 | 48 | 44 |
| 132 | 0 | 6 | 0 | 12 | 6 |
| 133 | 5 | 6 | 0 | 17 | 11 |
| 134 | 5 | 6 | 4 | 21 | 15 |
| 135 | 10 | 6 | 0 | 22 | 16 |
| 136 | 5 | 6 | 8 | 25 | 19 |
| 137 | 10 | 6 | 4 | 26 | 20 |
| 138 | 0 | 6 | 16 | 28 | 22 |
| 139 | 5 | 6 | 12 | 29 | 23 |
| 140 | 10 | 6 | 8 | 30 | 24 |
| 141 | 0 | 6 | 20 | 32 | 26 |
| 142 | 5 | 6 | 16 | 33 | 27 |
| 143 | 0 | 6 | 24 | 36 | 30 |
| 144 | 20 | 6 | 4 | 36 | 30 |
| 145 | 5 | 6 | 20 | 37 | 31 |
| 146 | 25 | 6 | 0 | 37 | 31 |
| 147 | 10 | 6 | 16 | 38 | 32 |
| 148 | 15 | 6 | 12 | 39 | 33 |
| 149 | 0 | 6 | 28 | 40 | 34 |
| 150 | 5 | 6 | 24 | 41 | 35 |
| 151 | 25 | 6 | 4 | 41 | 35 |
| 152 | 10 | 6 | 20 | 42 | 36 |
| 153 | 30 | 6 | 0 | 42 | 36 |
| 154 | 15 | 6 | 16 | 43 | 37 |
| 155 | 0 | 6 | 32 | 44 | 38 |
| 156 | 5 | 6 | 28 | 45 | 39 |
| 157 | 25 | 6 | 8 | 45 | 39 |
| 158 | 10 | 6 | 24 | 46 | 40 |
| 159 | 30 | 6 | 4 | 46 | 40 |
| 160 | 15 | 6 | 20 | 47 | 41 |
| 161 | 35 | 6 | 0 | 47 | 41 |
| 162 | 0 | 6 | 36 | 48 | 42 |
| 163 | 20 | 6 | 16 | 48 | 42 |
| 164 | 0 | 8 | 0 | 16 | 8 |
| 165 | 0 | 8 | 4 | 20 | 12 |
| 166 | 5 | 8 | 0 | 21 | 13 |
| 167 | 0 | 8 | 8 | 24 | 16 |
| 168 | 5 | 8 | 4 | 25 | 17 |
| 169 | 5 | 8 | 8 | 29 | 21 |
| 170 | 10 | 8 | 4 | 30 | 22 |
| 171 | 15 | 8 | 0 | 31 | 23 |
| 172 | 0 | 8 | 16 | 32 | 24 |
| 173 | 5 | 8 | 12 | 33 | 25 |
| 174 | 10 | 8 | 8 | 34 | 26 |
| 175 | 15 | 8 | 4 | 35 | 27 |
| 176 | 0 | 8 | 20 | 36 | 28 |
| 177 | 5 | 8 | 16 | 37 | 29 |
| 178 | 10 | 8 | 12 | 38 | 30 |
| 179 | 15 | 8 | 8 | 39 | 31 |
| 180 | 0 | 8 | 24 | 40 | 32 |
| 181 | 20 | 8 | 4 | 40 | 32 |
| 182 | 5 | 8 | 20 | 41 | 33 |
| 183 | 10 | 8 | 16 | 42 | 34 |
| 184 | 0 | 8 | 28 | 44 | 36 |
| 185 | 20 | 8 | 8 | 44 | 36 |
| 186 | 25 | 8 | 4 | 45 | 37 |
| 187 | 10 | 8 | 20 | 46 | 38 |
| 188 | 0 | 8 | 32 | 48 | 40 |
| 189 | 0 | 10 | 0 | 20 | 10 |
| 190 | 0 | 10 | 4 | 24 | 14 |
| 191 | 5 | 10 | 0 | 25 | 15 |
| 192 | 0 | 10 | 8 | 28 | 18 |
| 193 | 0 | 10 | 12 | 32 | 22 |
| 194 | 10 | 10 | 4 | 34 | 24 |
| 195 | 15 | 10 | 0 | 35 | 25 |
| 196 | 0 | 10 | 16 | 36 | 26 |
| 197 | 15 | 10 | 4 | 39 | 29 |
| 198 | 0 | 10 | 20 | 40 | 30 |
| 199 | 5 | 10 | 16 | 41 | 31 |
| 200 | 15 | 10 | 8 | 43 | 33 |
| 201 | 0 | 10 | 24 | 44 | 34 |
| 202 | 5 | 10 | 20 | 45 | 35 |
| 203 | 25 | 10 | 0 | 45 | 35 |
| 204 | 10 | 10 | 16 | 46 | 36 |
| 205 | 15 | 10 | 12 | 47 | 37 |
| 206 | 0 | 10 | 28 | 48 | 38 |
| 207 | 20 | 10 | 8 | 48 | 38 |
| 208 | 0 | 12 | 0 | 24 | 12 |
| 209 | 0 | 12 | 4 | 28 | 16 |
| 210 | 5 | 12 | 0 | 29 | 17 |
| 211 | 5 | 12 | 4 | 33 | 21 |
| 212 | 10 | 12 | 0 | 34 | 22 |
| 213 | 0 | 12 | 12 | 36 | 24 |
| 214 | 5 | 12 | 8 | 37 | 25 |
| 215 | 10 | 12 | 4 | 38 | 26 |
| 216 | 0 | 12 | 16 | 40 | 28 |
| 217 | 5 | 12 | 12 | 41 | 29 |
| 218 | 10 | 12 | 8 | 42 | 30 |
| 219 | 15 | 12 | 4 | 43 | 31 |
| 220 | 0 | 12 | 20 | 44 | 32 |
| 221 | 20 | 12 | 0 | 44 | 32 |
| 222 | 5 | 12 | 16 | 45 | 33 |
| 223 | 10 | 12 | 12 | 46 | 34 |
| 224 | 15 | 12 | 8 | 47 | 35 |
| 225 | 0 | 12 | 24 | 48 | 36 |
| 226 | 0 | 14 | 0 | 28 | 14 |
| 227 | 0 | 14 | 4 | 32 | 18 |
| 228 | 5 | 14 | 0 | 33 | 19 |
| 229 | 5 | 14 | 4 | 37 | 23 |
| 230 | 10 | 14 | 0 | 38 | 24 |
| 231 | 0 | 14 | 12 | 40 | 26 |
| 232 | 10 | 14 | 4 | 42 | 28 |
| 233 | 15 | 14 | 0 | 43 | 29 |
| 234 | 15 | 14 | 4 | 47 | 33 |
| 235 | 0 | 14 | 20 | 48 | 34 |
| 236 | 20 | 14 | 0 | 48 | 34 |
| 237 | 0 | 16 | 0 | 32 | 16 |
| 238 | 0 | 16 | 4 | 36 | 20 |
| 239 | 5 | 16 | 0 | 37 | 21 |
| 240 | 5 | 16 | 4 | 41 | 25 |
| 241 | 10 | 16 | 0 | 42 | 26 |
| 242 | 0 | 16 | 12 | 44 | 28 |
| 243 | 5 | 16 | 8 | 45 | 29 |
| 244 | 10 | 16 | 4 | 46 | 30 |
| 245 | 0 | 18 | 0 | 36 | 18 |
| 246 | 0 | 18 | 4 | 40 | 22 |
| 247 | 5 | 18 | 0 | 41 | 23 |
| 248 | 0 | 18 | 8 | 44 | 26 |
| 249 | 5 | 18 | 4 | 45 | 27 |
| 250 | 10 | 18 | 0 | 46 | 28 |
| 251 | 0 | 18 | 12 | 48 | 30 |
| 252 | 0 | 20 | 0 | 40 | 20 |
| 253 | 0 | 20 | 4 | 44 | 24 |
| 254 | 5 | 20 | 0 | 45 | 25 |
| 255 | 0 | 20 | 8 | 48 | 28 |
| 256 | 0 | 24 | 0 | 48 | 24 |

# Conclusions

The A-A-MAP resource allocation indication in non-user specific A-MAP IE has already proposed in this paper by removing some combinations of A-A-MAP resource allocations, which can be tabulated as follows:

- non-FFR Group 1 using QPSK 1/4, and Group 2 using QPSK 1/2;

- non-FFR Group 1 using QPSK 1/8, and Group 2 using QPSK 1/2;

- FFR Group 1 using QPSK 1/2, Group 2 using QPSK 1/8, and Group 3 using QPSK 1/2;

- FFR Group 1 using QPSK 1/2, Group 2 using QPSK 1/4, and Group 3 using QPSK 1/2.

- References

[1] P802.16m Amendment Working document, 802.16m-09/0010r2, June 1, 2009.