| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed 802.16m AWD Text for A-MAP periodicity (AWD-15.3.6.2.2) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hujun Yin, Yi Hsuan, Roshni Srinivasan, Jing Zhu, Yujian Zhang Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text a resource indexing scheme for the DL and UL and proposed text to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed 802.16m AWD Text for A-MAP periodicity

Hujun Yin, Yi Hsuan, Roshni Srinivasan, Jing Zhu, Yujian Zhang

Intel Corporation

- 1. Introduction.

In the current AWD (80216m-09_0010r2), A-MAP regions can be located in every DL subframes (n=1) or every two DL subframes (n=2). While providing some flexibility on A-MAP periodicity, this design increases implementation complexity. Down-selection of the n values is preferred if there is no or little impact on the overall system performance. We evaluate the impact of different n values from the aspect of co-existence, HARQ timing, AMAP IE, micro-sleep, and non-user specific overhead. The comparison results show that n=1 is the preferred value for A-MAP periodicity.

2. Comparison of n=1 and n=2

Co-existence consideration:

- MAC co-existence allows multiple radios (WiFi/MiMAX/BT) operate at different time slots

- n=1 allows better scheduling flexibility in time domain

- n=1 is preferred for co-existence

HARQ Timing consideration:

- A-MAP overhead:

  - DL HARQ, when n=2, 1 bit is needed in A-MAP to indicate which DL subframe is targeted.

  - UL HARQ: when n=2, 1 more bit might be needed to indicate UL subframe.

- Latency:

  - Take UL HARQ as an example, some DL subframes with small latency might be excluded in n=2 case. For TDD 5:3, green field, 3 subframe processing time case (the bottom row of the figure below), latency for subframe#5 is 1 when n=1. If n=2, the DL subframe should be either #0 or #2, where the latency is 2 and 3, respectively

- UL HARQ ACK indexing

  - Easier as AMAP is available every subframe

- n=1 is perferred for HARQ timing

A-MAP IE consideration:

- n=1 is more friendly for resource indexing as resource relevance is simplified

- ACKCH indexing is more straight forward due to continuous availability of AMAP

- n=1 is preferred for AMAP IE design

Non-user specific A-MAP overhead:

- n=1 has more non-user specific overhead than n=2

- n=1 non-user specific overhead ~1/2 PRU/subframe (2% @5MHz, 1%@10MHz, 0.5% @20MHz)

- n=2 non-user specific overhead~1/4 PRU/subframe (1% @5MHz, 0.5% @10MHz, 0.25% @20MHz)

- n=2 has 0.25~1% overhead saving

Micro-sleep consideration:

- When n=2, terminal can go sleep in 2nd subframe if AMAP is not detected in the 1st subframe

- However, AMAP is FDMed with data. Processing delay will be 3+ symbols, the Micro-sleep time will be small

- n=2 does not have significant gain over n=1 in power save

- New mechanism is needed to support micro-sleep

In conclusion,

- n=1 can support better flexibility for co-existence

- n=1 can simplify HARQ timing and AMAP IE design

- n=1 and n=2 have small difference in supporting micro-sleep, MAC level micro-sleep is needed

- n=2 can reduce non-user specific AMAP overhead (<1%)

- n=1 is preferred for down selection

- Amendment text proposal for inclusion in 802.16m-09/0010r2

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ --------- Recommended AWD Text Proposal (page 185)------------]

# Advanced MAP (A-MAP)

The Advanced MAP (A-MAP) carries unicast service control information. Unicast service control information consists of user-specific control information and non-user-specific control information. User-specific control information is further divided into assignment information, HARQ feedback information, and power control information, and they are transmitted in the assignment A-MAP, HARQ feedback A-MAP, and power control A-MAP, respectively. All the A-MAPs share a region of physical resources called A-MAP region.

A-MAP regions shall be located in all DL subframes. DL data allocations corresponding to an A-MAP region occupy resources in the subframe where the A-MAP region is located.

A-MAP regions shall be located Nsubframe, A-MAP subframes apart in a frame. If an A-MAP region is allocated in subframe i, the next A-MAP region shall be allocated in subframe i+Nsubframe, A-MAP. In particular, for a frame with Nsubframe,DL DL subframes, A-MAP regions shall be present in subframe i, where i=j* Nsubframe, A-MAP and j is any non-negative integer such that j* Nsubframe, A-MAP< N subframe,DL. DL data allocations corresponding to the A-MAP region can correspond to resources in any subframes between successive A-MAP regions.  The values of Nsubframe, A-MAP can be 1 or 2. Other values of Nsubframe, A-MAP (3 and 4) are FFS. For example, for Nsubframe, A-MAP =2, an A-MAP region in subframe i can point to resource allocation in subframe i or i+1 and the next A-MAP region is in subframe i+2.

Figure 472 illustrates the location of a MAP region for Nsubframe, A-MAP=1 and 2 cases in the TDD mode.

- Figure 472: Example on the Location of A-MAP regions in a TDD system with a 4:4 subframe DL:UL split

-------------------------------  End Text Proposal ---------------------------------------------------