| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for the IEEE 802.16m AWD on HF-A-MAP Indexing (AWD-15.3.6) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Yujian Zhang, Yi Hsuan, Hujun Yin Intel Corporation | +86-10-82611515 {yujian.zhang, yi.hsuan, hujun.yin}@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | Proposed text for HF-A-MAP indexing in P802.16m Amendment Working Document |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the IEEE 802.16m AWD on HF-A-MAP Indexing (AWD-15.3.6)

Yujian Zhang, Yi Hsuan, Hujun Yin

Intel Corporation

- Introduction

The contribution proposes the text on HF-A-MAP indexing for 802.16m amendment [1]. Detailed design is given in [2].

- References

[1] IEEE 802.16m-09/0010r2, “802.16m Amendment Working Document (AWD)”

[2] IEEE 802.16m-09/1366, “HARQ Feedback Channel Indexing”

- Text proposal for inclusion in the 802.16m amendment

------------------------------------------- Start of Proposed Amendment Text -----------------------------------------------

### 15.3.6.3.2.3 HARQ Feedback A-MAP

…

Each HF-A-MAP IE carries 1 bit information. Depending on the channel conditions, the modulation can be QPSK or BPSK. If QPSK is used, 2 HF-A-MAP IEs are mapped to a point in the signal constellation. If BPSK is used, each HF-A-MAP IE is mapped to a point in the signal constellation. The repetition number is FFS.

For persistent allocation, HF-A-MAP resource index is specified in HFA of UL Individual Persistent A-MAP IE or UL Composite Persistent A-MAP IE.

For group resource allocation, HF-A-MAP resource index for the l-th AMS in GRA allocation is(istart+l∗NHF−A−MAPNGRA)modNHF−A−MAP, where istart is the ACK Channel Offset in UL group resource allocation A-MAP IE, NHF-A-MAP is the total number of HF-A-MAP configured, and NGRA is the Use Bit Map Size in UL group resource allocation A-MAP IE.

For resource allocation using UL basic assignment A-MAP, HF-A-MAP resource index is (M(j)+n)modNHF−A−MAP, where j is HF-A-MAP Index Parameter in Non-user specific A-MAP IE, n is HFA in UL basic assignment A-MAP IE, M(j) is STID when j=0 and M(j) is lowest LRU index of corresponding UL transmission when j=1.

…

### 15.3.6.5.2.1 Non-user-specific A-MAP IE

Non-user-specific A-MAP IE consists of information that is not dedicated to a specific user or a specific group of users. It includes information required to decode assignment A-MAP IE The number of assignment A-MAPs in each assignment A-MAP group is indicated in the Non-user-specific A-MAP IE. A-MAP IE also includes HF-A-MAP Index Parameter, which is used to indicate which transmission parameter is used to calculate HF-A-MAP index. The detailed information included in non-user specific information is TBD. The non-user specific A-MAP IE is shown in Table 666.

Table 666—Non-user specific A-MAP IE

| Syntax | Size [bits] | Notes |
| --- | --- | --- |
| Assignment A-MAP size | TBD | Indicate the number of assignment A-MAPs in each assignment A-MAP group as shown in Table 667. |
| HF-A-MAP Index Parameter | 1 | Indicate which transmission parameter is used to calculate HF-A-MAP index. |

--------------------------------------------- End of Proposed Amendment Text ----------------------------------------------