| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD-15.2.3 Simplified MAC PDU Formats |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Baowei Ji, Farooq Khan, Zhouyue Pi, Ying Li Samsung Telecommunications America | bji@sta.samsung.com +1-972-761-7167 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | Propose to streamline the MAC PDU format design using only GMH and 4 types of extended headers. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document (AWD) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Simplified MAC PDU Formats

Baowei Ji, Farooq Khan, Zhouyue Pi, Ying Li

Samsung Telecommunications America

- Introduction

In the AWD, as many as 8 header formats are specified such as GMH, CMH, EH, FPEH, FEH, MEH, MEHB, First MEHB. This complicates the system implementation and limits spectral efficiency. Interestingly, even this number of headers are still not able to support features such as fragmentation, polling for ARQ feedback, and the differentiation of ARQ data from ARQ feedback.

A streamline design would be to use FlowID to differentiate FPEH from FEH, so there is no need for EH types. One GMH could point at another GMH, so there is no need for MEH, MEHB or Frist MEHB.

- Suggested proposal

We propose:

- To use only three header formats: GMH, FPEH, andFEH.

- FPEH is for ARQ, FEH is for MAC management messages. They are differentiated by FlowID.

- One GMH could point at the next GMH if multiple FlowIDs presence in one data burst.