| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AWD-15.2.16.2.1 Specification of the Initial Sleep Window |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Baowei Ji Samsung Telecommunication America Yeongmoon Son Samsung Electronics Co., Ltd. | bji@sta.samsung.com +1-972-761-7167 |
| Re: | 802.16m AWD: IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” |  |
| Abstract | To analyze the lack of specification of initial sleep window and to propose two alternatives for TGm to consider |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document (AWD) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Specification of the Initial Sleep Window

Baowei Ji, Yeongmoon Son

Samsung Electronics

- Introduction

The AWD text correctly states that “The first sleep cycle on entry to Sleep Mode from Active Mode does not contain a Listening Window”. But, it does not specify the length of the initial sleep window. This could lead to problems given that sleep window parameter is not included in the sleep control message either.

- Suggested proposal

We propose two alternative proposals for TGm to consider.

Proposal I: (the initial sleep window) = (the Initial Sleep Cycle) – (the Default Listening Window)

As shown in Figure 1, the initial sleep window equals to the initial sleep cycle minus the default listening window. The next sleep cycle begins from the default listening window, if there is traffic or signaling in DL and/or UL, the sleep cycle equals to the initial sleep cycle. Otherwise, the sleep cycle equals to twice of the initial sleep cycle. In both cases, there is no ambiguity in sleep mode operation.

Figure 1: Initial sleep window (ISW) equal to initial sleep cycle (ISC) minus default listening window (DLW)

Proposal II: (the initial sleep window) = (the Initial Sleep Cycle)

As shown in Figure 2, the initial sleep window equals to the initial sleep cycle. The next sleep cycle begins from the default listening window, if there is traffic or signaling in DL and/or UL, the sleep cycle equals to the initial sleep cycle. Otherwise, the sleep cycle equals to twice of the initial sleep cycle. In both cases, there is no ambiguity in sleep mode operation.

Figure 2: Initial sleep window (ISW) equal to initial sleep cycle (ISC)

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Page 89, line 34, Section 15.2.16.2.1]

The period of the Sleep Cycle is measured in units of frames. A sleep cycle is the sum of a Sleep Window and a Listening Window. The first sleep cycle on entry to Sleep Mode from Active Mode does not contain a Listening Windowcontains only a sleep window which equals to the Initial Sleep Cycle minus the Default Listening Window.

[Alternative proposal]

The period of the Sleep Cycle is measured in units of frames. A sleep cycle is the sum of a Sleep Window and a Listening Window. The first sleep cycle on entry to Sleep Mode from Active Mode does not contain a Listening Windowcontains only a sleep window which equals to the Initial Sleep Cycle.

[Replace subclause 15.2.17.1:]

-------------------------------  Text End  ---------------------------------------------------