| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed AWD Text of Interference Avoidance and Interference Mitigation for Femtocell |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Yung-Han Chen, Chie Ming Chou, Fang-Ching (Frank) Ren, Chun-Yen Wang, Ming-Hung Tao, Ying-Chuan Hsiao ITRI | chenyunghan@itri.org.tw MHTao@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for contribution: Support for Femtocell BS (80216m-09_0028r1.doc) |  |
| Abstract | This contribution illustrates proposed text of interference avoidance and interference mitigation for Femtocell environment. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed AWD Text of Interference Avoidance and Interference Mitigation for Femtocell

Yung-Han Chen, Chie Ming Chou, Fang-Ching (Frank) Ren, Chun-Yen Wang

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

1. Introduction

In femtocell network, the number of femtocell BS is huge and licensed bandwidth of operator is limited. A femtocell BS uses the same carrier frequency with macro ABS is an inevitable solution. Unfortunately, when same carrier frequency is used, huge interference may be introduced. As shown in figure 1, there are two worst cases:

(1) When the location of femtocell BS is close to a macro ABS, the DL signal power from the macro ABS may be large enough to interfere with the DL signal in a femtocell. For example in Figure 1, MS1 served by femtocell BS1 may suffer strong interference from macro ABS (the signal transmitted to MS2). This case can be detected during femtocell BS’s initialization.

(2) When an AMS served by a macro ABS is near to a femtocell BS, its UL signal may interfere with the UL signal of the femtocell. For example in Figure 1, MS3 served by the macro BS will cause UL interference to the femtocell BS. Besides, an AMS served by the macro BS may also generate significant UL interference to the femtocell BS even thought it is not near to the femtocell, especially when the AMS is far from the macro BS and has to increase transmission power. This case could happen dynamically and could be dependent on the location of AMS served by the overlapped macro ABS.

Figure 1. Interference when femtocell BS uses the same carrier frequency with macro ABS

To well support interference avoidance and mitigation, we propose a flexible radio resource reservation and sharing method to achieve optimum resources utilization.

2. Proposal

The concept of reserving radio resources from macro cell for Femtocell BS’s usage is proposed to reduce the interference between a macro ABS and a Femtocell BS. A Femtocell zone is a region which was originally used by a macro ABS. When a newly deployed or operating Femtocell BS finds that there is no clear channel to provide a good transmission for its serving AMSs or serving AMSs feed back the complaint messages about interference, the Femtocell BS will request neighbor macro ABS to reserve its resource(s) for Femtocell BS’s transmission. Once the macro ABS agrees to grant this region to Femtocell BS, the macro ABS would not allocate the resource in this region to its AMSs. On the other hand, a Femtocell BS could start to schedule the transmission in this region after receiving the agreement. The Femtocell zone might be shared with multiple Femtocell BSs to acquire the spatial multiplexing benefit. If there is no AMSs under Femtocell BS’s coverage or interference problem has been resolved, then the Femtocell zone could be released to macro ABS.

A Femtocell BS places an indicator in its SFH to indicate the allocation of Femtocell zone. This can avoid AMS reading signaling from other BSs. Inside the Femtocell zone, an associated A-MAP will be given to specify the corresponding data location for individual AMSs. From AMS’s point of view, it reads the broadcast message first and knows the allocation of Femtocell zone. Then the AMS could decode and acquire its individual data based on the indictor in the Femtocell zone. Figure 2 is an example of control structure of Femtocell zone.

Figure 2. The control structure of Femtocell zone

Furthermore, during Femtocell BS’s SON or configuration phase, a Femtocell BS may coordinate with macro ABS to reserve a radio resource region for putting its broadcast control signaling such as SFH. The A-Preamble can be used to derive the resource block arrangements.

3. Suggested AWD Text

--------------------------------------------------------  Text Start  ------------------------------------------------------------

# 15.x Support for Femtocell BS

## 15.x.y Interference Avoidance and Interference Mitigation

An MS may be requested by its serving macro BS or Femtocell BS to report the signal strength measurement of neighbor BSs, including macro and/or Femtocell BSs. The reported information can be used by the serving BS to coordinate with its neighbor BSs to mitigate the interference at the MSs. Large interference from an inaccessible Femtocell BS may trigger a nearby MS to report the interference to the serving BS, and the report information should include system information of the inaccessible Femtocell BS (e.g., BS_ID of the femtocell BS). The serving BS and/or the network may request the interfering Femtocell BS to mitigate the interference by reducing transmission power, and/or blocking some resource region. In order to enable the interference avoidance or mitigation schemes, the Femtocell BS shall be capable to scan the signals transmitted from neighbor BSs.

The interference between Femtocells and/or macro cells can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner. A Femtocell BS may detect resources occupied by other BSs and reserve the resources autonomously. The resource reservation information for a Femtocell BS is provided by S-SFH. An AMS connected to a macro ABS or Femtocell BS may detect interfered resource from surrounding Femtocells and/or macro ABSs and report to the serving BS, so that the serving BS may select appropriate resources for its traffic.

In case that a Femtocell share the same frequency partition(s) with overlay macro cell, the Femtocell BS shall inform the overlay macro BS which UL RUs are interfered and the overlay macro BS shall reschedule the interfering AMSs to other UL RUs. A macro cell AMS interfered certain BSs shall request to be rescheduled to other subframe.

In order to reduce interference on SFH of Femtocells and/or macro BSs, a Femtocell shall transmit its SFH in different time offsets with respect to the selected SA-Preamble.

A Femtocell BS may select the carrier frequency to avoid the mutual interference between macro/micro cells and Femtocells or among Femtocells based on the measurement result of surrounding reception power.

--------------------------------------------------------  Text End  ------------------------------------------------------------