| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handover Enhancements |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Stavros Tzavidas Mark Marsan Motorola | stavros.tzavidas@motorola.com |
| Re: | 802.16m amendment working document |  |
| Abstract | This contribution proposes several enhancements that are needed in the handover section. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handover Enhancements

Stavros Tzavidas, Mark Marsan

Motorola

# Introduction

The handover section in the current standard draft does not specify how to handle DSx transactions that are not completed at the time a handover is initiated (i.e. an AAI_HO-CMD message is sent by the ABS). Without clear rules on how to handle/terminate such DSx transactions, the AMS may be unable to communicate with the target ABS (T-ABS) even after a successful handover as we explain next.

Typically, during HO preparation, the S-ABS does not transmit information about pending (i.e. unfinished) DSx transactions to the target ABS(s). As a result, the AMS and the T-ABS may lose synchronization, i.e. the T-ABS may be unaware of DSx transactions that the AMS considers completed. In such cases, the T-ABS and the AMS apply different parameters to data belonging to the corresponding service flows and it becomes impossible for one side to decode the data sent by the other side, even if the data is transmitted without errors.

This situation can be rectified by following the following simple rules:

- The S-ABS shall not initiate any DSx transactions after it sends AAI_HO-CMD to the AMS.

- The AMS shall not initiate any DSx transactions after it receives AAI_HO-CMD from the S-ABS

- Both sides should ignore any DSx messages that are received after the AAI_HO-CMD is transmitted (at the ABS side) or received (at the AMS side).

- For pending DSx transactions we can distinguish the following cases, depending on which side initiated the transaction and which messages have been received by the ABS:

| ABS-initiated DSx transactions (ABS sent DSx-REQ) |  |
| --- | --- |
| If… | The DSx transaction shall be considered… |
| DSx-REQ sent but DSx-RSP not received | Failed |
| DSx-RSP received | Complete. The AAI_HO-CMD shall contain the status indication that would normally be included in DSx-ACK (or it may repeat the information sent in DSx-ACK, if the ACK has already been transmitted). |
| AMS-initiated DSx transactions (AMS sent DSx-REQ) |  |
| If… | The DSx transaction shall be considered… |
| DSx-REQ not received by the ABS | Failed. The ABS does not include the transaction ID in the AAI_HO-CMD. |
| DSx-REQ received but DSx-RSP not yet transmitted. | Complete. The AAI_HO-CMD shall contain the status information that would normally be included in DSx-RSP. DSx-ACK is not required. |
| DSx-RSP transmitted but DSx-ACK not received yet. | Complete. The AAI_HO-CMD shall repeat the status information that was included in DSx-RSP. DSx ACK is not required. |

# Text Proposal

----------------------------------------------Start of the text proposal ------------------------------------------------------

Modify section 15.2.5.3 “AAI_HO-CMD” as follows:

15.2.5.3 AAI_HO-CMD

The S-ABS shall send AAI_HO-CMD to initiate the HO procedure, or to acknowledge the AAI_HO-REQ sent by the AMS. The AAI_HO-CMD message serves one of the following purposes.

• HO command

• HO rejection in response of AAI_HO-REQ from AMS

In case of HO command function, AAI_HO-CMD message should include one or more target ABS. For each target ABS, the following parameters are included:

- Target ABS preamble index

- Optional T-ABS MAC identity if preamble index has ambiguity (placeholder for dense overlay deployment or femtocell deployment)

- HO_Reentry_Mode

- Action Time

- Disconnect Time (if HO_Reentry_Mode=1, disconnect time > action time; if HO_Reentry_Mode=0, Disconnect Time <= Action Time)

- Optional EBB configuration if HO_Reentry_Mode = 1

  - HO_Reentry_interval

- Resource_Retain_Time

- CDMA_RNG_FLAG

- Optional dedicated CDMA ranging code/opportunity if CDMA_RNG_FLAG=1

- HO optimization related context, e.g., Indication of which components of the AMS' static and dynamic context are available at the target ABS.

- Ranging initiation deadline.

- Service level prediction, which indicates the level of service the MS can expect from this BS. The following encodings apply:

- 0 = No service possible for this MS

- 1 = Some service is available for one or several service flows authorized for the MS or No service level prediction available.

- 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet.

- 3 = No service level prediction available.

In case of HO command function, AAI_HO-CMD message shall include a status indication for every pending DSx transaction. For each pending DSx transaction, the ABS includes the transaction ID and a status flag. If the status flag is 1 the AMS shall consider the DSx transaction as successfully completed. Otherwise the AMS shall consider the DSx transaction as failed.

In case of HO rejection function, AAI_HO-CMD message shall not include any target ABS.

Insert the following subsection in section 15.2.6.3.3 “HO Preparation”:

### 15.2.6.3.3.1 Handling of DSx transactions during HO

The serving ABS shall not initiate any DSx transactions after sending the AAI_HO-CMD message. The AMS shall not initiate any DSx transactions after receiving the AAI_HO-CMD message.

The ABS shall ignore any DSx messages received after sending the AAI_HO-CMD message. The AMS shall ignore any DSx messages received after receiving the AAI_HO-CMD message.

When sending the AAI_HO-CMD message to the AMS, the Serving ABS shall include the status of all pending DSx transactions as described in section 15.2.5.3. The Serving ABS may also include the status of recently completed DSx transactions.

Upon receiving the AAI_HO-CMD message the AMS shall decide on the status of pending DSx transactions according to the following rules:

- If the transaction ID of a pending DSx transaction is not included in AAI_HO-CMD, the transaction shall be considered as failed.

- The AMS shall ignore any information contained in AAI_HO-CMD for transactions that have been completed (DSx-ACK received prior to receiving AAI_HO-CMD).

- If the status of a pending transaction in the AAI_HO-CMD message is set to 1 (success) the transaction shall be considered complete. For complete transactions, no further control messages shall be required. Otherwise, the transaction shall be considered as failed.

-------------------------------------------------- End of the text proposal --------------------------------------------------