| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Description of MIMO Encoding Format for CDR |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics Kiran Kuchi CEWIT | d.mazzarese@samsung.com kkuchi@cewit.org.in *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) and 15.3.6.5 DL Control Information Elements |  |
| Abstract | This contribution proposes to define the accepted conjugate data repetition scheme as a new MIMO Encoder Format. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Description of MIMO Encoding Format for CDR

David Mazzarese et Al.

Samsung Electronics

# Introduction

A conjugate data repetition scheme has been adopted by comment 416 in database 80216m-09/0023r3 at session 61. Although this comment is not reflected in 80216m-09/0010r2 for reasons explained in 80216m-09/0010r2, it needs a more appropriate description in the specifications. Due to the fact that this scheme is adding a symbol repetition that is not reflected in the coding rates of the MCS levels in Table 744, this effect should be taken into account in the STC rate. Due to the similarity between this scheme and SFBC, we propose to add a new MIMO Encoding Format to describe CDR, and thus to map the conjugated symbol to an adjacent subcarrier. A new Downlink MIMO mode is also needed (MIMO mode 5).

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Insert section “15.3.7.1.1.4 CDR encoding” on line 36 page 232 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.1.1.4 CDR encoding

The input to the MIMO encoder is represented by a 1 × 1 vector.

s = s1	(203)

The MIMO encoder generates the SFBC matrix.

(204)

The CDR matrix, x, occupies two consecutive subcarriers.

-------------------------------  Text End  ---------------------------------------------------

<Modify section “15.3.7.1.3 Downlink MIMO modes” on line 15 page 234 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.1.3 Downlink MIMO modes

There are five six MIMO transmission modes for unicast DL MIMO transmission as listed in Table 687.

Table 687 – Downlink MIMO modes

| Mode index | Description | MIMO encoding format (MEF) | MIMO precoding |
| --- | --- | --- | --- |
| Mode 0 | OL SU-MIMO (Tx diversity) | SFBC | non-adaptive |
| Mode 1 | OL SU-MIMO (SM) | VE | non-adaptive |
| Mode 2 | CL SU-MIMO (SM) | VE | adaptive |
| Mode 3 | OL MU-MIMO (SM) | HE | non-adaptive |
| Mode 4 | CL MU-MIMO SM) | HE | adaptive |
| Mode 5 | OL SU-MIMO (Tx diversity) | CDR | non-adaptive |

-------------------------------  Text End  ---------------------------------------------------

<Add a section on line 50 page 237 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.3.4 MIMO mode 5

Null tones are inserted in the interlaced pilot locations occupied by the other sectors whose cell_ID is distinct from the serving cell_ID. Data mapping skips pilot tones and nulls tones.

-------------------------------  Text End  ---------------------------------------------------

<Modify Table 688 on line 50 page 234 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 688 – DL MIMO parameters

|  | Number of transmit antennas | STC rate per layer | Number of streams | Number of subcarriers | Number of layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |
| MIMO mode 3 and MIMO mode 4 | 2 | n.a. | 2 | 1 | 2 |
|  | 4 | n.a. | 2 | 1 | 2 |
|  | 4 | n.a. | 3 | 1 | 3 |
|  | 4 | n.a. | 4 | 1 | 4 |
|  | 8 | n.a. | 2 | 1 | 2 |
|  | 8 | n.a. | 3 | 1 | 3 |
|  | 8 | n.a. | 4 | 1 | 4 |
| MIMO mode 5 | 2 | 1/2 | 1 | 2 | 1 |
|  | 4 | 1/2 | 1 | 2 | 1 |
|  | 8 | 1/2 | 1 | 2 | 1 |

-------------------------------  Text End  ---------------------------------------------------

<Modify section “15.3.7.2.1.1 Encoding of MIMO modes” on line 6 page 236 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2.1.1 Encoding of MIMO modes

15.3.7.2.1.1.1 MIMO mode 0

SFBC encoding of section “SFBC encoding” on page 231 shall be used with MIMO mode 0.

15.3.7.2.1.1.1 MIMO mode 1

Vertical encoding of section “Vertical encoding” on page 231 shall be used with MIMO mode 1. The number of streams is Mt≤min(Nt,Nr), where Nr is the number of receive antennas and Mt is no more than 8.

15.3.7.2.1.1.1 MIMO mode 2

Vertical encoding of section “Vertical encoding” on page 231 shall be used with MIMO mode 2. The number of streams is Mt≤min(Nt,Nr), where Mt is no more than 8.

15.3.7.2.1.1.1 MIMO mode 5

CDR encoding shall be used with MIMO mode 5.

-------------------------------  Text End  ---------------------------------------------------

<Modify Table 689 on line 2 page 238 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 689—Supported Permutation for each DL MIMO mode

|  | DRU | Mini-band based CRU (diversity allocation) | Mini-band and Subband based CRU (localized allocation) |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes | Yes | No |
| MIMO mode 1 | Yes, with Mt=2 | Yes | Yes |
| MIMO mode 2 | No | Yes, with Mt=1 | Yes |
| MIMO mode 3 | No | No | Yes |
| MIMO mode 4 | No | Yes | Yes |
| MIMO mode 5 | No | Yes | Yes |

-------------------------------  Text End  ---------------------------------------------------

<Modify the row corresponding to parameter MEF in Table 668 on line 50 page 198 as follows>

-------------------------------  Text Start  ---------------------------------------------------

Table 668: DL Basic Assignment A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| DL-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | DL Basic Assignment A-MAP IE |
| MCS | [4] | Depends on supported modes, 16 modes assumed as baseline |
| MEF | 2 | MIMO encoder format 0b00: SFBC 0b01: Vertical encoding 0b10: Horizontal encoding 0b11: n/a CDR |

-------------------------------  Text End  ---------------------------------------------------