| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean-up of encoding and precoding sections of downlink SU MIMO and MU MIMO modes |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | Clean-up of encoding and precoding sections of downlink SU MIMO and MU MIMO modes |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Clean-up of encoding and precoding sections of downlink SU MIMO and MU MIMO modes

David Mazzarese et Al.

Samsung Electronics

# Introduction

Headers of level 7 for MIMO mode 0, MIMO mode 1 and MIMO mode 2 under subclauses "15.3.7.2.1.1 Encoding of MIMO modes" and "15.3.7.2.1.2 Precoding of MIMO modes" are not necessary. The short paragraphs that they contain are self-explanatory. The same is true for the MU-MIMO section.

We propose to replace the headers of level 7 by bullets.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.7.2 Transmission schemes for data channels” on line 4 page 236 as follows>

-------------------------------  Text Start  ---------------------------------------------------

15.3.7.2 Transmission schemes for data channels

15.3.7.2.1 Encoding, and precoding and mapping of SU-MIMO modes

15.3.7.2.1.1 Encoding of SU-MIMO modes

15.3.7.2.1.1.1 MIMO mode 0

- MIMO mode 0: SFBC encoding of section “SFBC encoding” on page 231 shall be used with MIMO mode 0.

15.3.7.2.1.1.1 MIMO mode 1

- MIMO mode 1: Vertical encoding of section “Vertical encoding” on page 231 shall be used with MIMO mode 1. The number of streams is Mt ≤ min(Nt,Nr), where Nr is the number of receive antennas and Mt is no more than 8.

15.3.7.2.1.1.1 MIMO mode 2

- MIMO mode 2: Vertical encoding of section “Vertical encoding” on page 231 shall be used with MIMO mode 2. The number of streams is Mt ≤ min(Nt,Nr), where Mt is no more than 8.

15.3.7.2.1.2 Precoding of SU-MIMO modes

15.3.7.2.1.2.1 MIMO mode 0

- MIMO mode 0: Non-adaptive precoding of section “Non-adaptive precoding” on page 232 with Mt=2 streams shall be used with MIMO mode 0.

15.3.7.2.1.2.1 MIMO mode 1

- MIMO mode 1: Non-adaptive precoding of section “Non-adaptive precoding” on page 232 with Mt streams shall be used with MIMO mode 1.

15.3.7.2.1.2.1 MIMO mode 2

- MIMO mode 2: Adaptive precoding of section “Adaptive precoding” on page 234 shall be used with MIMO mode 2.

15.3.7.2.2 Encoding, and precoding and mapping of MU-MIMO modes

15.3.7.2.2.1 Encoding of MU-MIMO modes

Multi-user MIMO schemes are used to enable a resource allocation to communicate data to two or more MSs. Multi-user transmission with one stream per user is supported for MU-MIMO.

MU-MIMO includes the MIMO configuration of 2Tx antennas to support up to 2 MSs, and 4Tx or 8Tx antennas to support up to 4 MSs, with 1 stream per MS.

Both OL MU-MIMO (mode 3) and CL MU-MIMO (mode 4) are supported with the following encoding:

15.3.7.2.2.1 Encoding of MIMO mode 3

- MIMO mode 3: Horizontal encoding of section “Horizontal encoding” on page 232 shall be used with MIMO mode 3.

15.3.7.2.2.2 Encoding of MIMO mode 4

- MIMO mode 4: Horizontal encoding of section “Horizontal encoding” on page 232 shall be used with MIMO mode 4.

15.3.7.2.2.32 Precoding of MU-MIMO modes

15.3.7.2.2.3.1 MIMO mode 3

- MIMO mode 3

Non-adaptive precoding of section 15.3.7.1.2.1 shall be used with MIMO mode 3.

Inside the OL region with OL MU MIMO, the precoder W with 2 streams is predefined and fixed over time.

Outside the OL region with OL MU MIMO, the precoder W is an NtxMt sub-matrix of a predefined NtxMaxMt matrix.

The precoding matrix W used by the BS is represented in Equation (207).

W(k)=[v1(k)v2(k)⋯vM(k)]								(207)

Where vi(k) is the precoding vector for the i-th MS on the k-th subcarrier.

vi(k) shall be used for precoding the pilot symbols on the i-th pilot stream on the k-th subcarrier.

15.3.7.2.2.3.2 MIMO mode 4

- MIMO mode 4

Adaptive precoding of section 15.3.7.1.2.2 shall be used with MIMO mode 4.

In CL MU MIMO, the precoder W is an Nt × M matrix for each subcarrier. It is used to communicate to MMSs simultaneously. The form and derivation of the precoding matrix does not need to be known at the MS. The BS determines the precoding matrix based on the feedback received from the MS.

The BS shall construct the precoding matrix W as represented in Equation (208).

W(k)=[v1(k)v2(k)⋯vM(k)]								(208)

Where, vi(k) is the precoding vector for the i-th MS on the k-th subcarrier. (207) (208)

-------------------------------  Text End  ---------------------------------------------------