| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to Table 692 (DL MIMO control parameters) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | Implementation issue relative to comment 280 in database 80216m-09_0023r3, where contribution C80216m-09/1057r1 was accepted. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to the Table 692 (DL MIMO control parameters)

David Mazzarese et Al.

Samsung Electronics

# Introduction

This is an editorial implementation issue relative to comment 280 in database 80216m-09_0023r3, where contribution C80216m-09/1057r1 was accepted. An incomplete set of instructions in C80216m-09/1057r1 did not allow the editor to identify the location of the proposed changes.

The changes proposed for Table 692 in C80216m-09/1057r1 are proposed again in this contribution with the right markup format.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Modify Table 692 on line 7 page 242 as follows>

Table 692—DL MIMO control parameters

| Parameters | Description | Value | Control Channel (IE) |  | Notes |
| --- | --- | --- | --- | --- | --- |
| Broadcast Information |  |  |  |  |  |
| Nt | Number of transmit antennas at the BS | 2, 4, 8 |  | SFH (system information) | Nt must be known before decoding the DL A-MAP IE |
| OL_Region [TBD] | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) To indicate the location of the OL region and the number of streams (1 or 2) | TBD |  | Additional broadcast Broadcast information | OL MIMO transmission region with improved intercell interference statistics |
| - SU_CT | - SU base codebook type |  |  | - Broadcast information | - SU base codebook subset indication |
| - MU_CT (TBD) | - MU base codebook type |  |  | - Broadcast information | - MU base codebook subset indication |
| - BC_SI | - Rank-1 base codebook subset indication for interference mitigation with PMI coordination | - BitMAP - (Same size as rate-1 codebook for each number of transmit antenna) - 8 bits if Nt=2 - 16 bits if Nt=4 - 8 bits if Nt=8 |  | - Additional broadcast Broadcast information | - Rate-1 codebook element restriction/recommendation information - It shall be ignored if CCE CM = 0b00, 0b01 or 0b10 |
| - MaxMt (TBD) | - Maximum number of streams | - 0b00: 2 - 0b01: 3 - 0b10: 4 - 0b11: reserved |  | - Broadcast information | - If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| Unicast Information |  |  |  |  |  |
| - MEF | - MIMO encoder format | - 0b00: SFBC - 0b01: Vertical encoding - 0b10: Horizontal encoding - 0b11: n/a |  | - A-MAP IE (unicast) | - MIMO encoder format. - [MEF bitfield may not be explicitly indicated in DL A-MAP IE] |
| - Mt | - Number of streams in transmission | - 1 to 8 - 0b000: 1 - 0b001: 2 - 0b010: 3 - 0b011: 4 - 0b100: 5 - 0b101: 6 - 0b110: 7 - 0b111: 8 - (Mt <= Nt ) |  | - A-MAP IE (unicast) | - Number of streams in the transmission. - MEF= 0b00: Mt =2 MEF= 0b01: Mt <= 4 - [Bit-field length is variable, depending on the number of Tx at BS] |
| - RU allocation (TBD) | - RU [and stream] indicator for the burst of data | - TBD |  | - A-MAP IE (unicast) | - Refer to DL control group. |
| - SI (TBD) | - Index of pilot stream allocation | - 1 to 4 - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |  | - A-MAP IE (unicast) | - SI shall be indicated if MEF =0b010 is VE - [Bit-field length is variable, depending on the number of Tx at BS] - RU allocation and SI can be merged together depending on other DG’s decision |
| Feedback Allocation IE |  |  |  |  |  |
| - MFM | - MIMO feedback mode | - Refer to Table 691 |  | - Feedback allocation IE - (unicast) | - To decide the feedback content and related MS processing |
| - MaxMt | - Maximum number of streams | - If MFM indicates a SU feedback mode: 1, 2, 4 or 8. - If MFM indicates a MU feedback mode: 1, 2, 3, or 4. |  | - Feedback allocation IE | - If MFM indicates a SU feedback mode: the maximum number of streams scheduled for each user - If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| - DLRU (TBD) | - Downlink RU, indicating - which RUs or which type of RU (DRU or miniband-based CRU) to work on for feedback | - TBD (Tree structure, bit map etc) |  | - Feedback allocation IE (unicast) | - To process CQI (PMI) estimation for the indicated RUs. Refer to other DG |
| - FT | - MIMO feedback type | - 0b00: codebook - 0b01: sounding |  | - Feedback allocation IE (unicast) |  |
| - CS indication | - Codebook subset type for CL MIMO modes 2 and 4 | - Base codebook or codebook subset |  | - Feedback allocation IE - (unicast) | - Depending on the MFM and CS indication, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. |
| - CM | - Codebook feedback mode for CL MIMO modes 2 and 4 | - 0b00: standard mode with codebook coordination disabled - 0b01: transformation mode with codebook coordination disabled - 0b10: differential mode with codebook coordination disabled - 0b11: standard mode with codebook coordination enabled |  | - Feedback allocation IE - (unicast) | - Enabled when FT = 0b00 - In codebook-based feedback mode: - Codebook coordination disabled: MS finds PMI within whole broadcasted codebook type entry - Codebook coordination enabled: MS shall find the rate-1 PMI within broadcasted codebook entries indicated by BC_SI |
| CCE | Codebook Coordination Enable | 0b0: disable 0b1: enable |  |  | CCE = 0b0: MS finds PMI within whole broadcasted codebook type entry. CCE = 0b1: When MS finds rate-1 PMI, it finds within broadcasted codebook entries indicated by BC_ST, [SU_CT and MU_CT] |