| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to Table 705 (DL 4Tx OL SU-MIMO codebook subset) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.7 (DL-MIMO) |  |
| Abstract | An index that represents the number of bits in the notation of the DL 4Tx base codebook for ranks 3 and 4 used in Table 705 should be changed from 4 to 6. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to the Table 705 (DL 4Tx OL SU-MIMO codebook subset)

David Mazzarese et Al.

Samsung Electronics

# Introduction

An index that represents the number of bits in the notation of the DL 4Tx base codebook for ranks 3 and 4 used in Table 705 should be changed from 4 to 6. When the feedback size for the 4Tx PMI is set to 6 bits, the full base codebook is used, and 6 bits are used for reporting any rank. There is no switching between 6 bits or 4 bits depending on the rank of the PMI. So 6 bits should be used consistently.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Modify Table 705 on line 22 page 258 as follows>

Table 705 – CDL,OL,SU(4,1,4), CDL,OL,SU(4,2,4), CDL,OL,SU(4,3,2) and CDL,OL,SU(4,4,1)

| CDL,OL,SU(4,1,4,n) |  | CDL,OL,SU(4,2,4,n) |  | CDL,OL,SU(4,3,2,n) |  | CDL,OL,SU(4,4,1,n) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | C(4,1,6,m) in base codebook | n | C(4,2,6,m) in base codebook | n | C(4,3,6,m) in base codebook | n | C(4,4,6,m) in base codebook |
| 0 | C(4,1,6,8) | 0 | C(4,2,6,23) | 0 | C(4,3,46,12) | 0 | C(4,4,46,4) |
| 1 | C(4,1,6,10) | 1 | C(4,2,6,29) | 1 | C(4,3,46,13) |  |  |
| 2 | C(4,1,6,9) | 2 | C(4,2,6,27) |  |  |  |  |
| 3 | C(4,1,6,11) | 3 | C(4,2,6,25) |  |  |  |  |