| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Corrections to Table 740 (UL 4Tx OL SU-MIMO codebook subset) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | David Mazzarese, Hokyu Choi, Heewon Kang Samsung Electronics | d.mazzarese@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Category: AWD comments / Area: Chapter 15.3.10 (UL-MIMO) |  |
| Abstract | Clarification of the definition of PMI in Table 740 (UL 4Tx OL SU-MIMO codebook subset) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Corrections to the Table 740 (UL 4Tx OL SU-MIMO codebook subset)

David Mazzarese et Al.

Samsung Electronics

# Introduction

An editor's note explains that a sentence accepted in a contribution at Session 61 references an equation (203) that was deleted by another comment at the same session. This sentence defined the PMI for the UL 4Tx base codebook. This contribution proposes to directly indicate the exact codeword of the UL 4Tx base codebook in Table 740 to avoid the reference.

- References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”, 2009-06-31.

[2] IEEE 802.16m-07/002r8, “802.16m System Requirements”, 2009-01-15.

[3] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”, 2009-05-31.

[4] IEEE 802.16m-08/004r5, “The IEEE 802.16m Evaluation Methodology Document”, 2009-01-15.

[5] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”, 2008-09-18.

# Text proposal for inclusion in the 802.16m amendment

<Modify section “15.3.10.5.1.2 OL SU-MIMO subset for four transmit antennas” on line 34 page 318 as follows>

- 15.3.10.5.1.2 OL SU-MIMO subset for four transmit antennas

Table 739 gives the number of codewords Nw for each rank of the OL SU-MIMO codebook subset for 4Tx.

Table 739 – Size of the DL 4Tx OL SU-MIMO codebook subset

| Rank | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- |
| Nw | 4 | 4 | 4 | 4 |

The codewords, CUL,OL,SU(4, Mt,4,n), of the OL SU-MIMO codebook subset for four transmit antennas, CUL,OL,SU(4, Mt,4), are given in Table 740 for each rank Mt. The corresponding codewords, Cbase,UL(4,Mt,6,m), of the uplink base codebook for four transmit antennas, Cbase,UL(4,Mt,6), are given in Table 740.

Table 740 – CUL,OL,SU(4,1, 4), CUL,OL,SU(4,2,4), CUL,OL,SU(4,3,4) and CUL,OL,SU(4,4,4)

| CUL,OL,SU(4,1,4,n) |  | CUL,OL,SU(4,2,4,n) |  | CUL,OL,SU(4,3,4,n) |  | CUL,OL,SU(4,4,4,n) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| n | PMI m in UL base codebook of rank 1 Cbase,UL(4,1,6,m) | n | PMI m in UL base codebook of rank 2 Cbase,UL(4,2,6,m) | n | PMI m in UL base codebook of rank 3 Cbase,UL(4,3,6,m) | n | PMI m in UL base codebook of rank 3 Cbase,UL(4,4,6,m) |
| 0 | 9 Cbase,UL(4,1,6,9) | 0 | 9 Cbase,UL(4,2,6,9) | 0 | 9 Cbase,UL(4,3,6,9) | 0 | 9 Cbase,UL(4,4,6,9) |
| 1 | 15 Cbase,UL(4,1,6,15) | 1 | 15 Cbase,UL(4,2,6,15) | 1 | 15 Cbase,UL(4,3,6,15) | 1 | 15 Cbase,UL(4,4,6,15) |
| 2 | 49 Cbase,UL(4,1,6,49) | 2 | 49 Cbase,UL(4,2,6,49) | 2 | 49 Cbase,UL(4,3,6,49) | 2 | 49 Cbase,UL(4,4,6,49) |
| 3 | 55 Cbase,UL(4,1,6,55) | 3 | 55 Cbase,UL(4,2,6,55) | 3 | 55 Cbase,UL(4,3,6,55) | 3 | 55 Cbase,UL(4,4,6,55) |

The PMI m in the base codebook of rank Mt with Nt = 4 is given by vm+1 as in <<equation (203)>>.

[Editor’s note: equation 203 was deleted by comment 286 at Session 61>>>.