| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Passive Other RAT Discovery |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Won-Ik Kim, Kwangjae Lim, Sungcheol Chang, Jaesun Cha, Sungkyung Kim, Hyunjae Kim, Chulsik Yoon ETRI 138 Gajeongno, Yusong-gu, Daejeon, 305-700, KOREA | Voice: +82-42-860-6471 E-mail: woniks@etri.re.kr |
| Re: | Category: AWD comments / Area: Chapter 15.2.6.5.2.1.1 (Passive Other RAT Discovery) “Comments on AWD 15.2.6.5.2.1.1 (Passive Other RAT Discovery)” |  |
| Abstract | The contribution proposes the text of passive other RAT discovery section to be included in the 802.16m AWD |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m AWD. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |

Clarification on Passive Other RAT Discovery

Won-Ik Kim, Kwangjae Lim, Sungcheol Chang, Jaesun Cha, Sungkyung Kim, Hyunjae Kim, Chulsik Yoon

ETRI

# 1. Discussion

According to IEEE 802.16m AWD [1], AMS’s location information may be used to reduce scanning overhead and signaling overhead on other RAT discovery. Other RAT information which an AMS shall use for other RAT scanning may be restricted to specific access technologies, based on the distance between AMS’s location and RAP(Radio Access Point)’s location. In order to configure the optimized RAT list based on the distance, it may be required the information for RAP’s cell coverage as well as the location information of AMS and RAPs. The RAP’s cell coverage may be expressed as RAP’s cell type such as macro, micro, and femto (OSG/CSG).

In active other RAT discovery using 802.16m scanning, an ABS shall be responsible for configuring the recommended RAT information and providing it to the AMS. If an AMS provides its current location information with scanning request message to the ABS, the ABS may select available RAPs which may be scanned by the AMS immediately and configure the recommended RAT information including the available RAPs.

In case of passive other RAT discovery, an ABS may broadcast the other RAT information, which includes all RAPs neighboring with the ABS. In order to reduce the other RAT scanning overhead, an AMS may select and scan available RAPs out of a number of RAPs in the other RAT information. For this, if another RAT can support LBS-related information, it is necessary to provide the AMS with additional information such as RAP location and RAP cell type of the other RAT. In this contribution, we propose the additional information (i.e., RAP location and RAP cell type) to be included in the RAP information of AAI_ORAT-MSG.

# 2. Proposed Text for 802.16m AWD (802.16m-09/0010r2),

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

[Insert the information elements in the section 15.2.6.5.2.1.1 (Line 41, page 41)]

15.2.6.5.2.1.1 Passive Other RAT Discovery

………

The AAI_ORAT-MSG message includes the following:

- RAT Type: This field specifies air interface technology type.

- Pre-registration supported: This field indicates whether pre-registration is supported or not for Inter RAT handover.

- PHY Profile ID: The PHY Profile ID contains the information related to scan the corresponding RAP. The contents of PHY profile ID are TBD.

- Network boundary indication: This field indicates the whether the ABS which is sending this message is located in the boundary area of the 802.16m network or not.

- RAN Information: The RAN information specifies information for different radio access networks with various RATs defined by different standard bodies

- RAP Information: The RAP information specifies information for different radio access points (such as carrier frequency, BSID, preamble, RAP location, RAP cell type).

[--------------------------------------------------End of Text Proposal----------------------------------------------------]

# References

[1] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document (AWD)”