| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for Advanced Preamble design with tone dropping support in the IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Changyin Sun, Huiying Fang, Yanfeng Guan,Bo Sun ZTE Corporation | Voice: E-mail: fang.huiying@zte.com.cn sun.changyin@zte.com.cn |
| Re: | Category – AWD Comments / Area: Chapter 15.3.6.1 (Advanced Preamble) |  |
| Abstract | The contribution proposes the text of Advanced Preamble design with tone dropping in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for Advanced Preamble design with tone dropping support in the IEEE 802.16m Amendment

Changyin Sun, Huiying Fang,Yanfeng Guan,Bo Sun

ZTE Corporation

# Instruction

The contribution proposes the text of Advanced Preamble with tone dropping support to be included in the 802.16m amendment. The proposed text is developed so that it can be readily combined with IEEE P802.16 Rev2/D8 [1], it is compliant to the 802.16m SRD [2] and the 802.16m SDD [3], and it follows the style and format guidelines in [4].

# Reference

[1] IEEE P802.16 Rev2/D8, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” Dec. 2008.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

# Proposal for AP for tone dropping support

Motivation

The current SDD has tone dropping description as “Tone dropping based on 10 and 20 MHz systems can be used to support other various bandwidths”. But in AWD, the AP design only supports regular bandwidth such as 5,10 and 20MHz，so there is need of AP design to support irregular bandwidth with tone dropping.

Assumption

- Irregular bandwidths falling within 5~10 and 10~20 MHz systems;

- 5 MHz Physical Bandwidth for PA-Preamble;

- SA-Preamble occupies the entire irregular system bandwidth , based on 10 and 20 MHz systems SA sub-blocks;

- PA-Preamble now carries irregular bandwidth information.

AP design options

With Tone dropping, the irregular bandwidth is obtained by dropping some of the subcarriers based on 10 and 20 MHz systems.

The dropped subcarriers may have different granularity, here we consider sub-block defined in SA-preamble design and sub-band in resource mapping, the relationship is shown in Fig 1 with 5~10MHz bandwidth as an example.

10M

5M

DC

E

F

G

H

A

B

C

D

Sub-Band

Sub-Block

Fig 1

Option 1：AP design based on sub-block dropping

For regular bandwidth, the SA-preamble has a scalable design, that is, the 5MHz sequences are reused for 10 and 20MHz BW by repetition coding 8 sub-block {ABCDEFGH} as shown in Fig 2. Each sub-block consists of 18 bits deployed over 54 sub-carriers.

Fig 2

For irregular bandwidth, the SA-preamble is obtained by dropping some of sub-blocks of the regular bandwidth on both side of DC carrier.

10M

5M

DC

E

F

G

H

A

B

a subblock

Irregular BW

Fig 3

Base on the sub-block dropping design, the irregular BW range and corresponding SA are listed below:

Table.1 AP design based on sub-block dropping

| Subblock of SA after dropping | Irregular BW range |
| --- | --- |
| S0={A,B,C,D,E,F,G,H} | 5 MHz |
| S1={H,A,B,C,D,E,F,G,H,A} | 6.8-7.9MHz |
| S2={G,H,A,B,C,D,E,F,G,H,A,B} | 7. 9-8.9MHz |
| S3={F,G,H,A,B,C,D,E,F,G,H,A,B,C} | 8.9-10 MHz |
| S4={E,F,G,H,A,B,C,D,E,F,G,H,A,B,C,D} | 10MHz |

Option 2：AP design based on sub-band dropping

For regular BW systems, such as 5MHZ, the maximum number of sub-bands that can be formed is denoted as Nsub where Nsub=NPRUN1,so Nsub=6 for 5MHz system, and each sub-band occupy 72 sub-carriers.

For irregular bandwidth, the bandwidth occupied by resource is firstly obtained by dropping some of sub-bands of the regular bandwidth, then, the SA-preamble will consist of the sub-blocks of the regular bandwidth lying within the bandwidth occupied by resource. The bandwidth relationship of resource and SA is shown in Fig 4 with 5~10MHz bandwidth as an example. We can see that the BW of SA is less than BW of resource in this example.

Irregular BW

5M

E

F

G

H

A

Fig  4

Base on the sub-band dropping design, the irregular Resource BW and corresponding SA are listed below:

Table.2. AP design based on sub-band dropping

| Sub-band of resource / Subblock of SA | Resource BW/ SA BW |
| --- | --- |
| { SB1,SB2,SB3,SB4,SB5,SB6}/ (S0) | 5 MHz |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7}/ （S0） | 6. 5/5 |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7,SB8}/ （S1） | 7.2/6.8 |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7,SB8,SB9}/ （S2） | 7.9/7.9 |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7,SB8,SB9,SB10}/ （S2） | 8.6/7.9 |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7,SB8,SB9,SB10,SB11}/ （S3） | 9.3/8.9 |
| { SB1,SB2,SB3,SB4,SB5,SB6,SB7,SB8,SB9,SB10,SB11,SB12}/ （S4） | 10MHz |

Pros and cons of the two options

Cons of the Option1:

For Option1, occupied PRU number of the SA obtained by dropping some subblocks ( for example, occupied PRU number of S1 in Table 1）may be 4k+2, but not 4k，so if resource BW=SA BW is assumed, special permutation rule other than EQ.177 in AWD is required.

For Option2,  some of the BWs is not reasonable, for  example, 9.3MHz which is much closer to 10MHz, and some of the BWs have not unique corresponding SA design, for  example,7.9 and 8.6MHz have the same SA design.

Proposed design

Based on the discussion above, we propose to use the combined dropping of sub-block and sub-band: the sub-carrier of the dropping unit is m×l.c.m(54,72), where m is an integer, that is, the dropping unit is the Least Common Multiple of sub-block and sub-bands.

The features of the proposed design:

- Uniform coverage of the whole BW range;

- Minimum PA sequence number to reduce cell search complex;

- SA occupy almost full bandwidth;

- No special permutation rule needed for resource of irregular BW system.

Proposed design for tone dropping

1) Irregular BW info is carried by different PA sequences,

2) BW(both regular and irregular) and corresponding SA design is shown below,

3) SA occupied almost full bandwidth,

| Irregular BW info carried by PA | Sub-block set in SA |
| --- | --- |
| 5M(512 FFT) | A,B,C,D,E,F,G,H |
| 10M(or 1024 FFT) | E,F,G,H, A,B,C,D,E,F,G,H,A,B,C,D |
| 20M(or 2048 FFT) | E,F,G,H,A,B,C,D,E,F,G,H,A,B,C,D, E,F,G,H, A,B,C,D,E,F,G,H,A,B,C,D |
| Irregular BW 1(=7.9MHz for 28/25 series) (1024 FFT，Nused（DC excluded）=648) | G,H,A,B,C,D,E,F,G,H,A,B |
| Irregular BW 2(=13.7 MHz for 28/25 series) (2048 FFT，Nused（DC excluded）=1080) | C,D,E,F,G,H,A,B,C,D,E,F,G,H,A,B,C,D,E,F |
| Irregular BW 3(=15.8 MHz for 28/25 series) (2048 FFT，Nused（DC excluded）=1296) | A,B,C,D,E,F,G,H,A,B,C,D,E,F,G,H, A,B,C,D,E,F,G,H, |
| Irregular BW 4(=17.9 MHz for 28/25 series) (2048 FFT，Nused（DC excluded）=1512) | G,H,A,B,C,D,E,F,G,H,A,B,C,D,E,F,G,H, A,B,C,D,E,F,G,H,A,B |

- Proposed Text

[In IEEE 802.16m-09/0010r2, section 15.3.6.1.1, Insert the following below Table 653 ]

-------------------------------------------------  Text #1 Start  ---------------------------------------------------------------

To support the irregular bandwidth with tone dropping, at least one of the reserved PA sequences (eg. 4 reserved PA sequences) is used to indicate the irregular bandwidth.

-------------------------------------------------  Text #1 End   ---------------------------------------------------------------

[In IEEE 802.16m-09/0010r2, section 15.3.6.1.2, Insert the following into line 19 of page.155 ]

----------------------------------------------------------  Text #2 Start  -----------------------------------------------------

For support of irregular bandwidth by tone dropping, the dropping unit of SA sub-block should take both sub-block and sub-band into account, the sub-carrier number of the dropping unit is m×l.c.m(54,72), where m is an integer and l.c.m is abbr. of the Least Common Multiple. And the supported irregular bandwidth is indicated by PA index in Table 653.

----------------------------------------------------------  Text #2 End  -------------------------------------------------------