| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal of Sleep Mode Operation in IEEE 802.16m |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Hung-Yu Wei, Ching-Chun Chou National Taiwan University Frank Ren Chun-Yen Wang ITRI | hywei@cc.ee.ntu.edu.tw |
| Re: | TGm Call for Comments and Contributions on Amendment Working Text |  |
| Abstract | The contribution proposes modification to the text of Sleep mode operation in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal of Sleep Mode Operation in IEEE 802.16m

Hung-Yu Wei, Ching-Chun Chou

National Taiwan University

Frank Ren, Chun-Yen Wang

ITRI

- Introduction:

Section 15.2.16.2.3.1 of the 802.16m-09/0010r2 AWD specifies the traffic indication operation in sleep mode. AMS should check if there is positive traffic indication on receiving traffic indication message. On positive indication, there will be DL MAC PDU from the ABS to the AMS. The AMS could end the listening window on negative indication.

In view of energy conservation and efficient wireless transmission, we recommend that CQI should be taken as a parameter for DL transmission when the sleep mode is in progress. In current AWD, CQI is reported using the fast feedback channel. Fast feedback channel could be combined with the sleep mode operation for more efficient energy conservation.

The AMS with good channel quality should be scheduled DL transmission. In contrast, the AMS with poor channel quality should enter sleep mode for energy saving. Such channel-aware sleep mode operation could conserve the energy of the AMS with non-real-time traffic requirement.

Therefore, we propose a mechanism to integrate CQI report from fast feedback channel and traffic indication in sleep mode operation.

AMS with CQI higher than threshold should report

AMS

ABS

ABS sends DL non-real-time data

CQI_Thershold_High=4

CQI_Thershold_Low=2

AMS

ABS

AMS with poor CQI should continue sleeping

CQI Level=1

CQI_Thershold_High=4

CQI_Thershold_Low=2

Figure 1: Illustration for the combination with DL data transmission and CQI report

CQI Level=5

- Proposed Text for 802.16m amendment

In the chapter “15.2.16.2.3.1 Traffic Indication,” add the following text at line #35 on page 91

---------------------------------------------------------Text Start----------------------------------------------------------------

“The ABS may send positive indication if the fast feedback channel reported CQI is above CQI_threshold_high. The ABS may also send negative indication when the received CQI quality is below CQI_threshold_low, or no CQI report is received.”

---------------------------------------------------------Text End----------------------------------------------------------------