| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on AWD Idle Mode Messages |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Yongho Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7188 E-mail: gwpark@lge.com |
| Re: | IEEE 802.16m-09/0012, “Call for Comments and Contributions on Project 802.16m Amendment Working Document”. Target topic: “Idle Mode” |  |
| Abstract | The contribution proposes the texts of contents regarding the idle mode operations to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on AWD Idle Mode Messages

Giwon Park, Kiseon Ryu, Yongho Kim, Jin Sam Kwak

LG Electronics.

- Introduction

This contribution includes the proposed texts for idle mode sections to be in the 802.16m Amendment. The technical texts and ToCs are inherited from the 802.16m SRD [1], 802.16m SDD [2], and the 802.16 Rev2/D9 [3]. In this contribution, amendment text including several idle mode messages and illustrations is proposed for the idle mode section in which 16m members have enough consensus.

- Rationale/Motivation of modifications

This contribution modified the AWD by adding the detailed descriptions for the following issues.

  - Idle mode messages and parameters

AAI_DREG-REQ/RSP, AAI_RNG-REQ/RSP for LU, AAI_PAG-ADV, PGID Information

- References

[1] IEEE 802.16m-07/002r8, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

- Text proposal for the 802.16m amendment

[Add the following text in the section 15.2.5]

-----------------------------------------------------------Start of the Text---------------------------------------------------------

## 15.2.5 AAI MAC Management Messages

## 15.2.5.x AAI_DREG-REQ

An AMS may send a AAI_DREG-REQ message to a ABS in order to notify the ABS of AMS deregistration from normal operation service from the ABS. The AAI_DREG-REQ message includes one of the following parameters:

      - De-registration Request Code

      - Paging Cycle Request

      - AMS mobility Information

An AMS may inform its mobility (slow, medium, fast) when the AMS enters the idle mode. The AMS mobility information may be used to assign paging group(s) to the AMS.

Table xx. AMS Mobility Information field

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| AMS Mobility Information | 2 | Mobility Information of AMS 0b00 = Slow 0b01 = Medium 0b10 = Fast 0b11 = Reserved |

      - Idle Mode Retain Information

The Idle Mode Retain Information provided as part of this message is indicative only. Network reentry from idle mode process requirements may change at the time of actual reentry. For each bit location, a value of 0 indicates the information for the associated reentry management messages shall not be retained and managed, a value of 1 indicates the information for the associated reentry management message shall be retained and managed.

## 15.2.5.x AAI_DREG-CMD

The AAI_DREG-CMD message shall be transmitted by the ABS to force the AMS to change its access state. The ABS may transmit the AAI_DREG-CMD message unsolicited or in response to a AAI_DREG-REQ message. Upon receiving a AAI_DREG-CMD message, the AMS shall take the action indicated by the action code.

The AAI_DREG-REQ message includes one of the following parameters:

- Paging Information

The Paging Information defines the PAGING CYCLE, PAGING OFFSET, PAGING MESSAGE OFFSET, Paging-Group-ID and Paging Interval Length parameters to be used by the AMS in IDLE mode.

Table xx. Paging Information format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| Paging Information () { |  |  |
| PAGING_CYCLE | 16 | Cycle in which the paging message is transmitted within the paging group. |
| PAGING_MESSAGE_OFFSET | 16 | This offset determines the frame which AMS shall monitor paging message within the Paging Listening Interval. |
| Multiple_Paging_Group_Flag | 1 | This field indicates whether AMS is assigned to multiple paging groups or not. |
| if (Multiple_Paging_Group_Flag == 1) { |  |  |
| Num_Paging_Group_IDs | 8 | Number of paging group the AMS is assigned to (First paging group within the message is called primary paging group and rest of the assigned paging group is called secondary paging group). |
| for(i=0; i< Num_Paging_Group_IDs; i++) { |  |  |
| Paging_Group_ID | 16 | ID of the Paging Group the AMS is assigned to. |
| } |  |  |
| PRIMARY_PAGING_OFFSET | 16 | Primary paging offset is associated with the primary paging group. This offset determines the superframe within the cycle from which the paging interval starts. Shall be smaller than PAGING_CYCLE value. |
| SECONDARY_PAGING_OFFSET | 16 | Secondary paging offset is associated with the Secondary paging group. Secondary Paging Offset is used if only the AMS is assigned to multiple paging groups. This offset determines the superframe within the cycle from which the paging interval starts. Shall be smaller than PAGING_CYCLE value. |
| } |  |  |
| if (Multiple_Paging_Group_Flag == 0) { |  |  |
| Paging-Group-ID | 16 | ID of the Paging Group the AMS is assigned to. |
| PAGING_OFFSET | 16 | This offset determines the superframe within the cycle from which the paging interval starts. Shall be smaller than PAGING_CYCLE value. |
| } |  |  |
| } |  |  |

- AMS Temporary Identifier

- Paging Controller ID

- Idle Mode Retain Information

- REQ-Duration

## 15.2.5.x AAI_RNG-REQ

The AAI_RNG-REQ message shall be transmitted by the AMS when the AMS is attempting to location update. The AAI_RNG-REQ message for LU shall include the following parameters:

- Ranging Purpose Indication

- Paging Group ID(s)

- AMS Temporary Identifier

- Paging Controller ID

- Power Down Indicator

- AMS Mobility Information

An AMS may inform its mobility (slow, medium, fast) during location update procedure. The AMS mobility information may be used to assign paging group(s) to the AMS.

## 15.2.5.x AAI_RNG-RSP

The AAI_RNG-RSP message shall be transmitted by the ABS in response to a received RNG-REQ. The AAI_RNG-RSP message for LU shall include the following parameters:

- Location Update Response

- Paging Group ID(s)

- AMS Temporary Identifier

- Paging Controller ID

- Paging Information

## 15.2.5.x AAI_PAG-ADV

The AAI_PAG_ADV message shall be broadcasted by the ABS during the ABS paging listening interval. The AAI_PAG-ADV message shall include the following parameters:

- Num_Positive_Paging_Group_IDs

This field indicates the number of PG ID which paging indicator bitmap in PGID information is set to non-zero.

- Paging Group Bitmap

The size of Paging Group Bitmap is equal to Num_PGIDs in PGID information. Each bit indicates the presence of the AMS’s paging message associated with the corresponding paging group.

- AMS Temporary Identifier

- Action Code

- Extension Flag

Table xx. AAI-PAG-ADV message

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| AAI_PAG-ADV () { |  |  |
| Management Message Type | TBD | AAI_PAG-ADV message |
| if (Paging indicator usage flag == 1) { |  |  |
| for (i = 0; i < Num_Positive_Paging_Group_IDs; i++ ) { | - | Num_Positive_Paging_Group_IDs is the number of PGIDs of which paging indicator bitmap in PGID information is set to non-zero. |
| Num_AMS | TBD | Number of AMS Identifier |
| for(j=0; j<Num_AMS; j++;) { |  |  |
| AMS temporary identifier | TBD | Temporary Identifier of the AMS |
| Action Code | 2 | Paging action instruction to AMS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| } |  |  |
| } |  |  |
| } //End of if (Paging indicator usage flag == 1) |  |  |
| else if (Paging indicator usage flag == 0) { |  |  |
| Paging Group Bitmap | Variable | The size of Paging Group Bitmap is equal to Num_PGIDs in PGID information. Each bit indicates the presence of the AMS’s paging message associated with the corresponding paging group. |
| for(j=0; j< Num_Positive_Paging_Group_IDs; j++) { |  | Num_Positive_Paging_Group_IDs is the number of PG ID which is set to one. |
| Num_AMS | TBD | Number of AMS Identifier |
| for(k=0; k<Num_AMS; k++;) { |  |  |
| AMS temporary identifier | TBD | Temporary Identifier of the AMS |
| Action Code | 2 | Paging action instruction to AMS 0b00 = No action required 0b01 = Perform ranging to establish location and acknowledge message 0b10 = Enter network 0b11 = Reserved |
| } |  |  |
| } |  |  |
| } //End of else if (Paging indicator usage flag == 0) |  |  |
| Extension Flag | 1 | Extension flag is used to indicate the presence of other paging message in the subsequent frame. 0: Current paging message is last paging message within paging listening interval. 1: Other paging message is transmitted in the subsequent frame. |
| } |  |  |

## 15.2.17.2.3 Operation during paging listening interval

[Section 15.2.7.2.3, page 100, line 62]

The location of PGID(s) is TBD.

Table xxx. PGID Information

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| Format of PGID information(){ | - | - |
| Num_PGIDs | 3 | Number of Paging Group IDs |
| Paging indicator usage flag | 1 | 0: Paging indicator bitmap is not included. 1: Paging indicator bitmap is included. |
| for(i=0;i<Num_PGIDs;i++) { | - | - |
| PG ID | 16 | Paging group identifier |
| } | - | - |
| if (Paging indicator usage flag==1) { | - | - |
| Paging indicator bitmap | 4 * Number of PGIDs | The paging indication bitmap size is 4 bit and is assigned per paging group. Each bit refers to a frame within a super frame and indicates the presence of the AMS’s paging message within the frame. 0x00: If this bit is set to 1, AMS’s paging message is transmitted in 1st frame during the paging listening interval (1 superframe). 0x01: If this bit is set to 1, AMS’s paging message is transmitted in 2nd frame during the paging listening interval (1 superframe). 0x02: If this bit is set to 1, AMS’s paging message is transmitted in 3rd frame during the paging listening interval (1 superframe). 0x03: If this bit is set to 1, AMS’s paging message is transmitted in 4th frame during the paging listening interval (1 superframe). |
| } | - | - |
| } | - | - |

-----------------------------------------------------------End of the Text---------------------------------------------------------