| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Design of the BR ACK A-MAP IE |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk Ronny Kim LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16m-09/0028, “Call for Contributions on Project 802.16m Amendment Content (draft)”. |  |
| Abstract | The contribution proposes the text of contents for DSx MAC management message to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Design of the BR ACK A-MAP IE

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Yongho Kim

LG Electronics

- Introduction

This contribution includes the proposed text for the BR ACK A-MAP IE in the IEEE 802.16m Amendment. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE S80216m-09/1418, “Design of the BR ACK A-MAP IE”

- Text proposal for the 802.16m amendment

Remedy 1

On the 16m AWD 80216m-09_0010r2, page 65, line 51, make the following changes to subsection 15.2.11.1.1:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.11.1.1 Contention-based random access bandwidth request

The BR Acknowledgement A-MAP IE indicates the decoding status of the BR opportunities in the previous UL frame. It shall include successfully detected code indexes in ascending order. It includes the allocation information only for the fixed size of bandwidth such as BW-REQ message. The allocation shall be in the first UL subframe relevant to the A-MAP region regardless of DL/UL ratio and Nsubframe, A-MAP. The UL resource and HFA are allocated to the code index whose grant indicator is ‘1’ in ascending order. The maximum number of the HARQ retransmission is TBD.

Table 34—BR-ACK A-MAP IE Format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| ACK A-MAP IE() { | - | - |
| A-MAP Type | 4 | BR ACK A-MAP IE |
| BR-ACK Bitmap | N_BR_Slots | Each bit indicates the decoding status of BR code in the corresponding BR opportunity. 0b0: No BR code is detected, 0b1: At least one code is detected. |
| MSB of resource start offset | 2 | 0b00, 0b01, 0x10 :2bit- MSB of the resource start offset of the resource allocation 0b11: No grant exist in this BR ACK A-MAP IE. |
| If (MSB of resource start offset !=0b11) { |  |  |
| LSB of resource start offset | 5 | This field is LSB of the resource start offset of the Resource allocation for BR Header. |
| HFA start offset | [4] | This field is start offset of HARQ Feedback Allocation. [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Allocation size | 2 | Allocated resource size 0b00: 1 LRU, 0b01: 2 LRUs, 0b10 : 3 LRUs, 0b11: 4LRUs |
| } |  |  |
| For (i=0; N_BR_Slots; i++) { |  |  |
| If (BR-ACK Bitmap[i] == 1) { |  |  |
| Number of Received codes (L) | TBD | The number of BR code indices included in this ACK A-MAP IE. |
| For (j=0; j<L; j++) { |  |  |
| Code index | 5 | Code index received in the BR opportunity |
| MSG decoding indicator | 1 | To indicate the decoding status of quick access message |
| If (MSB of resource start offset !=0b11) { |  |  |
| Grant indicator | 1 | To indicate whether grant of BR Header for the code index is included or not If this bit is set, the UL resource is allocated with fixed size and MCS. |
| } } } } |  |  |
| MCRC | 16 | CRC masked by the reserved STID for BR ACK A-MAP |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 2

On the 16m AWD 80216m-09_0010r2, page 201, line 34, make the following changes to subsection 15.3.6.5.2.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.2.2 DL basic assignment A-MAP IE

Table 669 describes Assignment A-MAP IE Types.

Table 669—A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Feedback Allocation A-MAP IE |
| 0b0011 | Reserved BR-ACK A-MAP IE |
| 0b0100 | Reserved |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 3

On the 16m AWD 80216m-09_0010r2, page 229, line 49, add the following subsection (move this BR-ACK A-MAP IE from 15.2.11.1.1 to 15.3.6.5.2.12)

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.2.12 BR-ACK A-MAP IE

Table xxx—BR-ACK A-MAP IE Format

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| ACK A-MAP IE() { | - | - |
| A-MAP Type | 4 | BR ACK A-MAP IE |
| BR-ACK Bitmap | N_BR_Slots | Each bit indicates the decoding status of BR code in the corresponding BR opportunity. 0b0: No BR code is detected, 0b1: At least one code is detected. |
| MSB of resource start offset | 2 | 0b00, 0b01, 0x10 :2bit- MSB of the resource start offset of the resource allocation 0b11: No grant exist in this BR ACK A-MAP IE. |
| If (MSB of resource start offset !=0b11) { |  |  |
| LSB of resource start offset | 5 | This field is LSB of the resource start offset of the Resource allocation for BR Header. |
| HFA start offset | [4] | This field is start offset of HARQ Feedback Allocation. [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Allocation size | 2 | Allocated resource size 0b00: 1 LRU, 0b01: 2 LRUs, 0b10 : 3 LRUs, 0b11: 4LRUs |
| } |  |  |
| For (i=0; N_BR_Slots; i++) { |  |  |
| If (BR-ACK Bitmap[i] == 1) { |  |  |
| Number of Received codes (L) | TBD | The number of BR code indices included in this ACK A-MAP IE. |
| For (j=0; j<L; j++) { |  |  |
| Code index | 5 | Code index received in the BR opportunity |
| MSG decoding indicator | 1 | To indicate the decoding status of quick access message |
| If (MSB of resource start offset !=0b11) { |  |  |
| Grant indicator | 1 | To indicate whether grant of BR Header for the code index is included or not If this bit is set, the UL resource is allocated with fixed size and MCS. |
| } } } } |  |  |
| MCRC | 16 | CRC masked by the reserved STID for BR ACK A-MAP |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------