| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text of A-MAP structure and signaling for IEEE 802.16m Amendment |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Jinsoo Choi, Sunam Kim, Jinsam Kwak LG Electronics | {emptylie, snkim816, samji}@lge.com |
| Re: | IEEE 802.16m-09/0028r1, “Call for Comments and Contributions on Project 802.16m Amendment Content” Target topic: “DL Control Structure” Section |  |
| Abstract | The contribution provides the proposed text of A-MAP structure and signaling to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text of A-MAP Structure and Signaling
for IEEE 802.16m Amendment

Jinsoo Choi, Sunam Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution provides the proposed text of A-MAP structure and related signaling field in non-user specific A-MAP to be included in the DL control section of the AWD.

- Modification and rationale on current AWD

- The A-MAP allocation in multiple FFR case

  - When the multiple frequency partitions are supported, the A-MAP can be allocated in reuse 1 frequency zone or reuse 3 frequency zone or both frequency zones.

- The frequency partition (FP) configuration for non-user specific A-MAP

  - It is recommended that non-user specific A-MAP is addressed in the highest power level frequency partition to reduce the required resource overhead. If it can be allocated into both reuse 1 frequency zone and reuse 3 frequency zone, the indication for FP configuration should be contained in SFH to avoid unnecessary MS detection trial.

- Non-user specific A-MAP information and supporting table

  - Current AWD addresses non-user specific A-MAP signals the number of assignment A-MAP channels of each MLRU group with a table for informing assignment resources.

  - Based on the current AWD, the details of the table entry should be filled out according to applied MCS level and IE size. For minimizing the overhead/complexity of non-user specific A-MAP design, one unified table to inform the number of A-A-MAP channels is recommended to be designed, irrespective of bandwidth and FP configuration. In the proposed remedy 3, one exemplary table for non-user specific A-MAP is represented assuming 16MLRUs for A-A-MAP.

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[2] IEEE 802.16m-08/003r9a, “The Draft IEEE 802.16m System Description Document”

[3] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

-------------------------------  Text Start  ---------------------------------------------------

# DL Control Structure

# Advanced Preamble

# DL Control Channels

# Superframe Header

# Advanced Map (A-MAP)

[Proposed remedy #1: Modify from the line 28 to the line 29 of page 186 with following remedy]

If frequency resource of a DL Subframe is composed of reuse 3 and reuse 1 frequency zone, the reuse 1 frequency zone or highest power level frequency partition in reuse 3 frequency zone may contain an A-MAP region.

# Non-user specific A-MAP

[Proposed remedy #2: After the line 1 of page 187, insert following sentence]

If frequency resource of a DL Subframe is composed of reuse 3 and reuse 1 frequency zone, the non-user specific A-MAP can be allocated into reuse 1 frequency zone or power boosted reuse 3 frequency zone. The indication for frequency partition (FP) configuration of non-user specific A-MAP should be conveyed in SFH

# DL Control Information Elements

# Broadcast Control Information Elements

# Unicast Control Information Elements

# Non-user specific A-MAP IE

[Proposed remedy #3: From the line 17 to 29 with table 667 of page 198, modify as following]

Table 667 [Detailed entry is TBD] shows the number of the assignment A-MAPs in each assignment A-MAP group.

Table 667- The number of assignment A-MAPs in each assignment A-MAP group

(Unit: The number of assignment A-MAP channels per each group)

| index | Assignment A-MAP group-1. | Assignment A-MAP group-2 | Assignment A-MAP group-3 | Assignment A-MAP group-4 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 0 | 0 | 0 |
| 2 | 2 | 0 | 0 | 0 |
| 3 | 3 | 0 | 0 | 0 |
| 4 | 4 | 0 | 0 | 0 |
| 5 | 5 | 0 | 0 | 0 |
| 6 | 6 | 0 | 0 | 0 |
| 7 | 7 | 0 | 0 | 0 |
| 8 | 8 | 0 | 0 | 0 |
| 9 | 9 | 0 | 0 | 0 |
| 10 | 10 | 0 | 0 | 0 |
| 11 | 11 | 0 | 0 | 0 |
| 12 | 12 | 0 | 0 | 0 |
| 13 | 13 | 0 | 0 | 0 |
| 14 | 14 | 0 | 0 | 0 |
| 15 | 15 | 0 | 0 | 0 |
| 16 | 16 | 0 | 0 | 0 |
| 17 | 0 | 1 | 0 | 0 |
| 18 | 1 | 1 | 0 | 0 |
| 19 | 2 | 1 | 0 | 0 |
| 20 | 3 | 1 | 0 | 0 |
| 21 | 4 | 1 | 0 | 0 |
| 22 | 5 | 1 | 0 | 0 |
| 23 | 6 | 1 | 0 | 0 |
| 24 | 7 | 1 | 0 | 0 |
| 25 | 8 | 1 | 0 | 0 |
| 26 | 9 | 1 | 0 | 0 |
| 27 | 10 | 1 | 0 | 0 |
| 28 | 11 | 1 | 0 | 0 |
| 29 | 12 | 1 | 0 | 0 |
| 30 | 13 | 1 | 0 | 0 |
| 31 | 14 | 1 | 0 | 0 |
| 32 | 0 | 2 | 0 | 0 |
| 33 | 1 | 2 | 0 | 0 |
| 34 | 2 | 2 | 0 | 0 |
| 35 | 3 | 2 | 0 | 0 |
| 36 | 4 | 2 | 0 | 0 |
| 37 | 5 | 2 | 0 | 0 |
| 38 | 6 | 2 | 0 | 0 |
| 39 | 7 | 2 | 0 | 0 |
| 40 | 8 | 2 | 0 | 0 |
| 41 | 9 | 2 | 0 | 0 |
| 42 | 10 | 2 | 0 | 0 |
| 43 | 11 | 2 | 0 | 0 |
| 44 | 12 | 2 | 0 | 0 |
| 45 | 0 | 3 | 0 | 0 |
| 46 | 1 | 3 | 0 | 0 |
| 47 | 2 | 3 | 0 | 0 |
| 48 | 3 | 3 | 0 | 0 |
| 49 | 4 | 3 | 0 | 0 |
| 50 | 5 | 3 | 0 | 0 |
| 51 | 6 | 3 | 0 | 0 |
| 52 | 7 | 3 | 0 | 0 |
| 53 | 8 | 3 | 0 | 0 |
| 54 | 9 | 3 | 0 | 0 |
| 55 | 10 | 3 | 0 | 0 |
| 56 | 0 | 4 | 0 | 0 |
| 57 | 1 | 4 | 0 | 0 |
| 58 | 2 | 4 | 0 | 0 |
| 59 | 3 | 4 | 0 | 0 |
| 60 | 4 | 4 | 0 | 0 |
| 61 | 5 | 4 | 0 | 0 |
| 62 | 6 | 4 | 0 | 0 |
| 63 | 7 | 4 | 0 | 0 |
| 64 | 8 | 4 | 0 | 0 |
| 65 | 0 | 5 | 0 | 0 |
| 66 | 1 | 5 | 0 | 0 |
| 67 | 2 | 5 | 0 | 0 |
| 68 | 3 | 5 | 0 | 0 |
| 69 | 4 | 5 | 0 | 0 |
| 70 | 5 | 5 | 0 | 0 |
| 71 | 6 | 5 | 0 | 0 |
| 72 | 0 | 6 | 0 | 0 |
| 73 | 1 | 6 | 0 | 0 |
| 74 | 2 | 6 | 0 | 0 |
| 75 | 3 | 6 | 0 | 0 |
| 76 | 4 | 6 | 0 | 0 |
| 77 | 0 | 7 | 0 | 0 |
| 78 | 1 | 7 | 0 | 0 |
| 79 | 2 | 7 | 0 | 0 |
| 80 | 0 | 8 | 0 | 0 |
| 81 | 0 | 0 | 1 | 0 |
| 82 | 1 | 0 | 1 | 0 |
| 83 | 2 | 0 | 1 | 0 |
| 84 | 3 | 0 | 1 | 0 |
| 85 | 4 | 0 | 1 | 0 |
| 86 | 5 | 0 | 1 | 0 |
| 87 | 6 | 0 | 1 | 0 |
| 88 | 7 | 0 | 1 | 0 |
| 89 | 8 | 0 | 1 | 0 |
| 90 | 9 | 0 | 1 | 0 |
| 91 | 10 | 0 | 1 | 0 |
| 92 | 11 | 0 | 1 | 0 |
| 93 | 12 | 0 | 1 | 0 |
| 94 | 0 | 1 | 1 | 0 |
| 95 | 1 | 1 | 1 | 0 |
| 96 | 2 | 1 | 1 | 0 |
| 97 | 3 | 1 | 1 | 0 |
| 98 | 4 | 1 | 1 | 0 |
| 99 | 5 | 1 | 1 | 0 |
| 100 | 6 | 1 | 1 | 0 |
| 101 | 7 | 1 | 1 | 0 |
| 102 | 8 | 1 | 1 | 0 |
| 103 | 9 | 1 | 1 | 0 |
| 104 | 10 | 1 | 1 | 0 |
| 105 | 0 | 2 | 1 | 0 |
| 106 | 1 | 2 | 1 | 0 |
| 107 | 2 | 2 | 1 | 0 |
| 108 | 3 | 2 | 1 | 0 |
| 109 | 4 | 2 | 1 | 0 |
| 110 | 5 | 2 | 1 | 0 |
| 111 | 6 | 2 | 1 | 0 |
| 112 | 7 | 2 | 1 | 0 |
| 113 | 8 | 2 | 1 | 0 |
| 114 | 0 | 3 | 1 | 0 |
| 115 | 1 | 3 | 1 | 0 |
| 116 | 2 | 3 | 1 | 0 |
| 117 | 3 | 3 | 1 | 0 |
| 118 | 4 | 3 | 1 | 0 |
| 119 | 5 | 3 | 1 | 0 |
| 120 | 6 | 3 | 1 | 0 |
| 121 | 0 | 4 | 1 | 0 |
| 122 | 1 | 4 | 1 | 0 |
| 123 | 2 | 4 | 1 | 0 |
| 124 | 3 | 4 | 1 | 0 |
| 125 | 4 | 4 | 1 | 0 |
| 126 | 0 | 5 | 1 | 0 |
| 127 | 1 | 5 | 1 | 0 |
| 128 | 2 | 5 | 1 | 0 |
| 129 | 0 | 6 | 1 | 0 |
| 130 | 0 | 0 | 2 | 0 |
| 131 | 1 | 0 | 2 | 0 |
| 132 | 2 | 0 | 2 | 0 |
| 133 | 3 | 0 | 2 | 0 |
| 134 | 4 | 0 | 2 | 0 |
| 135 | 5 | 0 | 2 | 0 |
| 136 | 6 | 0 | 2 | 0 |
| 137 | 7 | 0 | 2 | 0 |
| 138 | 8 | 0 | 2 | 0 |
| 139 | 0 | 1 | 2 | 0 |
| 140 | 1 | 1 | 2 | 0 |
| 141 | 2 | 1 | 2 | 0 |
| 142 | 3 | 1 | 2 | 0 |
| 143 | 4 | 1 | 2 | 0 |
| 144 | 5 | 1 | 2 | 0 |
| 145 | 6 | 1 | 2 | 0 |
| 146 | 0 | 2 | 2 | 0 |
| 147 | 1 | 2 | 2 | 0 |
| 148 | 2 | 2 | 2 | 0 |
| 149 | 3 | 2 | 2 | 0 |
| 150 | 4 | 2 | 2 | 0 |
| 151 | 0 | 3 | 2 | 0 |
| 152 | 1 | 3 | 2 | 0 |
| 153 | 2 | 3 | 2 | 0 |
| 154 | 0 | 4 | 2 | 0 |
| 155 | 0 | 0 | 3 | 0 |
| 156 | 1 | 0 | 3 | 0 |
| 157 | 2 | 0 | 3 | 0 |
| 158 | 3 | 0 | 3 | 0 |
| 159 | 4 | 0 | 3 | 0 |
| 160 | 0 | 1 | 3 | 0 |
| 161 | 1 | 1 | 3 | 0 |
| 162 | 2 | 1 | 3 | 0 |
| 163 | 0 | 2 | 3 | 0 |
| 164 | 0 | 0 | 4 | 0 |
| 165 | 0 | 0 | 0 | 1 |
| 166 | 1 | 0 | 0 | 1 |
| 167 | 2 | 0 | 0 | 1 |
| 168 | 3 | 0 | 0 | 1 |
| 169 | 4 | 0 | 0 | 1 |
| 170 | 5 | 0 | 0 | 1 |
| 171 | 6 | 0 | 0 | 1 |
| 172 | 7 | 0 | 0 | 1 |
| 173 | 8 | 0 | 0 | 1 |
| 174 | 0 | 1 | 0 | 1 |
| 175 | 1 | 1 | 0 | 1 |
| 176 | 2 | 1 | 0 | 1 |
| 177 | 3 | 1 | 0 | 1 |
| 178 | 4 | 1 | 0 | 1 |
| 179 | 5 | 1 | 0 | 1 |
| 180 | 6 | 1 | 0 | 1 |
| 181 | 0 | 2 | 0 | 1 |
| 182 | 1 | 2 | 0 | 1 |
| 183 | 2 | 2 | 0 | 1 |
| 184 | 3 | 2 | 0 | 1 |
| 185 | 4 | 2 | 0 | 1 |
| 186 | 0 | 3 | 0 | 1 |
| 187 | 1 | 3 | 0 | 1 |
| 188 | 2 | 3 | 0 | 1 |
| 189 | 0 | 4 | 0 | 1 |
| 190 | 0 | 0 | 1 | 1 |
| 191 | 1 | 0 | 1 | 1 |
| 192 | 2 | 0 | 1 | 1 |
| 193 | 3 | 0 | 1 | 1 |
| 194 | 4 | 0 | 1 | 1 |
| 195 | 0 | 1 | 1 | 1 |
| 196 | 1 | 1 | 1 | 1 |
| 197 | 2 | 1 | 1 | 1 |
| 198 | 0 | 2 | 1 | 1 |
| 199 | 0 | 0 | 2 | 1 |
| 200 | 0 | 0 | 0 | 2 |

-------------------------------  Text End  ---------------------------------------------------