| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Design of the CDMA Allocation A-MAP IE |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk Ronny Kim LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16m-09/0028, “Call for Contributions on Project 802.16m Amendment Content (draft)”. |  |
| Abstract | The contribution proposes the text of contents for DSx MAC management message to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Design of the CDMA Allocation A-MAP IE

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Yongho Kim

LG Electronics

- Introduction

This contribution includes the proposed text for the CDMA Allocation A-MAP IE in the IEEE 802.16m Amendment. The technical text and ToC are inherited from the 802.16m SRD [1], and the IEEE 802.16m SDD [2] and the IEEE P802.16 Rev2/D9 [3]. Moreover, this contribution follows the tentative outline and style guide in the IEEE 802.16m Amendment [4].

- References

[1] IEEE 802.16m-07/002r7, “802.16m System Requirements Document (SRD)”

[2] IEEE 802.16m-08/003r7, “The Draft IEEE 802.16m System Description Document”

[3] IEEE P802.16 Rev2 / D9, “Draft IEEE Standard for Local and Metropolitan Area Networks: Air Interface 
for Broadband Wireless Access,” January. 2009.

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/0010r2, “IEEE 802.16m Amendment Working Document”

[6] IEEE S80216m-09/1420, “Design of the CDMA Allocation A-MAP IE”

- Text proposal for the 802.16m amendment

Remedy 1

On the 16m AWD 80216m-09_0010r2, page 67, line 50, make the following changes to subsection 15.2.11.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.11.2 Grant

In the bandwidth request procedure, the grant for BW-REQ message is allocated by CDMA Allocation A-MAP IE, which is shown in Table 35. CDMA Allocation A-MAP IE is used for allocation of bandwidth to a user that requested bandwidth using a ranging code or BR code. AMS decodes the IE and checks the MCRC field by its specific 12-bit RA-ID and 4-bit Masking Indicator. The RA-ID is calculated by a hash function with the AMS' random access attributes (i.e., frame number index, sub-frame number index, opportunity index and code index). Masking indicator indicates the identifier used for CRC masking, as shown in Table 36. The allocation shall be in the first UL subframe relevant to the A-MAP region regardless of DL/UL ratio and Nsubframe, A-MAP. ABS requests adjustment of timing and power to AMS by using Timing Adjust, Power Level Adjust and Offset Frequency Adjust in CDMA Allocation A-MAP IE for Ranging. The maximum number of the HARQ retransmission is TBD.

Table 35 – CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  |  |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| if MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource Assignment Information | TBD | Each bit indicates the decoding status of BR code in the orresponding BR opportunity. 0b0: No BR code is detected, 0b1: At least one code is detected |
| MCS | 5 | Modulation and coding scheme used for the burst |
| HFA | 4 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Reserved } | TBD |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Resource Assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs. |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Offset Frequency Adjust | 6 | Relative change in transmission frequency. Signed in units of Hz |
| Reserved } | TBD |  |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 2

On the 16m AWD 80216m-09_0010r2, page 68, line 34, make the following changes to subsection 15.2.11.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.2.11.2 Grant

Table 36—Description of the Masking Indicator

| Masking Indicator | Description |
| --- | --- |
| 0b0000 | MCRC is masked by 12-bit STID |
| 0b0001 | MCRC is masked by 12-bit RAID for Ranging |
| 0b0010 | MCRC is masked by 12-bit RAID for bandwidth request |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 3

On the 16m AWD 80216m-09_0010r2, page 201, line 36, make the following changes to subsection 15.3.6.5.2.2:

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.2.2 DL basic assignment A-MAP IE

Table 669 describes Assignment A-MAP IE Types.

Table 669—A-MAP IE Types

| A-MAP IE Type | Usage |
| --- | --- |
| 0b0000 | DL Basic Assignment A-MAP IE |
| 0b0001 | UL Basic Assignment A-MAP IE |
| 0b0010 | Feedback Allocation A-MAP IE |
| 0b0011 | Reserved |
| 0b0100 | Reserved CDMA Allocation A-MAP IE |
| 0b0101 | Reserved |
| 0b0110 | Reserved |
| 0b0111 | Reserved |
| 0b1000 | Reserved |
| 0b1001 | Reserved |
| 0b1010 | Reserved |
| 0b1011 | Reserved |
| 0b1100 | Reserved |
| 0b1101 | Reserved |
| 0b1110 | Reserved |
| 0b1111 | Reserved |

-----------------------------------------------------------End of the Text---------------------------------------------------------

Remedy 4

On the 16m AWD 80216m-09_0010r2, page 229, line 49, add the following subsection (move this CDMA Allocation A-MAP IE from 15.2.11.2 to 15.3.6.5.2.13)

-----------------------------------------------------------Start of the Text---------------------------------------------------------

15.3.6.5.2.13 CDMA Allocation A-MAP IE

Table xxx – CDMA Allocation A-MAP IE format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  |  |
| A-MAP IE type | 4 | CDMA Allocation A-MAP IE |
| if MCRC is masked with RAID and masking indicator for BR { |  |  |
| Resource Assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation [If ABS assigns HFA implicitly, this parameter is unnecessary.] |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Reserved } | TBD |  |
| Else if MCRC is masked with RAID and masking indicator for Ranging { |  |  |
| Resource Assignment Information | TBD |  |
| HFA | 4 | HARQ Feedback Allocation |
| Timing Adjust | 10 | Amount of advance time required to adjust AMS transmission. Unsigned in units of 1/Fs. |
| Power Level Adjust | 4 | Relative change in transmission power level. Signed in units of 1 dB |
| Offset Frequency Adjust | 6 | Relative change in transmission frequency. Signed in units of Hz |
| Reserved } | TBD |  |
| MCRC | 16 | CRC masked by RA-ID and Masking Indicator |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------