| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Management Message Frameworks |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes MAC management message frameworks. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Management Message Frameworks

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang,

Jose Puthenkulam, Shantidev Mohanty

Intel

- Introduction

Table 1 shows the MAC management message frameworks to be used to define 16m MAC management messages.

| Functional areas | 802.16-2009 |  |  |  | 802.16m Amendment |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MAC Mgmt Messages | Message Transport | Management Connections |  | 16m reuse |  | Transport (Broadcast, unicast) | Mgmt connection (realtime, non-realtime) | HARQ (Y/N) | Encryption (Y/N) |
|  |  |  |  |  | Yes/No | if yes, any changes? |  |  |  |  |
| System Information | UCD | Fragmentable broadcast | Broadcast CID |  |  |  |  |  |  |  |
|  | DCD | Fragmentable broadcast | Broadcast CID |  |  |  |  |  |  |  |
|  | DL-MAP | Broadcast | Broadcast CID |  |  |  |  |  |  |  |
|  | UL-MAP | Broadcast | Broadcast CID |  |  |  |  |  |  |  |
|  | SII-ADV | Fragmentable broadcast | Broadcast CID |  |  |  |  |  |  |  |
| Network Entry | RNG-REQ | Random access or unicast | Initial ranging or basic CID |  |  |  |  |  |  |  |
|  | RNG-RSP | Random access or unicast | Initial ranging or basic CID |  |  |  |  |  |  |  |
|  | REG-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | REG-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | SBC-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | SBC-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | TFTP-CPLT | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | TFTP-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
| Network Exit | DREG-CMD | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | DREG-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
| Connection Management | DSA-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSA-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSA-ACK | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSC-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSC-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSC-ACK | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSD-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSD-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DSX-RVD | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | DBPC-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | DBPC-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
| Security | PKM-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | PKM-RSP | Unicast or broadcast | Primary or broadcast CID |  |  |  |  |  |  |  |
| Multicast | MBS_MAP |  |  |  |  |  |  |  |  |  |
|  | MCA-REQ | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | MCA-RSP | Unicast | Primary CID |  |  |  |  |  |  |  |
| ARQ | ARQ-Feedback | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | ARQ-Discard | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | ARQ-Reset | Unicast | Basic CID |  |  |  |  |  |  |  |
| KPI | REP-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | REP-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
| Advanced Antenna | AAS-FBCK-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | AAS-FBCK-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | AAS_Beam_Select | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | AAS_BEAM_REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | AAS_BEAM_RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
| Sleep Mode | MOB_SLP-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_SLP-RSP | Unicast or broadcast | Basic or broadcast CID |  |  |  |  |  |  |  |
|  | MOB_TRF-IND | Broadcast | Broadcast CID |  |  |  |  |  |  |  |
| Handover | MOB_NBR-ADV | Unicast or broadcast | Primary or broadcast CID |  |  |  |  |  |  |  |
|  | MOB_SCN-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_SCN-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_BSHO-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_MSHO-REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_BSHO-RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_HO-IND | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | MOB_SCN-REP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | MOB_ASC-REP | Unicast | Primary CID |  |  |  |  |  |  |  |
|  | MOB_MIH-MSG | Unicast | Primary CID |  |  |  |  |  |  |  |
| Paging | MOB_PAG-ADV | Broadcast | Broadcast CID |  |  |  |  |  |  |  |
| Power control | PMC_REQ | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | PMC_RSP | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | FPC |  |  |  |  |  |  |  |  |  |
| LBS | LBS-ADV | Broadcast | Broadcast CID |  |  |  |  |  |  |  |
| MIMO | PRC-LT-CTRL | Unicast | Basic CID |  |  |  |  |  |  |  |
| MISC | RES-CMD | Unicast | Basic CID |  |  |  |  |  |  |  |
|  | CLK-CMP | Broadcast | Broadcast CID |  |  |  |  |  |  |  |

Table 1: MAC Management Frameworks