| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC Management Messages |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Joey Chou, Shantidev Mohanty, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang, Jose Puthenkulam, Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes MAC management messages. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Control Messages

Joey Chou, Muthaiah Venkatachalam, Sassan Ahmadi, Xiangying Yang,

Jose Puthenkulam, Shantidev Mohanty

Intel

- Introduction

This contribution proposes the MAC control messages.

- Proposed text

----------------------------------------------- Start of the proposed text ------------------------------------------

15.2.x MAC Control messages

The peer-to-peer protocol of MAC layers in BS and MS communicate using the MAC control messages to perform the control plane functions. MAC control messages shall be carried in a MAC PDU to be transported in broadcast, unicast or random access connections. There are two types of uniast control connections – Basic connection and Primary connection. HARQ shall be enabled for MAC control messages sent on the unicast control connections. Encryption may be enabled for unicast MAC control messages. MAC control messages may be fragmented. Table 1 lists the MAC control messages that shall be defined in the ASN.1 format, as shown in Appendix X.

Table 1: MAC Control Messages

| Message Names | Message Description | Connections |
| --- | --- | --- |
| SFH | Super Frame Header | Broadcast |
| ESI | Extended System Information | Broadcast |
| SII-ADV | Service Identity Information Advertisement | Broadcast |
| RNG-REQ | Ranging Request | Initial Ranging or Basic |
| RNG-RSP | Ranging Response | Initial Ranging or Basic |
| REG-REQ | Registration Request | Primary |
| REG-RSP | Registration Response | Primary |
| SBC-REQ | SS Basic Capability Request | Basic |
| SBC-RSP | SS Basic Capability Response | Basic |
| DREG-REQ | Deregistration Request | Basic |
| DREG-CMD | Deregistration Command | Basic |
| DSA-REQ | Dynamic Service Addition Request | Primary |
| DSA-RSP | Dynamic Service Addition Response | Primary |
| DSA-ACK | Dynamic Service Addition Acknowledge | Primary |
| DSC-REQ | Dynamic Service Change Request | Primary |
| DSC-RSP | Dynamic Service Change Response | Primary |
| DSC-ACK | Dynamic Service Change Acknowledge | Primary |
| DSD-REQ | Dynamic Service Deletion Request | Primary |
| DSD-RSP | Dynamic Service Deletion Response | Primary |
| DSX-RVD | DSx Received | Primary |
| PKM-REQ | Privacy Key Management Request | Primary |
| PKM-RSP | Privacy Key Management Response | Primary or broadcast |
| ARQ-FBCK | Stand-alone ARQ Feedback | Basic |
| HARQ-MAP | HARQ Map | Basic |
| MOB_SLP-REQ | Mobile Sleep Request | Basic |
| MOB_SLP-RSP | Mobile Sleep Response | Basic or broadcast |
| MOB_TRF-IND | Mobile Traffic Indication | Broadcast |
| MOB_NBR-ADV | Neighbor Advertisement | Primary or broadcast |
| MOB_SCN-REQ | Scanning Interval Allocation Request | Basic |
| MOB_SCN-RSP | Scanning Interval Allocation Response | Basic |
| MOB_SCN-REP | Scanning Result Report | Primary |
| MOB_MSHO-REQ | MS Handover Request | Basic |
| AAI_HO-CMD | BS Handover Command | Primary |
| MOB_HO-IND | Handover Indication | Basic |
| AAI_L2-XFER | AAI L2 Transfer | Primary |
| MOB_PAG-ADV | BS Paging Advertisement | Broadcast |
| PMC-REQ | Power Mode Change Request | Basic |
| PMC-RSP | Power Mode Change Response | Basic |

Appendix X ASN.1 Definition of MAC Control Messages

This Appendix defines MAC control messages using ASN.1 notation. The Packed Encoding Rules (PER) shall be used to produce compact transfer syntax for MAC control message to be transmitted over the air interface efficiently.

----------------------------------------------- End of the proposed text ------------------------------------------

Example of TLV vs. ASN.1 message

Table xxx—AAI_PAG-ADV Message Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI_PAG-ADV_Message_format() { | — | — |
| Management Message Type = TBD | 8 | — |
| Paging_Group_IDs bitmap | variable | The size of Paging_Group_IDs bitmap is the number of paging group IDs in the PGID info message, each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included |
| For (i = 0; i < Num_Postive_Paging_Group_IDs; i++) { | — | Num_Postive_Paging_Group_IDs is the number of bits set to one in the Paging_Group_IDs bitmap |
| Num_AMSs | 8 | Number of paged AMSs in this message |
| For (j = 0; j < Num_AMSs; j++) { | — | — |
| Temporary Identifier | 10 | Temporary identifier, paging cycle, and paging offset are used to uniquely identify an AMS in the idle mode in a particular paging group |
| Paging Cycle | 4 | Paging cycle consists of paging unavailable and paging available interval |
| Action Code | 2 | Paging Action instruction to AMS 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved |
| } | — | — |
| } | — | — |
| Extension Flag | 1 | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame |
| } | — | — |

-- ASN.1 types to be imported to ASN.1 modules

AsnTypeExports DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

EXPORTS PagingGroupPagingData, AmsPagingData;

-- Value of Configuration Change Count provided in DCD defining the

-- burst profile associated with DIUC

PagingGroupPagingData ::= SEQUENCE {

numberOfPagedAms           INTEGER (0..255),

node           SEQUENCE OF PagingGroupPagingData OPTIONAL,

amsPagingData              AmsPagingData

}

AmsPagingData ::= SEQUENCE {

temporaryIdentifier        INTEGER (0..1023),

-- Temporary identifier, paging cycle

-- , and paging offset are used to

-- uniquely identify an AMS in the

-- idle mode in a particular paging group

pagingCycle                INTEGER (0..3),

-- paging unavailable and

-- paging available interval

actionCode             BIT STRING (SIZE (2)),

-- 0b00 perform network re-entry

-- 0b01 perform ranging

-- 0b10 perform LBS measurement

-- 0b11 Reserved

node           SEQUENCE OF AmsPagingData OPTIONAL

}

END

-- AAI_PAG-ADV is an AMS notification message that indicates presence

-- of DL traffic pending for the specified AMS or it is intended to poll

-- an AMS and request a location update without requiring a full network

-- entry or to request an AMS to perform LBS measurement.

AAI-PAG-ADV DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS PagingGroupPagingData, AmsPagingData FROM AsnTypeExports;

PagingAdvertizemwntMessage ::= SEQUENCE {

-- Each bit in the bitmap indicates the presence of paging

-- information for the corresponding PGIA:

--   0: not presence, or not available

--   1: presence

pagingGroupIdBitmap              BIT STRING (SIZE (4)),

pagingGroupPagingData            PagingGroupPagingData,

extensionFlag                    BOOLEAN

}

END