| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request to IEEE 802.16m AWD: S-SFH IE |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI Yih-Shen Chen MediaTek Yanfeng Guan, Huiying Fang ZTE Corporation | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw E-mail: yihshen.chen@mediatek.com E-mail: guan.yanfeng@zte.com.cn |
| Re: | Category: AWD-comments / Area: Chapter 15.3.6 (Downlink Control Structure) “Call for comments on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | This contribution proposes changes to the Section 15.3.6.5.1.2 of IEEE 802.16m AWD. |  |
| Purpose | To discuss and adopt the proposed text in the IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request to IEEE 802.16m AWD: S-SFH IE

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Yih-Shen Chen

MediaTek

Yanfeng Guan, Huiying Fang

ZTE Corporation

- Introduction

The usage of the parameter Sub-frame configuration in S-SFH SP2 IE is not defined in the current AWD [1]. In this contribution, we propose different usage and configurations for the sub-frame configuration under different duplexing modes, based on the following pre-knowledge: 1) In TDD mode, the DL LZone and the DL MZone shall be offset by a fixed number of subframes K, the maximum value of K is equal to the number of DL subframes minus two. 2) The UL LZone is able to FDM with the UL MZone.

- Proposed Text for IEEE 802.16m AWD

[Accept the following modifications marked in blue.]

============================ Start of Proposed Text ==================================

#### 15.3.6.5 DL Control Information Elements

#### 15.3.6.5.1.2 S-SFH IE

Table 664—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Duplexing mode | 1 |  |
| Sub-frame configuration (DL/UL ratio, duplexing mode) | 6 |  |
| Legacy support | 1 | Indicate whether legacy mode is supported or not |
| If (Duplexing mode == FDD) { |  |  |
| UL carrier frequency | [12] | (Need more discussion) |
| UL bandwidth | [3] | (Need more discussion) |
| If (Legacy support == 1) { |  |  |
| Sub-frame configuration | 7 | Bit #0~2: Number of DL Legacy Sub-frames Bit #3~6: Number of UL Legacy Sub-frames (0bxxx1111= UL LZone and MZone are in FDM mode) |
| FDM ratio | 3 | Indicate the UL LZone to MZone ratio in frequency domain |
| } |  |  |
| } |  |  |
| else { |  |  |
| If (Legacy support == 1) { |  |  |
| Sub-frame configuration | 6 | Indicate the DL to UL ratio as well as the number of legacy subframes, in the format of “(DL to UL ratio, number of DL legacy subframes, number of UL legacy subframes)” 0=(8:0, 1, 0); 1=(8:0, 2, 0); 2=(8:0, 3, 0); 3=(8:0, 4, 0); 4=(8:0, 5, 0); 5=(8:0, 6, 0); 6=(6:2, 1, 0); 7=(6:2, 1, 1); 8=(6:2, 1, 2); 9=(6:2, 1, FDM); 10=(6:2, 2, 0); 11=(6:2, 2, 1); 12=(6:2, 2, 2); 13=(6:2, 2, FDM); 14=(6:2, 3, 0); 15=(6:2, 3, 1); 16=(6:2, 3, 2); 17=(6:2, 3, FDM); 18=(6:2, 4, 0); 19=(6:2, 4, 1); 20=(6:2, 4, 2); 21=(6:2, 4, FDM); 22=(5:3, 1, 0); 23=(5:3, 1, 1); 24=(5:3, 1, 2); 25=(5:3, 1, 3); 26=(5:3, 1, FDM); 27=(5:3, 2, 0); 28=(5:3, 2, 1); 29=(5:3, 2, 2); 30=(5:3, 2, 3); 31=(5:3, 2, FDM); 32=(5:3, 3, 0); 33=(5:3, 3, 1); 34=(5:3, 3, 2); 35=(5:3, 3, 3); 36=(5:3, 3, FDM); 37=(4:4, 1, 0); 38=(4:4, 1, 1); 39=(4:4, 1, 2); 40=(4:4, 1, 3); 41=(4:4, 1, 4); 42=(4:4, 1, FDM); 43=(4:4, 2, 0); 44=(4:4, 2, 1); 45=(4:4, 2, 2); 46=(4:4, 2, 3); 47=(4:4, 2, 4); 48=(4:4, 2, FDM); 49=(3:5, 1, 0); 50=(3:5, 1, 1); 51=(3:5, 1, 2); 52=(3:5, 1, 3); 53=(3:5, 1, 4); 54=(3:5, 1, 5); 55=(3:5, 1, FDM); 56~63: Reserved *FDM means UL LZone and MZone are in FDM mode |
| FDM ratio | 3 | Indicate the UL LZone to MZone ratio in frequency domain |
| } |  |  |
| else { |  |  |
| Sub-frame configuration | 3 | Indicate the DL to UL ratio 0=8:0; 1=6:2; 2=5:3; 3=4:4; 4=3:5 |
| } |  |  |
| } |  |  |
| MSB bytes of BSID | 24 | Specifies the Operator ID |
| MAC protocol revision | 4 | Version number of IEEE 802.16m supported on this channel |
| FFR partitioning info for DL region | [11] | DL_SAC(4), DL_FPSC(3), DL_FPC(4) (Up to 11 bits, Need the decision from DL physical structure section) |
| FFR partitioning info for UL region | [11] | UL_SAC(4), UL_FPSC(3), UL_FPC(4) (up to 11 bits, Need the decision from UL physical structure section) |
| Initial ranging codes | 6 | 64 RNG codes (Need the decision from UL Ctrl section) |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (the highest order bits shall be unused and set to 0) (Need the decision from UL Ctrl or MAC operation section) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0–15 (Need the decision from UL Ctrl or MAC operation section) |
| NSP ID | 24 | Network service provider ID |
| Additional broadcast information indicator (ABI) | TBD |  |
| MS Transmit Power Limitation Level | 8 | Unsigned 8-bit integer. Specifies the maximum allowed MS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| Minimum level of power offset adjustment | TBD |  |
| Maximum level of power offset adjustment | TBD |  |
| reserved |  |  |
| } |  |  |

============================== End of Proposed Text ================================

References

- IEEE 802.16m-08/0010r2. IEEE 802.16m Amendment Working Document, June 2009.

- IEEE 802.16m-08/003r9. The Draft IEEE 802.16m System Description Document, May 2009.