| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change Request to IEEE 802.16m AWD: Operation During Idle Mode (AWD-15.2.17.2) |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI Yih-Shen Chen MediaTek Shantidev Mohanty, Muthaiah Venkatachalam Intel Corporation Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Kiseon Ryu, Giwon Park, Ronny Kim, JinSam Kwak LG Electronics | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw E-mail: yihshen.chen@mediatek.com E-mail: Shantidev.mohanty@intel.com E-mail: hyunjeong.kang@samsung.com E-mail: ksryu@lge.com |
| Re: | Category: AWD-comments / Area: Chapter 15.2.17 (Idle Mode) “Call for comments on Project 802.16m Amendment Working Document (AWD) Content” |  |
| Abstract | This contribution proposes changes to the Section 15.2.17.2 of IEEE 802.16m AWD. |  |
| Purpose | To discuss and adopt the proposed text in the IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change Request to IEEE 802.16m AWD: Operation during Idle Mode (AWD-15.2.17.2)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

Yih-Shen Chen

MediaTek

Shantidev Mohanty, Muthaiah Venkatachalam

Intel Corporation

Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung

Kiseon Ryu, Giwon Park, Ronny Kim, JinSam Kwak

LG Electronics

- Introduction

Temporary identifier is already defined in Section 15.2.1.2.3 “Temporary Identifier (TID)” in IEEE 802.16m Amendment Working Document (AWD) [1]. According to the TID definition in Section 15.2.1.2.3, we suggest some text modifications to Section 15.2.17.2 to keep consistency throughout the AWD. In addition, we also propose AAI_PAG-ADV message format in this contribution based on the content of Section 15.2.17.

- Proposed Text for IEEE 802.16m AWD

[Accept the following modifications marked in blue.]

============================ Start of Proposed Text ==================================

#### 15.2.17.2 Operation during Idle mode

#### 15.2.17.2.1 Broadcast paging message

A Paging message is an AMS notification message which either indicates the presence of DL traffic pending for the specified AMS or it is intended to poll an AMS and request a location update without requiring a full network entry or to request an AMS to perform LBS measurement.

A single Paging message may include the information for multiple AMSs.

Paging message includes identification of the AMSs (i.e. temporary identifier) to be notified of DL traffic pending or location update.

The Paging message also includes an action code directing each AMS notified via the inclusion of its identifier as appropriate:

• 0b00: Perform network re-entry

• 0b01: Perform ranging to establish location

• 0b10: Perform LBS measurement

An AMS shall terminate idle mode and reenter the network if it decodes a paging message that contains the AMS's temporary identifier identification and action code 0b00 (Re-enter Network). In the event that an AMS decodes a paging message that contains the AMS's temporary identifier identification and action code 0b01, it performs ranging for location update. When the AMS decodes a paging message that does not include its temporary identifier identification, it means that the AMS is not being paged and the AMS may enter its next paging unavailable interval.

The ABS shall transmit the paging message within a frame known to both the ABS and the AMS. If the ABS cannot transmit the entire paging message in a predetermined paging transmission frame, the remaining part of the paging message is transmitted in the earliest subsequent frame. The extension of paging listening interval shall be indicated by the extension flag in the paging message. Thus, in this case, an idle mode AMS remains awake and monitors the subsequent frames for paging message. After receiving the complete paging message, the idle mode AMSs returns to paging unavailable interval if the AMS is not paged.

Table xxx—Parameters for AAI_PAG-ADV Message Format

| Name | Value | Usage |
| --- | --- | --- |
| Paging_Group_IDs bitmap | Each bit in the bitmap indicates: 0: the paging information for the corresponding PGID is not included 1: the paging information for the corresponding PGID is included | The size of Paging_Group_IDs bitmap equals to the number of paging group IDs in the PGID info message. |
| Num_AMSs | Number of paged AMSs in a particular paging group | For each non-zero bit in the Paging_Group_IDs bitmap, there shall be one Num_AMSs. |
| Temporary Identifier | 0~1023 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Paging Cycle | 0b0000~0b1111 | Temporary Identifier and Paging Cycle are used to identify each paged AMS. |
| Action Code | 0b00 = Perform network re-entry 0b01 = Perform ranging to establish location 0b10 = Perform LBS measurement 0b11 = Reserved | Paging action instruction to each paged AMS. |
| Extension Flag | 0 = There is no remaining part of this paging message 1 = The remaining part of this paging message will be transmitted in the subsequent frame or sub-frame | If the ABS cannot transmit the entire paging message in a predetermined region, the Extension Flag will be set to 1. Then the remaining part of the paging message is transmitted in the earliest subsequent frame or sub-frame. |

#### 15.2.17.2.2 Operation during paging unavailable interval

[No modification in this sub-section.]

#### 15.2.17.2.3 Operation during paging listening interval

[Modify the last paragraph as shown in the following blue text.]

The AMS shall monitor pre-determined frame for paging message. If AMS's temporary identifier identification is included in the paging message, the AMS shall perform network re-entry or location update depending on the notification in the paging message. Otherwise, the AMS may return to the paging unavailable interval.

============================== End of Proposed Text ================================

References

- IEEE 802.16m-08/0010r2. IEEE 802.16m Amendment Working Document, June 2009.

- IEEE 802.16m-08/003r9. The Draft IEEE 802.16m System Description Document, May 2009.