| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ buffer management in 16m |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Muthaiah Venkatachalam Xiangying Yang Aran Bergman Intel | E-mail: muthaiah.venkatachalam@intel.com |
| Re: | TGm AWD |  |
| Abstract | This contribution proposes text for ARQ buffer management in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ buffer management in 16m

Muthaiah Venkatachalam, Xiangying Yang, Aran Bergman

Intel

- Introduction

With fully variable ARQ, the issue is that when the MS has missing intermediate sequence numbers in the ARQ window, the MS will not know the length of each of these missing “blocks” corresponding to the intermediate sequence numbers. This is because each block represented by a sequence number is fully variable and is mapped to a MAC PDU, which can be very variable, ranging upto 2KB in length.

Reserving 2KB per missing block is not the most optimal approach for a typical buffer constrained MS.

In this contribution, we propose text to resolve this issue in a simple fashion.

- Proposed solution

Proposed text:

15.2.13.2.6 MAXIMUM_ ARQ_BUFFER_SIZE

This is a negotiation parameter between ABS and AMS. AMS shall inform maximum ARQ buffer size to ABS during capability negotiation. When there are one or more ARQ enabled flows for a given MS, the ABS shall maintain the state ARQ_BUFFER_USED, which is the total ARQ buffer usage of all ARQ enabled flows at the MS. ABS should shall send ensure the transmission or retransmission of ARQ blocks not to exceeding MAXIMUM_ARQ_BUFFER_SIZE – ARQ_BUFFER_USED.