| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ buffer management in 16m |  |
| Date Submitted | 2009-07-13 |  |
| Source(s) | Muthaiah Venkatachalam Xiangying Yang Aran Bergman Intel Ming-Hung Tao Fang-Ching (Frank) Ren Yan-Xiu Zheng ITRI YihShen Chen MediaTek Youngbin Chang Rakes Taori Baowei Ji Samsung | E-mail: muthaiah.venkatachalam@intel.com mhtao@itri.org.tw yihshen.chen@mediatek.com |
| Re: | TGm AWD |  |
| Abstract | This contribution proposes text for ARQ buffer management in 16m. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ buffer management in 16m

Muthaiah Venkatachalam, Xiangying Yang, Aran Bergman

Intel

Ming-Hung Tao, Fang-Ching (Frank) Ren, Yan-Xiu Zheng

ITRI

YiShen Chen

MediaTek

Youngbin Chang, Rakes Taori, Baowei Ji

Samsung

- Introduction

With fully variable ARQ, the issue is that when the MS has missing intermediate sequence numbers in the ARQ window, the MS will not know the length of each of these missing “blocks” corresponding to the intermediate sequence numbers. This is because each block represented by a sequence number is fully variable and is mapped to a MAC PDU, which can be very variable with a very large maximum value.

Reserving max PDU size per missing block is not the most optimal approach for a typical buffer constrained MS.

In this contribution, we propose text to resolve this issue in a simple fashion.

- Proposed solution

Proposed text:

15.2.13.2.6 MAXIMUM_ ARQ_BUFFER_SIZE

This is a negotiation parameter between ABS and AMS. The MAX_ARQ_BUFFER_SIZE is the maximum size of the buffer (in bytes) that the AMS is able to allocate for the ARQ connection. The AMS shall inform this parameter to the ABS during capability negotiation.

When transmitting a new ARQ block, the ABS should ensure that the size of the new ARQ block does not exceed MAX_ARQ_BUFFER_SIZE – ARQ_BUFFER_USED.  ARQ_BUFFER_USED is defined as the sum of ARQ_BUFFER_USED_PER_FLOW for all the ARQ enabled flows for the AMS. ARQ_BUFFER_USED_PER_FLOW is calculated as the occupied buffer size of ARQ transmitter window (ARQ_TX_WINDOW_START to ARQ_TX_NEXT_SN-1(inclusive)).

If the ARQ blocks, with SN smaller than ARQ_TX_WINDOW_START, are part of same SDU as the first ARQ block in the ARQ transmitter window, then those ARQ blocks shall be included in the calculation of ARQ_BUFFER_USED_PER_FLOW.