| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Fast Device Capability Negotiation for IEEE 802.16m Systems |  |
| Date Submitted | 2009-07-14 |  |
| Source(s) | Muthaiah Venkatachalam Sassan Ahmadi Intel | E-mail: muthaiah.venkatachalam@intel.com |
| Re: | IEEE 802.16m-09/0028r1 Call for Comments and Contributions on Project IEEE 802.16m Amendment Content |  |
| Abstract | This contribution proposes a method to simplify device capability negotiation (SBC) procedures in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Fast Device Capability Negotiation for IEEE 802.16m Systems

Muthaiah Venkatachalam, Sassan Ahmadi

Intel

- Introduction

The unnecessary capability negotiation procedures and bulky MAC management messages that are currently included and exchanged in IEEE Std 802.16-2009 between the MS and BS during system entry (or re-entry) can be simplified and the access state processing can be made faster and more efficient by following the proposed method, resulting in more reliability and robustness and lower network entry latency (Control-Plane Latency and Call Setup Time)

In fact, the field test results from operator trails and networks suggest that the extended capability negotiations between the BS and MS could result in extremely long delays and potentially inability of system access for mobile stations in the cell edge due to channel errors. Therefore, the capability negotiations must be shortened and made more efficient.

- Proposed solution

This contribution proposes a simple solution for capability negotiation between advanced mobile station (AMS) and advanced base station (ABS) during network entry (re-entry). The TLV-coded MAC management messages exchanged during capability negotiations (see Pages 333-336 of [1]) are performed irrespective of the required MS or BS capabilities to ensure proper interoperability.

Immediately after completion of ranging, the MS informs the BS of its basic capabilities by transmitting an SBC-REQ message (SS basic capability request) with its capabilities set to “on” (see Figure 75 of [1]). The BS responds with an SBC-RSP message (SS basic capability response) with the intersection of the MS’s and the BS’s capabilities set to “on” (see Figure 76 and Figure 77 of [1]).

Instead, if we assume that the MS by default supports a basic set of features or configuration parameters (probably mandated by a system profile or by the standard specification per se), there would be no need to negotiate and configure the basic capabilities.

Proposed text:

-------------------------------------------------Begin Proposed Text-------------------------------------------------------------------

15.2.5.8 AAI_SBC_REQ

The AMS, by default, is expected to support the basic capabilities; therefore, the ABS can assume that the AMS supports basic capabilities and the transmission of AAI_SBC_REQ message may be skipped, if either the AMS or ABS wishes to operate based on the basic capability feature set.

If the AMS is capable of supporting higher revisions of physical layer or medium access control layer protocols or further wishes to use enhanced features, it shall send an AAI_SBC_REQ message to the ABS indicating the highest “CAPABILITY_INDEX” that it can support. The CAPABILITY_INDEX = 0 indicates the default capability index and basic feature set or configuration parameters and may not need to be signaled.

The upon receipt of the AAI_SBC_REQ message containing the “CAPABILITY_INDEX” from the AMS, the ABS determines whether it could allow or could support the requested feature set or MAC and/or PHY protocol revisions.  If the ABS does support or can allow the use of enhanced features, it shall respond with an AAI_SBC-RSP message to inform the AMS of its decision. The ABS shall only signal a “CAPABILITY_INDEX” which is numerically smaller than or equal to that requested by the AMS.

The higher numeric values of “CAPABILITY_INDEX”, the more enhanced features or higher protocol revisions are used.. The “CAPABILITY_INDEX” values range from 0 to N, where N denotes the maximum CAPABILITY_INDEX value. The features and configuration parameters included in the baseline class shall be sufficient to meet the minimum performance requirements of the standard.

In case of failure in any stages of operation, the AMS and ABS shall fall back to “Capability Class 0” and restart negotiations for a new “Capability Class”, if necessary.

Figure 1 shows the MAC management messages exchanged over the advanced air-interface, classification of the features into Capability Classes, AAI_SBC-REQ and AAI_SBC-RSP message formats.

Figure 1: AAI_SBC_REQ and AAI_SBC_RSP messages exchanged over the air-interface and feature classification

Management Message Type

CAPABILITY_INDEX

Figure 2: AAI_SBC-REQ message parameters

In Figure 2, the CAPABILITY_INDEX transmitted in the AAI_SBC_REQ message refers to the maximum capability class that the AMS can support.The maximum value of CAPABILITY_INDEX is FFS.

Management Message Type

CAPABILITY_INDEX

Figure 3: AAI_SBC-RSP message format

In Figure 3, the CAPABILITY_INDEX transmitted in the AAI_SBC_RSP message refers to the capability class that the ABS has allowed the AMS to use during the session. The value of CAPABILITY_INDEX signaled in the AAI_SBC_RSP message is numerically smaller than or equal to the CAPABILITY_INDEX signaled in the AAI_SBC_REQ message by the AMS.

-------------------------------------------------------End Proposed Text-----------------------------------------------------------------

References:

[1] IEEE Std 802.16-2009 specification