| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC management reliability updates |  |
| Date Submitted | 2009-07-06 |  |
| Source(s) | Muthaiah Venkatachalam Intel | E-mail: muthaiah.venkatachalam@intel.com |
| Re: | IEEE 802.16m-09/0028r1 Call for Comments and Contributions on Project IEEE 802.16m Amendment Content |  |
| Abstract | This contribution proposes updates to MAC management reliability section in AWD. |  |
| Purpose | To be discussed and adopted by TGm for IEEE 802.16m AWD |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC management reliability updates

Muthaiah Venkatachalam

Intel

Proposed text:

-------------------------------------------------Begin Proposed Text-------------------------------------------------------------------

15.2.19 MAC management reliability

802.16m shall provide fast and reliable delivery for MAC management messages. 802.16m shall use HARQ for all unicast MAC management messages with HARQ sequence numbering enabled. Large messages can be fragmented for transmission. Retransmission timers may be defined for MAC management messages. The transmitter may retransmit a complete message or a fragment of the message if the retransmission timer expires while waiting for acknowledgement of successful transmission. If the HARQ process is terminated with an unsuccessful outcome before the expiration of the retransmission timer, the transmitter may initiate retransmission of the message or the message fragment of the failed HARQ burst. The retransmission timer is typically set greater than “MAX HARQ latency”.

-------------------------------------------------------End Proposed Text-----------------------------------------------------------------

References:

[1] IEEE Std 802.16-2009 specification